# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415049

# Hefei, China, 14th – 18th Octerber, 2024

Agenda item:	6.8.3.1

Source: 	Xiaomi

Title: 	Discussion on system parameters for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 had some initial discussion on system parameters for NTN less than 5MHz, and agreement and open issues were captured in WF [1]. The remaining open issues are copied as follows:

System parameters Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - FFS Issue 1-2-6: Signalling of SS raster - FFS Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN - FFS

This contribution discusses above open issues on system parameter for NTN less than 5MHz and provides our proposals.

Discussion

In RAN#112 meeting, RAN4 agreed to introduce 3MHz CBW with 15KHz SCS for FR1 NTN bands, and FFS on whether the 3MHz channel bandwidth is mandatory or optional. From our perspective, we support 3MHz CBW is introduced as an optional UE capability, since UE may reuse the existing modem and RF components from TN to support NTN operations, and the 3MHz CBW introduced for NR TN is optional in Rel-18.

Proposal 1: RAN4 to define 3MHz channel bandwidth is optional for FR1 NR-NTN bands.

If RAN4 agrees 3MHz CBW is an optional UE capability for NTN bands, the exiting signalling can be reused. In Rel-18, UE capabilities of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 were introduced to support 3MHz CBW in symmetrical and asymmetrical configurations.

Proposal 2: If 3MHz CBW is optional for FR1 NTN bands, the exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

According to objective description in [2], the 3MHz channel bandwidth does not apply to RedCap/eRedcap NTN, as the 3MHz CBW is defined based on RAN1 design in Rel-18.

Proposal 3: 3MHz channel bandwidth for FR1 NTN bands does not apply to RedCap/eRedcap NTN.

Conclusion

In this contribution, we discussed the remaining open issues on system parameter for NTN less than 5MHz and provides our proposals:

Proposal 1: RAN4 to define 3MHz channel bandwidth is optional for FR1 NR-NTN bands.

Proposal 2: If 3MHz CBW is optional for FR1 NTN bands, the exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

Proposal 3: 3MHz channel bandwidth for FR1 NTN bands does not apply to RedCap/eRedcap NTN.

- Reference

[1]	R4-2413528	WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF	Xiaomi

[2]	RP-241281	Revised WID on Enhanced requirements and test methodology for NR and IoT NTN	Samsung, Xiaomi