# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415050

# Hefei, China, 14th – 18th Octerber, 2024

Agenda item:	6.8.3.2

Source: 	Xiaomi

Title: 	Discussion on UE RF requirements for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 had some initial discussion on UE RF requirements for NTN less than 5MHz, and the related WF was approved in [1]. This contribution discusses UE RF requirements for NTN less than 5MHz and provides our proposals.

Discussion

In Rel-18, 3MHz channel bandwidth with 15kHz SCS was introduced for NR TN. In order to enable more efficient deployment of NTN 3MHz channel bandwidth, the UE RF requirements for NTN 3MHz channel bandwidth should be introduced based on the requirements defined for TN 3MHz channel bandwidth. Therefore, compared with the UE RF requirements defined for TN 3MHz CBW, the additional RF impact due to introducing NTN 3MHz CBW is summarised in following table 1 and table 2.

Proposal 1: RAN4 to consider the RF impact as presented in Table 1 and Table 2 when defining Tx RF and Rx RF requirements for 3 MHz channel bandwidth for NTN UE.

Table 1 NTN UE Tx RF impact

| NTN UE Tx RF requirements | Impact to introduce 3MHz for NTN FR1 bands |
| --- | --- |
| 6.2.1 UE maximum output power | √ FFS on whether need to introduce 3MHz CBW for n254 in Table 6.2.1-2: Additional requirements for transmit power density |
| 6.2.2 UE maximum output power reduction | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.2.3 UE additional maximum output power reduction | √ Figure out which NS values need define A-MPR for 3 MHz CBW |
| 6.2.4 Configured transmitted power | No specification impact |
| 6.3.1 Minimum output power | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.3.2 Transmit OFF power | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.3.3 Transmit ON/OFF time mask | No specification impact |
| 6.3.4 Power control | No specification impact |
| 6.4.1 Frequency error | No specification impact |
| 6.4.2 Transmit modulation quality | No specification impact |
| 6.5.1 Occupied bandwidth | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.5.2.2 Spectrum emission mask | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.5.2.3 Additional spectrum emission mask | No specification impact |
| 6.5.2.4.1 NR ACLR | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.5.2.4.2 UTRA ACLR | No specification impact |
| 6.5.3.1 General spurious emissions | No specification impact |
| 6.5.3.2 Spurious emissions for UE co-existence | No specification impact |
| 6.5.3.3 Additional spurious emissions | √ New requirements need define for NS_02N, 03N, 04N, 05N |

Table 2: NTN UE Rx RF requirements

| NTN UE Rx RF requirements | Impact to introduce 3MHz for NTN FR1 bands |
| --- | --- |
| 7.3.2 Reference sensitivity power level | √ New requirements need define for 3MHz CBW |
| 7.4 Maximum input level | √ Whether can reuse the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE |
| 7.5 Adjacent channel selectivity | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.6.2 In-band blocking | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.6.3 Out-of-band blocking | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.6.4 Narrow band blocking | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 7.7 Spurious response | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.8 Intermodulation characteristics | No specification impact |
| 7.9 Spurious emissions | No specification impact |

Conclusion

In this contribution, we discussed the UE RF requirements for NTN less than 5MHz and provides our proposals:

Proposal 1: RAN4 to consider the RF impact as presented in Table 1 and Table 2 when defining Tx RF and Rx RF requirements for 3 MHz channel bandwidth for NTN UE.

- Reference

[1]	R4-2413528	WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF	Xiaomi