3GPP TSG-RAN WG4 Meeting #112bis			                               R4-2415056

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.12.2.2

Source:	Xiaomi

Title:			Discussion on OTA test for FR1 NTN devices

Document for:	Discussion

- Introduction

The test methodology and radiated performance metric for FR1 NTN devices has been extensively discussed in previous meetings but no consensus. This contribution continues to provide our views on these open issues.

- Discussion

performance metrics

Up to now, the performance metrics for FR1 NTN handheld UE were summarized as 5 directions as shown in the below [1]. And it was agreed direction 1 should be abandon due to the formfactor.

Sub-topic 3-2 UE performance metric Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

For NTN handheld UE, compared with traditional FR1 TN antenna, the antenna radiation pattern is generally optimized to have some directivity to compensate for the path loss caused by large distance. However, it is not so directional due to the small form factor and the need to ensure sufficient spherical coverage. Therefore, it is not appropriate to adopt peak EIRP/EIS metric for FR1 NTN UE.

As SAN are in the upper hemisphere of handheld UE and operators are not interested in the performance of in the other hemisphere, adopt direction 5 (only partial sphere) is reasonable and natural, FFS on the range of angles. However, adopting this direction needs to have a restriction on the orientation of the UE to ensure that the main beam is covered in the partial sphere, as different orientation of UE will heavily impact the performance.

Proposal 1: adopt direction 5 (only partial sphere TRP/TRS) as the performance metrics for NTN FR1 handheld UE, and the orientation of the UE shall be restricted to ensure that the main beam is covered in the partial sphere when OTA testing.

The other issue is whether performance metric should be different for GEO and NGEO (including LEO and MEO). Based on the UE capability in Ts38.306 in the follow, theoretically there will be 3 different UEs in the field, that is UE only support GSO, UE only supporting NGSO and UE supporting both. We think developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE. The reason is that the elevation angle of the fixed UE is generally fixed, so the spherical coverage can be smaller when it only supports the GSO scenario. However, the orientation of the handheld UE is more flexible, so even if the handheld UE only supports the GSO scenario, its spherical coverage is still required to be larger. In addition, from UE implementation point of view, if the supported satellite band is the same, the unified antenna design will be adopted.

Observation 1: As the spherical coverage for handheld UE should be large even it only supports GSO scenario, given the orientation of handheld UE is flexible, developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE.

| nonTerrestrialNetwork-r17 Indicates whether the UE supports NR NTN access. If the UE indicates this capability the UE shall support the following NTN essential features, e.g., timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT, acquiring NTN specific SIB and more than one TAC per PLMN broadcast in one cell. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 Indicates whether the UE supports the NTN features in GSO scenario or NGSO scenario. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports the NTN features for both GSO and NGSO scenarios, and also supports mobility between GSO and NGSO scenarios. | UE | No | No | No |

3	Conclusions

In this paper, we provide our views on developing test methodology and radiated performance metric for FR1 NTN devices, and provide the proposal as below:

Proposal 1: adopt direction 5 (only partial sphere TRP/TRS) as the performance metrics for NTN FR1 handheld UE, and the orientation of the UE shall be restricted to ensure that the main beam is covered in the partial sphere when OTA testing.

Observation 1: As the spherical coverage for handheld UE should be large even it only supports GSO scenario, given the orientation of handheld UE is flexible, developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE.

4 References

[1] R4-2413603, Way Forward for [111] TRP_TRS_MIMO_OTA, Vivo, CAICT

[2] R4-2411598,	Discussion on OTA test for FR1 NTN devices, Xiaomi

[3] RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, Moderator (RAN4 VC, Qualcomm Incorporated)