3GPP TSG-RAN WG4 Meeting #112bis			                                R4-2415059

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.3.4

Title: 	Discussion on RRM requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for enhanced requirements and test methodology for NR and IoT NTN was agreed and one of objectives for less than 5MHz for NTN is self-contained as below.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

And in the last RAN4 meeting, the agreements for less than 5MHz for NTN was achieved in [2, R4-2413974]. Therefore, in this discussion we provided further considerations on the RRM impacts for less than 5MHz NTN.

- Discussion

  - Baseline assumption RRM impacts

In the last RAN4 meeting RAN4 firstly discussed the baseline assumptions of RRM requirements for less than 5MHz NTN. And the following agreements was achieved.

Issue 1-2-2: Baseline assumption Issue 1-2-2-1: Channel bandwidth assumption Agreement: - To align with the agreement in BDaT session. Issue 1-2-2-2: SSB/PBCH assumption Agreement - To align with the agreement in BDaT session. Issue 1-2-2-3: CORRSET Agreement - To align with the agreement in BDaT session.

In principle, in comparison with the current NTN physical layer design, the potential changes could be mainly from SSB/PBCH reduction under less than 5MHz channel bandwidth as agreed in Rel18 for TN.

Figure 1.SSB for NTN

Fortunately, as stated in WID, the current RAN1 design in Re18 for TN can be leveraged to NTN. For an example, the PBCH mapping in [6,TS38.211] below can be take our baseline assumption when we define the corresponding RRM requirements.

7.4.3 SS/PBCH block 7.4.3.1 Time-frequency structure of an SS/PBCH block …… For cell search on a carrier with a channel bandwidth of 3 MHz, the UE is not expected to receive subcarriers 0 to 47 and 192 to 239 in any of the 4 OFDM symbols of the SS/PBCH block, where the remaining 12 resource blocks form the SS/PBCH block after puncturing. …… 7.3.2.2 Control-resource set (CORESET) …… For CORESET 0 on a carrier where the SS/PBCH block is detected at sync raster points defined in Tables 5.4.3.1-2 or 5.4.3.1-3 of [14, TS 38.101-1] and configured by the ControlResourceSetZero IE: - NRBCORESET and NsymbCORESET are defined by Table 13-0 in clause 13 of [5, TS 38.213]; - if NRBCORESET=12 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2; - if NRBCORESET=24 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2 or non-interleaved mapping as defined by clause 13 of [5, TS 38.213], followed by puncturing the 9 highest-numbered resource blocks to obtain the 15 resource blocks forming CORESET 0; - if NRBCORESET=24 on a carrier with a channel bandwidth of 5 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2, followed by puncturing the 4 highest-numbered resource blocks to obtain the 20 resource blocks forming CORESET 0; ……

Moreover, according to the WF from BDaT session [3, R4-2413528] below, we can conclude that:

Issue 1-1-1: Channel bandwidth - Agreement - Only introduce 3MHz channel bandwidth with 15kHz SCS Issue 1-1-2: Operating bands for 3MHz channel bandwidth - Agreement - Applicable NTN bands: band n256, n255 and n254 Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN. Issue 1-2-3: Synchronization raster - Agreement - Reuse TN sync raster for 3MHz channel bandwidth and extend to 3000MHz for NTN FR1 3MHz channel bandwidth as below table: Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 3000; N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1); 26638+3N + (M-3)/2; 26640 – 41636. - Include note1: NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1

Proposal 1: RAN4 can start the RRM impact analysis with the following assumptions at least based on Rel18 TN less than 5MHz WI:

- 15RBs maximum transmission bandwidth configuration (NRB) for 3M channel bandwidth [4]

- 12RBs for PBCH with 3MHz channel bandwidth [4]

- 15RBs forming CORESET 0 with 3MHz channel bandwidth[4]

  - RRM impacts

With the working assumption above, we can see the available SSB and PBCH resource in case of less than 5MHz NTN will be reduced. That is

Observation 1: The RRM requirements for NTN in Rel18[4, TS38.133] depending on PBCH allocation will be impacted due to reduced channel bandwidth.

Therefore, we firstly provide more detail analysis on the potential impacts.

- Handover

In the last RAN4 meeting, the general principle to update the handover requirements for less than 5MHz NTN was agreed as:

Issue 1-3-2: Handover Agreement: Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline. For NTN specific HO requirement: - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz. - Satellite switch time requirement will be defined for NTN less than 5MHz. - Further discuss the details of the requirements.

More especially, the RRM requirements for NTN handover was defined in TS38.133 6.1C [4]. One of key parameter is Tinterrupt which is given by the formula below, which is the key component of other requirements (e.g. SAN handover delay, SAN CHO handover delay)

6.1C.1.2.2 Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH or PUSCH, excluding the RRC procedure delay. In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover. 6.1C.1.2.2.1 Interruption time for RACH-based handover When intra-frequency or inter-frequency handover to NR SAN cell is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs. - Tprocessing is time for UE processing. Tprocessing can be up to 20ms. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and [10] ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]. - Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. 6.1C.1.2.2.2 Interruption time for RACH-less handover When intra-frequency or inter-frequency RACH-less handover to NR SAN cell is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is same as the one defined in section 6.1C.1.2.2.1. - T∆ is is same as the one defined in section 6.1C.1.2.2.1. - Tprocessing is same as the one defined in section 6.1C.1.2.2.1. - Tmargin is same as the one defined in section 6.1C.1.2.2.1. - TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH, dynamic grant based PUSCH, according to NW configuration and scheduling, or PRACH if no valid configured grant based PUSCH is selected. TIU can be up to the summation of SSB to PRACH occasion association period and [10] ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Additionally, as agreed in the last meeting for satellite switching delay requirements which is definitely dependent with the interruption time below.

6.1C.3.2.2 Interruption time for hard satellite switch with re-sync The interruption time is the time between t-service and the time the UE starts transmission of the new PRACH for RACH-based case or first UL transmission on PUSCH for RACH-less case if the UE only supports the feature for hard satellite switch and the hardSatelliteSwitch-Resync-NTN-r18 is enabled. When intra-frequency hard switch to NR SAN cell is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - T∆ is same as the one defined in section 6.1C.2.2.2.1. - Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

As defined in TS38.133[4，TS38.133 v18.6.0], we can see one of main contributors to decide the interruption time in 6.1C is Tsearch ,which is the time required to search the target NR SAN cell if the target cell is unknown when the handover command is received by the UE. In principle such cell search relies on PSS/SSS/PBCH detection. That is if the reduced PBCH used, obviously Tsearch here shall be revisited.

The other important factors which can impact the overall interruption time is T∆ for fine time tracking and acquiring full timing information of the target cell. In current NTN spec, T∆ is equal to Trs. . Similarly, in principle the tracking time is relies on PSS/SSS/PBCH detection performance which may depend on reference signal bandwidth.

Observation 2: The handover requirements in terms of Tinterrupt needs to be redefined because of the different Tsearch under the reduced PBCH bandwidth.

Proposal 2: The requirements for NTN paging interruption shall be redefined with extended Tsearch and T∆. How to extend them can be FFS (e.g. based on the simulation results for TN [7, R4-2310040]).

- RLM

In Rel18, as illustrated in the table below, the hypothetical PDCCH transmission parameter (e.g. 24PRBs BW) could be outside of channel bandwidth in case of less than 5MHz NTN [2].

Table 8.1C.2.1-1: PDCCH transmission parameters for out-of-sync evaluation [4]

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 8.1C.2.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Observation 3: For NTN less than 5MHz, the smaller BW needs to be used as the hypothetical PDCCH transmission parameter.

Similar conclusion can be conducted for the radio link recovery procedure. Thus, we suggest that:

Proposal 3: The requirements for RLM and RLF (including the hypothetical PDCCH transmission parameter) shall be revisited.

On the other hand, regarding to CSI-RS based RLM and RLF procedure would be impacted by the reduced bandwidth PDCCH as agreed in TS38.211, the current requirements for NTN RLM/RLF in TS38.133[] needs to be reconsidered also.

Observation 4: NTN CSI-RS based RLM/RLF requirements in TS38.133[4] needs to be reconsidered when the less than 5MHz bandwidth introduced.

However, since the similar issues on TN RLM/RLF with less than 5MHz was discussed and the finial conclusion is to deprioritized CSI-RS based RLM/RLF requirements because of the heavy working load [6]. For NTN RLM/RLF with less than 5MHz, we have same concerns especially the specification quality was expected to be improved as RAN task. Therefore, we suggest that:

R4#108 Agreement: [6, R4-2314273] Issue 1-1: Define CSI-RS-based L1 measurement requirements? - Do not define CSI-RS-based L1 measurements, RLM, BFD, CBD requirements in Rel-18. Based on this agreement, Issues 1-6, 1-7, 1-11 and 1-12 are closed. Additionally, Issues 1-20, 1-21, 1-25 and 1-26 are closed.

Also in the last RAN4 meeting, CSI-based L3 measurement with less than 5MHz NTN was deprioritized.

Issue 1-1-4: Other enhanced aspects beyond Rel18 Agreement: For less than 5MHz NTN in this WI: - Not define requirements for positioning, CGI, mTRP, CSI-RS based L3 measurement. - Not define requirements for Redcap.

Therefore, we can propose that:

Proposal 4: RAN4 can deprioritize CSI-based L1 measurements, RLM, BFD, CBD requirements for NTN with less than 5MHz.

- Measurement reporting period

For the measurement delay requirement, one of key components was the cell search and report the associated SSB index if the cell is unknown.

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Similar as the cell detection procedure in handover, we observed that:

Observation 5: the performance degradation with the less PBCH bandwidth is expected when performing intra/inter-frequency measurement in less than 5M Hz NTN.

Thus, we also suggest that:

Proposal 5: The requirements for measurement delay (e.g. time period for time index detection) shall be revisited.

- UE timing

The UE timing requirements including UE transmitter timing, MTTD/MRTD, TA was one of the most critical issues for NTN beyond Rel18. The main concerns for NTN timing error requirements are come from the high Doppler shift instead of the channel bandwidth. That is we didn’t observe the urgent necessity to redefine them just because of reduced channel bandwidth.

Observation 6: The current requirements for NTN timing can be reused for the less than 5MHz NTN.

In conclusion, in order to clear understanding the RRM works for this WI, the overall requirements for NTN RRM requirements defined in the current 38.133 are summarized in the table below.

Table 1. RRM impacts summary due to spectrum less than 5MHz

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Possible impacts if BW below 5MHz |
| --- | --- | --- |
| IDLE/inactive mode mobility | 4.2C Cell Re-selection for NR UE for Satellite Access | No impact on the core part but the test cases needs to be updated. |
| Handover | 6.1C Handover for SAN | FFS |
| UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy | 7.1C, 7.2C, 7.3C | No impact |
| RLM | 8.1C Radio Link Monitoring for Satellite Access 8.5C Link Recovery Procedures for Satellite Access | FFS on Hypothetical PDCCH transmission parameter |
| Measurement cell identification/measurement delay in RRC connected mode | 9.2C NR intra-frequency measurements for SAN 9.3C NR inter-frequency measurements for SAN | FFS on SSB based Cell identification/measurement delay |

- Conclusion

In this contribution, serval issues related to the RRM requirements under less than 5MHz NTN in Rel19 are discussed. The observations and proposals can be summarized as:

Proposal 1: RAN4 can start the RRM impact analysis with the following assumptions at least based on Rel18 TN less than 5MHz WI:

- 15RBs maximum transmission bandwidth configuration (NRB) for 3M channel bandwidth [4]

- 12RBs for PBCH with 3MHz channel bandwidth [4]

- 15RBs forming CORESET 0 with 3MHz channel bandwidth[4]

Observation 1: The RRM requirements for NTN in Rel18[4, TS38.133] depending on PBCH allocation will be impacted due to reduced channel bandwidth.

Observation 2: The handover requirements in terms of Tinterrupt needs to be redefined because of the different Tsearch under the reduced PBCH bandwidth.

Proposal 2: The requirements for NTN paging interruption shall be redefined with extended Tsearch and T∆. How to extend them can be FFS.(e.g. based on the simulation results for TN [7,R4-2310040	])

Observation 3: For NTN less than 5MHz, the smaller BW needs to be used as the hypothetical PDCCH transmission parameter.

Proposal 3: The requirements for RLM and RLF (including the hypothetical PDCCH transmission parameter) shall be revisited.

Observation 4: NTN CSI-RS based RLM/RLF requirements in TS38.133[4] needs to be reconsidered when the less than 5MHz bandwidth introduced.

Proposal 4: RAN4 can deprioritize CSI-based L1 measurements, RLM, BFD, CBD requirements for NTN with less than 5MHz.

Observation 5: the performance degradation with the less PBCH bandwidth is expected when performing intra/inter-frequency measurement in less than 5M Hz NTN.

Proposal 5: The requirements for measurement delay (e.g. time period for time index detection) shall be revisited.

Observation 6: The current requirements for NTN timing can be reused for the less than 5MHz NTN.

- References

- RP-240857

- R4-2413974

- R4-2413528

- TS38.133 v18.5.0

- TS38.211 v18.3.0

- R4-2314273

- R4-2310040