3GPP TSG-RAN WG4 Meeting #112bis			                                           R4-2415060

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.1

Title: 	Discussion on OD-SSB requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and the objectives are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

And in the last RAN4 meeting, the agreements for NES RRM were achieved in [4, R4-2413885]. Therefore, in this discussion, we provided more preliminary analysis on the potential RRM impacts due to these aspects in Rel19 NES.

- Discussion

  - On-demand SSB SCell

    - General scenarios

In the last RAN4 meeting, the general consideration on OD-SSB scenario in which the RRM requirements impact shall be studied in RAN4 was agreed.

Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement.

Meanwhile, according to previous RAN1 discussion below, these OD-SSB operation scenarios in RAN1 are illustrated in the figures below.

R1#116bis Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

For Scenario 3A/3B as illustrated in Figure 1, it is possible that there is not any activated OD-SSB available after UE reception on SCell activation command. And since the current RAN4 RRM requirements on SCell activation delay are rely on SCell activation command reception as a start point, the significantly impacts on the existing Scell activation delay requirements can be expected. Both SCell activation command reception and OD-SSB trigger command will be taken count into the start point of Scell activation. For an example, the later one can be taken as the start point of Scell activation delay or the uncertainty margin between them shall be added to the whole delay.

Figure 1.

TS38.133 v18.5.0 8.3.2 SCell Activation Delay Requirement for Deactivated SCell The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length ,

Observation 1: For OD-SSB scenario 3A/3B, the significant RRM requirement impacts on current RAN4 spec [2, TS38.133] are expected.

Meanwhile, up to now RAN1 has not the on-demand SSB SCell operation scenarios 3A/3B are still open.

Observation 2: The on-demand SSB SCell operation scenarios 3A/3B are still open.

That is RAN4 can also wait RAN1’s progress to avoid unnecessary working loads. Thus, we can propose that:

Proposal 1: RAN4 can start the discussion on RRM requirements for the scenarios 3A/3B of on-demand SSB upon RAN1’s further agreements.

    - OD-SSB type

Currently, whether on-demand SSB is CD-SSB or not is still under RAN1’s discussion. The agreements below was achieved.

Agreement (RAN1#116bis) - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

From RAN4 RRM requirement perspective (e.g. measurement based on SSB) whether SSB is CD-SSB or NCD-SSB can be same in terms of the measurement reporting delay itself. The possible impacts on the current spec based on always-on SSB is some applicable rules needed.

Observation 3: RAN4’s requirement on SCell SSB measurement is agnostic with CD/NCD-SSB type.

Generally RAN4 can start to discuss OD-SSB RRM requirements based on NCD firstly.

Proposal 2: RAN4 should start working on OD-SSB requirements for NCD-SSB. Defining OD-SSB requirements for CD-SSB should be postponed till RAN1 has decided.

    - RRM requirements impacts

In WID, the on-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation. It is obviously that if there is no always-on SSB the SCell measurement requirements on the deactivated SCell (e.g. 9.2.5.1 in TS38.133) will be huge impacted because UE needs synchronized to SCell to be activated by SSB detection.

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

In the last RAN4 meeting, there is some debating on whether OD-SSB  based L3 neighbour cell measurements. However, according to the latest RAN1 agreement below, RAN4 needs to consider the OD-SSB based L3 measurements as well.

R1#118 Agreement: Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4

Proposal 3: RAN4 shall define the both L1 and L3 measurement requirements based on OD-SSB.

Additionally, all Scell activation/deactivation delay requirements in current spec [2, TS38.133] needs to be re-specified. But since there are quite complicated SCell activation/deactivation enhancements from Rel15 to Rel18 (e.g. the multiple Scell activation in Rel16, fast SCell activation in Rel17), in order to simplify RAN4 discussion we can take single SCell activation as the baseline to redefine the SCell activation delay requirements with OD SSB.

Proposal 4: RAN4 can start to redefine SCell activation/deactivation RRM requirements under single SCell activation firstly.

Therefore, the necessary RRM requirements updates due to OD-SSB in current TS38.133[2] can be summarized as:

Table 1. RRM impacts summary due to OD-SSB

| RRM requirements in TS38.133 | Updates needed |
| --- | --- |
| 8.3. SCell Activation Delay Requirement for Deactivated SCell | Yes |
| 8.3.3 SCell Deactivation Delay Requirement for Activated SCell | Yes |
| 9.2.5 Intra-frequency measurements without measurement gaps | Yes |
| 9.2.6 Intra-frequency measurements with measurement gaps | Yes |
| 9.2.7 Intra-frequency measurements with NCSG | Yes |
| 9.3.4 Inter-frequency measurement with measurement gaps | Yes |
| 9.3.5 Inter-frequency measurements | Yes |
| 9.5.4.1SSB based L1-RSRP Reporting | Yes |

For the detailed RRM requirements updates, in our view it needs more discussion under the different conditions and scenarios. For an example, as illustrated in Figure 2 below, for the case of “Scenario #2 and Case #1“ and known Scell activation case, if the first SSB transmitted after OD SSB trigger command can located before the time “n + THARQ+5ms+TfirstSSB +margin”, UE can follow the current behavior for Scell activation and corresponding requirements defined in TS38.133 for known cases can be reused. Otherwise, the requirements in current spec needs to be updated. However, for the unknown case since the more than one SSB needed for the SCell timing and frequency synchronization, the current requirements could not be reused with the condition above for known cases indeed.

Observation 4: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be differentiated.

Figure 2.

Proposal 5: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined.

Also, if there are more than one SSB within the time window of [ t0,t0 + n + THARQ+5ms+TfirstSSB +margin] , in order to save more power up to NW implementation, gNB needs only to forward the SSB of Scell which is closer to the end of this window.

Observation 5: RAN4 can also consider whether further power saving optimization method needs to be accounted when specifying SCell activation/deactivation requirements with on-demand SSB.

Nevertheless, beside the scenarios, RAN1 also discussed the always-on SSB is cell based or not (case 2).

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

That is it is possible UE can receive both non-cell-defined on-demand SSB and legacy SSB together for case 2. If this is implemented by gNB. From UE perspective, it is also up to UE to perform the measurement based either of them.

Observation 6: In order to allow UE’s implement flexibility (e.g. reduce UE’s power consumption with longer measurement cycle), it needs not require UE performing the SCell measurement with OD-SSB if both of OD-SSB and legacy SSB are available.

Especially, within a successful measurement reporting period in which at least 4 measurement samples required, if there are different types of SSB used, this will significantly impact RAN4 current requirements since there is not any requirements defined by the hybrid measurement cycle in TS38.133.

Figure 3.

Observation 7: In order to simplify RAN4 discussion, RAN4 can define the SCell activation measurement requirements base on a single SSB period cycle instead of hybrid ones.

In summary, we can suggest that:

Proposal 6: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon

- legacy SSB or on-demanding SSB if both of them are activated

- further power saving

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: For OD-SSB scenario 3A/3B, the significant RRM requirement impacts on current RAN4 spec [2, TS38.133] are expected.

Observation 2: The on-demand SSB SCell operation scenarios 3A/3B are still open.

Proposal 1: RAN4 can start the discussion on RRM requirements for the scenarios 3A/3B of on-demand SSB upon RAN1’s further agreements.

Observation 3: RAN4’s requirement on SCell SSB measurement is agnostic with CD/NCD-SSB type.

Proposal 2: RAN4 should start working on OD-SSB requirements for NCD-SSB. Defining OD-SSB requirements for CD-SSB should be postponed till RAN1 has decided.

Proposal 3: RAN4 shall define the both L1 and L3 measurement requirements based on OD-SSB.

Proposal 4: RAN4 can start to redefine SCell activation/deactivation RRM requirements under single SCell activation firstly.

Observation 4: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be differentiated.

Proposal 5: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined.

Observation 5: RAN4 can also consider whether further power saving optimization method needs to be accounted when specifying SCell activation/deactivation requirements with on-demand SSB.

Observation 6: In order to allow UE’s implement flexibility (e.g. reduce UE’s power consumption with longer measurement cycle), it needs not require UE performing the SCell measurement with OD-SSB if both of OD-SSB and legacy SSB are available.

Observation 7: In order to simplify RAN4 discussion, RAN4 can define the SCell activation measurement requirements base on a single SSB period cycle instead of hybrid ones.

Proposal 6: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon

- legacy SSB or on-demanding SSB if both of them are activated

- further power saving

- References

- RP-240830

- TS38.133 v18.5.0

- TS38.331 v17.1.0

- R4-2413885