3GPP TSG-RAN WG4 Meeting #112bis		                                  R4-2415063

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.15.2.2

Title: 	Discussion on FR2-1 SSB based L3 measurement delay reduction by CCSF optimization

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NR Radio Resource Management (RRM) Phase 5 was agreed and the objectives are self-contained as below.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

And in the last RAN4 meeting, there were some initial discussion mainly on WI scope [2]. In this contribution we focus on some issues for UE not in multiple-Rx simultaneous reception mode in [2].

- Discussion

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization [Way forward]: FFS: - Option a (ZTE, CMCC, Intel, CATT, MTK): - Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently. - Note: if option a is agreeable, then work on wording polishing based on option 1a/1b. - Option b (HW, MTK, Samsung, E///): - Rel-19 CSSF optimization applies for case that UE is not capable of Rel-18 multi-Rx simultaneous reception. - Option c (Apple, Nokia, QC): - Rel-19 CSSF optimization applies for CA/DC scenarios with at least two FR2 serving cells, independently of the UE support of multi-Rx capabilities. - Option d (newly added): - Same Rel-19 CSSF optimization applies for the both cases: (1) UE is not capable of Rel-18 and Rel-19 multi-Rx simultaneous reception, (2) UE is capable of Rel-18 or Rel-19 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

In the latest RANP meeting, the clarification on this objective was discussed and the revised WID agreed [2,RP-242380] as:

- The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

Thus, in RAN4 we can also confirm these clarifications from RAN TSG as well.

Proposal 1: RAN4 shall follow the agreements for RANP [2]. No needs any further discussion on scenario defined in WID.

At same time in the previous meeting the assumption of searcher number was extensively discussed without any agreements.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF [Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

As in the WID, the number of searcher assumed as the baseline is 2. Thus, we can also persist the agreements of WID and Option 2 can be supported.

Proposal 2: RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF.

Secondly, the other important issue which can impact the discussion scope is whether the EN/NE DC needs to be included. And in the last RAN4 meeting, the following agreements were achieved.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - Final applicability of the solutions can be FFS - EN-DC - EN-DC with FR2 only intra band CA - EN-DC with FR2 only inter band CA - EN-DC with FR1 +FR2 CA (FR1 PSCell) - EN-DC with FR1 +FR2 CA (FR2 PSCell) - NE-DC - NE-DC with FR2 only intra band CA - NE-DC with FR2 only inter band CA - NE-DC with FR1 +FR2 CA (FR1 PCell) - SA - FR1+FR2 CA (FR1 PCell) - FR1+FR2 CA (FR2 PCell) - FR2 only intra-band CA - FR2 only inter-band CA - NR-DC - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

Observation 1:  The working load of CCSF definition and optimization for EN/SA/NE are quite huge especially combining with:

- CA band combination

- gap-based/gap-less/NCSG

- SSB/CSI-RS/PRS

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. |  |  |  |  |  |  |  |

On the other hand, it is not easily to justify whether the same CCSF factor can be reused for others one by one.

Thus, in order to simplify RAN4’s work and improve the specification quality we suggest that:

Proposal 3: Rel-19 CSSF optimization can be prioritized for the following cases:

| Current requirements in TS38.133 | Priority |
| --- | --- |
| 9.1.5.1.1 EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps | Low |
| 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps | high |
| 9.1.5.1.3 NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps | Low |
| 9.1.5.1.4 NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps | Low |
| 9.1.5.2.1 EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps | Low |
| 9.1.5.2.2 SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps | high |
| 9.1.5.2.3 NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps | Low |
| 9.1.5.2.4 NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps | Low |
| 9.1.5.2.5 SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps | high |
| 9.1.5.2.6 NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps | Low |
| 9.1.5.2.7 NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps | Low |
| 9.1.5.3 Monitoring of multiple layers within NCSG | Low |

- Conclusion

In this contribution, serval issues related toFR2-1 SSB based L3 measurement delay reduction for connected mode by CCSF optimization in Rel19 are discussed. The observations and proposals can be summarized as:

Proposal 1: RAN4 shall follow the agreements for RANP [2]. No needs any further discussion on scenario defined in WID.

Proposal 2: RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF.

Observation 1:  The working load of CCSF definition and optimization for EN/SA/NE are quite huge especially combining with:

- CA band combination

- gap-based/gap-less/NCSG

- SSB/CSI-RS/PRS

Proposal 3: Rel-19 CSSF optimization can be prioritized for the following cases:

| Current requirements in TS38.133 | Priority |
| --- | --- |
| 9.1.5.1.1 EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps | Low |
| 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps | high |
| 9.1.5.1.3 NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps | Low |
| 9.1.5.1.4 NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps | Low |
| 9.1.5.2.1 EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps | Low |
| 9.1.5.2.2 SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps | high |
| 9.1.5.2.3 NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps | Low |
| 9.1.5.2.4 NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps | Low |
| 9.1.5.2.5 SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps | high |
| 9.1.5.2.6 NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps | Low |
| 9.1.5.2.7 NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps | Low |
| 9.1.5.3 Monitoring of multiple layers within NCSG | Low |

- References

- RP-240830

- RP-242380

- TS38.133 v18.6.0

- TS38.331 v17.1.0