# 3GPP TSG-RAN WG4 Meeting #112bis				                    R4-2415064

# Hefei, Anhui, China, 14th - 18th October, 2024

Source: 	Xiaomi

Title: 	Initial discussion on Study of testability and interoperability for AI/ML mobility

Agenda Item:	6.18.3

Document for:	Discussion

# 1Introduction

In RAN#105 plenary meeting, the revised SID of “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR” has been approved in [1]. RAN4’s work was further refined as captured below:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we would like to provide some initial discussion on testability and interoperability for AI/ML mobility and provide our proposals.

- Discussion

RRM measurement prediction

RAN2 has discussed the RRM measurement prediction for several meetings, and has agreed to focus on L3 filtered cell level measurement, while deprioritize L3 filtered beam level prediction cases. Refer to the measurement model illustration in TS 38.300, we understand the L3 filtered cell-level measurement result is obtained in point C.

Fig1: Measurement Model in TS 38.300

3 sub-use cases listed below are considered for cell level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

And, to facilitate the discussion, RAN2 further classifies the evaluation scenarios and prioritizes them as follows:

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Proposal 1: For RRM measurement prediction, RAN4 to study the testability and interoperability for AI/ML mobility starting with the three prioritized scenarios:

- FR1 to FR1 inter-frequency intra-cell temporal domain case B

- FR1 to FR1 inter-frequency inter-cell frequency domain

- FR2 to FR2 intra-frequency intra-cell temporal domain case A

Based on the outcome from RAN2 study, measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

To verify the accuracy of the predicted L3 filtered cell-level measurement result, sufficient randomness and variation in time and spatial domain of Tx beam has to be configured in the test and emulated by the test equipment. For the scenarios of FR1 to FR1 inter-frequency intra-cell temporal domain case B and FR1 to FR1 inter-frequency inter-cell frequency domain, we understand that Tx beam number=1 can be assumed as the model input. While, for the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, the maximum numbers of beams to be tested that TE is capable to emulate in the testing environment may need to be further discussed. From our point of view, Tx beam number=8 can be considered as a starting point in this case.

Proposal 2: For RRM measurement prediction, RAN4 to study the maximum numbers of Tx beam that has to be configured in the test and emulated by the test equipment for FR1 and FR2 separately.

- The next issue is about how to obtain the ground truth value.

- Similar as the discussion in NR_AIML_Air WI, the channel model assumption for RRM measurement prediction need to be agreed first. There are two channel model assumptions: AWGN channel and Fading channel. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain. While, the Fading channel is a random channel, where variation in both spatial and time domain is included. AWGN channel model is not suitable since there is no any uncertainty in both spatial and time domain. It’s hard to verify the prediction functionality introduced by AI. Just as the progress achieved in Channel model assumption for AI based BM, we think the fading channel should be assumed in the test.

- For both the scenarios of FR1 to FR1 inter-frequency intra-cell temporal domain case B and FR1 to FR1 inter-frequency inter-cell frequency domain, we understand the variation in time domain would be enough for FR1 scenario, so the TDL channel model is preferred. While, the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, we prefer to use the CDL channel model.

Proposal 3: For RRM measurement prediction, RAN4 to study which fading channel should be used in the test, as this may differ between FR1 and FR2.

- Specifically, for the FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the multi-cell configuration need to be pay more attention. To verify the accuracy of the predicted L3 filtered cell-level measurement result in this scenario, multi-cell configuration in the test need to be further studied. We think the configuration in this case depends on the number of cells to be predicted, which needs to wait for RAN2 decision. If we only verify that case one cell to be measured and one cell to be predicted, we understand that the legacy test setup can be used as baseline. If we need to verify the case one cell to be measured and multiple cells to be predicted, we think the test setup need to be further studied.

Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

For the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, UE implementation of RX beam sweeping should be considered. Similar discussion also happened in NR_AIML_Air WI. For multiple-AOA test setup,  multiple directions might be addressed by the DUT with different antennas, so that RF baseline system assumptions are not valid. It’s more challenging to obtain accurate RSRP measurement as ground truth.

Proposal 5: For the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, multi-AoA setup in the test need to be further studied.

Measurement event prediction

Based on current RAN2 progress, study on measurement event prediction will be started after some further progress on RRM measurement prediction has been made. Hence, the related testability and interoperability discussion in RAN4 can pending on RAN2 progress.

Proposal 6: For Measurement event prediction, wait for RAN2 input to further study testability and interoperability issue.

RLF/HO prediction

HOF prediction is down prioritized in RAN2.

For RLF prediction, RAN2 has achieved initial agreements which are attached below:

Agreements

-	Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using

-	Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

-	Companies should report the prediction time window they have used in the simulations

However, the RLF specific Simulation assumptions, simulation methodology etc are still FFS. We think RAN4 can wait for more RAN2 progress to open the discussion.

Proposal 7: For RLF prediction, wait for RAN2 input to further study testability and interoperability issue.

- Conclusion

Proposal 1: For RRM measurement prediction, RAN4 to study the testability and interoperability for AI/ML mobility starting with the three prioritized scenarios:

- FR1 to FR1 inter-frequency intra-cell temporal domain case B

- FR1 to FR1 inter-frequency inter-cell frequency domain

- FR2 to FR2 intra-frequency intra-cell temporal domain case A

Proposal 2: For RRM measurement prediction, RAN4 to study the maximum numbers of Tx beam that has to be configured in the test and emulated by the test equipment for FR1 and FR2 separately.

Proposal 3: For RRM measurement prediction, RAN4 to study which fading channel should be used in the test, as this may differ between FR1 and FR2.

Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

Proposal 5: For the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, multi-AoA setup in the test need to be further studied.

Proposal 6: For Measurement event prediction, wait for RAN2 input to further study testability and interoperability issue.

Proposal 7: For RLF prediction, wait for RAN2 input to further study testability and interoperability issue.

- Reference

- RP-242393, Revised SID: Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR, OPPO