# 3GPP TSG-RAN WG4 Meeting #112bis				                    R4-2415065

# Hefei, Anhui, China, 14th - 18th October, 2024

Source: 	Xiaomi

Title: 	Discussion on simulation assumptions and results for LP-WUS/WUR

Agenda Item:	6.23.4.1

Document for:	Discussion

# 1Introduction

In RAN4#112 meeting, the updated link level simulation assumptions for LP-SS evaluation were agree in [1].

In this contribution, our initial link level simulation results for OOK based LP-SS are provided.

- Discussion

- Simulation assumptions

The simulation assumptions are given in Table 1:

Table 1: General parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| LP-SS block BW | 132 subcarriers for SCS=30kHz |
| LP-SS | OOK-4 with M = 2,4 |
| LP-SS overhead | 2 OFDM symbols |
| LP-SS sequences | M sequence |
| BS transmit antennas for LP-SS blocks | 1 tx |
| UE receive antennas | 1 rx |
| Subcarrier spacing | 30 kHz |
| Number of LP-SS blocks per SS burst set, K | 1 |
| SNR | -3dB |
| Propagation conditions | TDL-C 300ns |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| LP-SS (OOK based LP-WUR receiver) |  | OOK-4 with M = 2,4 | when Cell 1 uses OOK-4, OOK-4 is used for Cell 2. |
| LP-SS pattern |  | M sequence | M sequence |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OOK: 10 ppm | N/A |
| Timing error |  | Residual timing error: 2us |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | Set 1: SNR = -2.7 Set 2: SNR = -2.4 | Set 1: SNR = -11.7 Set 2: SNR = -8.4 |
| Ês/Iot | dB | -3 | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | FR1: 3 km/h |  |

- Simulation results

Table 3: simulation results for OOK-4, M=2, sync

|  |  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- | --- |
| Set1: [-2.7, -11.7] | AWGN | 1 sample | -2.64 | 1.04 |
|  |  | 2 sample | -1.85 | 0.68 |
|  |  | 3 sample | -1.57 | 0.48 |
|  | TDL-C | 1 sample | -4.26 | 3.50 |
|  |  | 2 sample | -3.30 | 2.39 |
|  |  | 3 sample | -3.07 | 1.97 |
| Set1: [-2.4, -8.4] | AWGN | 1 sample | -2.50 | 1.07 |
|  |  | 2 sample | -1.73 | 0.70 |
|  |  | 3 sample | -1.43 | 0.51 |
|  | TDL-C | 1 sample | -4.08 | 3.14 |
|  |  | 2 sample | -3.20 | 2.43 |
|  |  | 3 sample | -2.97 | 1.97 |

Table 4: simulation results for OOK-4, M=4, sync

|  |  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- | --- |
| Set1: [-2.7, -11.7] | AWGN | 1 sample | -3.40 | 1.33 |
|  |  | 2 sample | -2.31 | 0.92 |
|  |  | 3 sample | -1.92 | 0.71 |
|  | TDL-C | 1 sample | -4.90 | 3.28 |
|  |  | 2 sample | -3.39 | 2.53 |
|  |  | 3 sample | -3.79 | 2.00 |
| Set1: [-2.4, -8.4] | AWGN | 1 sample | -2.96 | 1.58 |
|  |  | 2 sample | -2.02 | 1.11 |
|  |  | 3 sample | -1.61 | 0.90 |
|  | TDL-C | 1 sample | -4.39 | 3.29 |
|  |  | 2 sample | -3.41 | 2.49 |
|  |  | 3 sample | -2.87 | 1.89 |

Based on evaluation results shown in previous section, we could make following observations:

- For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-4 with M=2 and OOK-4 with M=4;

- Considering the potential interference as well as UE implementation(RF) margin, we think measurement with 3 samples can achieve the CA/DC Idle Mode Measurements accuracy requirements (e.g., +/- 6 dB) in all evaluated cases.

Observation 1: For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-1, OOK-4 with M=2 and OOK-4 with M=4.

Observation 2: Assuming accuracy requirements of +/- 6 dB, measurement with 3 LP-SS samples is sufficient at SNR = -3dB in TDL-C channel.

- Conclusion

Observation 1: For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-1, OOK-4 with M=2 and OOK-4 with M=4.

Observation 2: Assuming accuracy requirements of +/- 6 dB, measurement with 3 LP-SS samples is sufficient at SNR = -3dB in TDL-C channel.

- Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Vivo