# 3GPP TSG-RAN WG4 Meeting #112bis				                    R4-2415066

# Hefei, Anhui, China, 14th - 18th October, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	6.23.4.2

Document for:	Discussion

# 1Introduction

In RAN#103 meeting, the latest revised WID on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved [1]. The related RRM work was initially discussed in last meeting. A WF was achieved and approved [2]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

- LP-WUR requirements at RRC_IDLE/INACTIVE state

- General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Based on the discussion in previous meetings, RRM measurement relaxation case#1 and case#3 has already be agreed.

In case 1, i.e., fully offloading case, MR stops perform any RRM measurement and LP-WUR performs serving cell measurement. There is still FFS on the condition and details for the MR neighboring cell measurement. From our perspective, this is the case where UE could perform the greatest level of relaxation measurement. We prefer not to consider any neighbor cell measurement in this case.

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case #1, except for the higher priority frequency layer search.

In case 3, MR has RRM relaxation on serving cell and neighbour cell measurement and LP-WUR performs serving cell measurement. There is still FFS on whether to combine the serving cell measurements from the two radios, i.e. MR and LR. Firstly, we understand the measurements combination across MR and LR refers to the serving cell measurement results obtained by filtering the measurement samples from both MR and LR. In this case, we don’t think it is reasonable to consider combining the measurements across the two radios.

Proposal 2: For RRM relaxation case #3, RAN4 not to consider combining the measurement samples across the MR and LR to get serving cell measurement results.

In case 2, MR has RRM relaxation on serving cell measurement (no neighbour cell measurement is needed) and LP-WUR performs serving cell measurement. We think this is the case UE in the cell center, when UE does not need to measure neighbor cell, as well as no higher priority inter-f or inter-RAT configured. From our perspective, this is the typical case for starting LP-WUR monitoring and no need to start MR measurement. So, we prefer to de-prioritize this case.

Proposal 3: It is suggested to de-prioritize RRM relaxation case #2.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

Based on RAN1 progress, there would be two types of LP-WUR, one is capable of OOK envelope detection and the other one is capable of OFDM sequence detection. For LP-WUR with OOK envelope detection, i.e. OOK-based LP-WUR, only time domain processing could be conducted. The measurement performance is expected to be deteriorated compared with the LP-WUR with OFDM sequence detection, i.e. OFDM-based LP-WUR when conducting LP-SS based measurement with overlaid OFDM symbols. While for the OFDM based LP-WUR, we also think the measurement performance might be different for LP-SS with overlaid OFDM sequences and PSS/SSS considering the different OFDM sequence design. From our perspective, the impact on requirements due to different LP-WUR type and different reference signal, i.e., newly designed LP-SS (with/without overlaid OFDM sequences) and existing PSS/SSS should be studied. As the design for LP-SS with overlaid OFDM sequences is still not determined in RAN1, RAN4 can suspend study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions.

Proposal 4: RAN4 to suspend study on measurement requirements for OFDM based LP-WUR based on LP-SS  in idle/inactive state until more RAN1 conclusions.

Proposal 5: RAN4 to define measurement requirements for:

- OOK-based LP-WUR serving cell measurement based on LP-SS in Idle/Inactive state, and

- OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS in Idle/Inactive state.

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Considering the core requirements to be specified for MR RRM relaxation, we support to discuss MR measurement with relaxation for both serving cell and neighbor cell.

Proposal 6: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements in idle/inactive mode for UE supporting LP-WUR.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

In last meeting, companies showed their opinions on the entry/exist conditions for LP-WUR related operations, including LP-WUR measurement, LP-WUS monitoring and MR RRM measurement relaxation.

For the entry/exist conditions for LP-WUS monitoring, we understand that this issue is under discussion in RAN1. We think RAN4 should wait for RAN1 progress. RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered RAN1 if needed.

Proposal 7: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1/2 if needed.

As we understand that the LP-WUR measurement is initially introduced to guarantee the performance of LP-WUS monitoring, we think the measurement is not expected to start later than the LP-WUS monitoring. Also, we see little advantage in early starting of the LR measurement. If UE still use MR for paging reception, it is reasonable to use MR for RRM measurements as MR also need measurement for T/F synchronization. In this case, the LR measuremnt results seems useless. So, we prefer to define the same entry/exit condition of LP-WUR measurement and LP-WUS monitoring.

Proposal 8: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring.

RAN4 also discuss the entry/exit conditions for MR RRM measurement relaxation. Based on our analysis in the previous section, we think it is more reasonable that the MR RRM relaxation is executed when LR is used for serving cell measurement. If the LR is not doing measurement, the evaluation of measurement relaxation can only depend on serving cell measurement in MR, which is aligned with Rel-16 power saving functionality. If the LR is doing RRM measurement while MR is also doing normal RRM measurement, there seems little benefit but more power consumption. So, from our perspective, the entry/exit conditions for MR RRM measurement relaxation should also be the same as LP-WUR serving cell measurement. However, we understand RAN2 is currently working on the criteria design, it is suggest RAN4 to focus on the relaxation cases.

Proposal 9: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Issue 1-2-4: LP-WUR operating carrier frequency

One LS was sent by RAN2 during last meeting.

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Basically, the current RRM discussion focus on the delay and accuracy requirements for the LR. These requirements are based on the LP-SS or PSS/SSS, and not related to the operating band.

Proposal 10: From RRM perspective, no issue and specification support needed are identified if LR does not support LP-WUS reception on all the bands supported by the UE.

Issue 1-1-9: LP-WUR status

LP-WUR status before entering offloading or after exiting offloading is also under discussion. In our understanding, this issue is related to the enter/exist conditions for LP-WUS entry/exit condition. Based on our knowledge, RAN2 is considering configure both MR-based thresholds and LR-based thresholds for LP-WUS entry/exit evaluation. If the LP-WUS entry/exit condition is agreed that both MR measurement results and LR measurement results are needed, we think the LP-WUR status should be on. At the present stage, we think RAN4 can wait for further progress.

Proposal 11: The discussion on LP-WUR status before entering offloading or after exiting offloading can be pending until further conclusions from RAN1/2 are available.

Issue 1-1-11: Considerations on higher priority frequency layer

In accordance with the legacy measurement procedure for MR only UE, measurements for neighbor cell could be divided into two parts. One is for intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority, and the decision for MR to conduct these measurements hinges on the results from the serving cell measurements. When the signal quality of the serving cell reduce to certain threshold, the intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority will be triggered. The other one is for NR inter-frequency or inter-RAT frequencies with higher priority, and the decision for MR to conduct the measurements depend on NW configuration. In this case, as long as NW configuration indicates the priority information, UE needs to perform measurements of higher priority NR inter-frequency or inter-RAT frequencies.

Observation 1: According to existing cell selection/reselection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

We think the handling on NR inter-frequency or inter-RAT frequencies measurements with higher priority should also follow the legacy.

- Proposal 12: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case#1 and case#3.

- MR RRM relaxation

Based on the conclusion reached in study phrase, UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbor cells are applied and UE MR enters in ultra-deep sleep during LP-WUS monitoring. To obtain enough power saving gain, one of the objectives in the WI is to “specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR”. In Rel-16 power saving WI, relaxed RRM measurement requirements for neighbor cell in RRC_IDLE/INACTIVE mode are introduced. UE meeting the RRM relaxation criteria is allowed to conduct measurements with longer intervals. Generally, we agree that the Rel-16 relaxation method could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR.

Proposal 13: The Rel-16 relaxation method, i.e., extending measurement interval, could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR. The detailed scaling factor can be discussed after RAN4 have concrete agreements on the MR RRM relaxation cases.

- Accuracy

During discussion in previous meeting, it was agreed to wait RAN1 progress in target SNR/SINR and use the RAN1 agreed target SNR/SINR as baseline for RRM simulation assumption. Meanwhile, companies still have different understanding on whether/how to define the accuracy. From our perspective, we support the idea that the accuracy discussed here is for simulation assumption and no need to define dedicated accuracy requirement in performance section. As per the existing serving cell measurement requirements defined in RRC_IDLE/INACTIVE state, no corresponding measurement accuracy requirement introduced. Since we are discussing RRM requirements for LP-WUR in RRC_IDLE/INACTIVE state, we prefer to follow the same logic as for MR.

Proposal 14: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Following up on RAN1#117 agreement, RAN1 has agrees several candidate SNR values for LP-WUS and LP-SS design:

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

These candidate SNR values can be used for RRM simulation evaluation. We provide our initial simulation results at SNR=-3 dB in the accompany contribution [3].

The requirements of measurement accuracy, side condition of SNR and measurement delay are to be decided in RAN4, and they are interconnected. Considering the serving cell RRM measurement results play a significant role on the mobility schemes, we prefer that RAN4 start the work from the measurement accuracy requirements. Based on the desired accuracy, the number of the reference signal samples could be decided. Then, the measurement period requirements are derived the number of reference signal samples within a given time.

The LP-SS is used for serving cell measurement. In our view, the accuracy target for LP-WUR RRM measurement should be comparable to the SSB based RRM measurement for the main receiver, in order to avoid UE performance degradation. Based on the existing performance requirements, RAN4 has defined intra-frequency RSRP/RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements in clause 10.1.2B and10.1.7B for UE capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers. As the LP-WUR is only expected to perform RRM serving cell measurement in RRC idle mode, we prefer to take the accuracy requirements as for CA/DC Idle Mode Measurements, i.e., ±6dB RSRP measurement accuracy and ±4dB RSRQ measurement accuracy, as baseline.

Proposal 15: It is suggested to take the accuracy requirement defined for CA/DC Idle Mode Measurements, i.e., ±6dB RSRP measurement accuracy and ±4dB RSRQ measurement accuracy, as the starting point when determining the measurement accuracy in RRC_IDLE/INACTIVE state for LP-WUR serving cell measurement.

- LP-WUR requirements at RRC_CONNECTED mode

In RRC_CONNECTED mode, as clearly described in the objectives, UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think there should be no RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode expected. However, we are fine not to jumps to conclusions in this early stage.

Proposal 16: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Conclusion

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case #1, except for the higher priority frequency layer search.

Proposal 2: For RRM relaxation case #3, RAN4 not to consider combining the measurement samples across the MR and LR to get serving cell measurement results.

Proposal 3: It is suggested to de-prioritize RRM relaxation case #2.

Proposal 4: RAN4 to suspend study on measurement requirements for OFDM based LP-WUR based on LP-SS  in idle/inactive state until more RAN1 conclusions.

Proposal 5: RAN4 to define measurement requirements for:

- OOK-based LP-WUR serving cell measurement based on LP-SS in Idle/Inactive state, and

- OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS in Idle/Inactive state.

Proposal 6: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements in idle/inactive mode for UE supporting LP-WUR.

Proposal 7: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1/2 if needed.

Proposal 8: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring.

Proposal 9: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Proposal 10: From RRM perspective, no issue and specification support needed are identified if LR does not support LP-WUS reception on all the bands supported by the UE.

Proposal 11: The discussion on LP-WUR status before entering offloading or after exiting offloading can be pending until further conclusions from RAN1/2 are available.

Observation 1: According to existing cell selection/reselection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

We think the handling on NR inter-frequency or inter-RAT frequencies measurements with higher priority should also follow the legacy.

- Proposal 12: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case#1 and case#3.

Proposal 13: The Rel-16 relaxation method, i.e., extending measurement interval, could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR. The detailed scaling factor can be discussed after RAN4 have concrete agreements on the MR RRM relaxation cases.

Proposal 14: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Proposal 15: It is suggested to take the accuracy requirement defined for CA/DC Idle Mode Measurements, i.e., ±6dB RSRP measurement accuracy and ±4dB RSRQ measurement accuracy, as the starting point when determining the measurement accuracy in RRC_IDLE/INACTIVE state for LP-WUR serving cell measurement.

Proposal 16: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Vivo

- R4-2414909, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1