# 3GPP TSG-RAN WG4 Meeting #112bis				                    R4-2415067

# Hefei, Anhui, China, 14th - 18th October, 2024

Source: 	Xiaomi

Title: 	Initial discussion on Conditional Intra-CU LTM for NR mobility enhancements Phase 4

Agenda Item:	6.24.2.2

Document for:	Discussion

# 1Introduction

In RAN#105 plenary meeting, the revised WID of “NR mobility enhancements Phase 4” has been approved in [1]. The support of conditional Intra-CU LTM was confirmed, and the detailed objective is captured below:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we would like to focus on the uplink capacity enhancements and provide some initial discussion on the potential RRM impact.

- Discussion

In Rel-18, LTM was introduced, offering the advantage of reduced interruption time. In Rel-19 NR mobility enhancement, conditional intra-CU LTM is agreed to be supported. We understand that this conditional intra-CU LTM may be akin to CHO or CPAC, as the procedure can be executed without the need for a signaling exchange with the source cell beforehand. We think related RRM requirements should be specified in RAN4.

Basically, we suppose that RAN4 need to define the delay requirements for conditional LTM Cell Switch. In Rel-18, delay requirements for LTM cell switch were specified. LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch. When conditional LTM Cell Switch is supported, at least the starting point of the conditional LTM Cell Switch should be specified. And then, the components of the conditional LTM cell switch delay should be further determined.

Furthermore, both early DL synchronization and early UL synchronization are supportedin Rel-18 intra-CU LTM. To minimize the interruption of conditional LTM, we suppose that both early DL synchronization and early UL synchronization can be considered in the conditional LTM. However, the conclusion depend on progress in other WGs.

Proposal 1: RAN4 to specify RRM requirements for the newly introduced conditional intra-CU LTM, while detailed discussions should be deferred until further progress is made in other working groups.

- Conclusion

Proposal 1: RAN4 to specify RRM requirements for the newly introduced conditional intra-CU LTM, while detailed discussions should be deferred until further progress is made in other working groups.

- Reference

- RP-242356	, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom