# 3GPP TSG-RAN WG4 Meeting #112bis				                    R4-2415068

# Hefei, Anhui, China, 14th - 18th October, 2024

Source: 	Xiaomi

Title: 	Discussion on (e)RedCap RRM requirements in NTN for NR Phase 3

Agenda Item:	6.26.4.1

Document for:	Discussion

# 1Introduction

In the last RAN4#112 meeting, an initial discussion on RRM requirements for Rel-19 NR NTN phase3 occurred and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on (e)RedCap RRM requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

General consideration on RRM requirements

#### Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN Online agreement <Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps <Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

In last meeting, discussion on what RRM requirements to be defined for (e)RedCap UE with FR1-NTN was initiated. Companies reached consensus on most of the requirements, while several options remain open for further discussion. We want to share our opinions on the NR Rel-17/18 Conditional Handover and RACH-less Handover. In our understanding, these two features are not exclusive to NTN from signaling configuration point of view. So, we think NR Rel-17/18 Conditional Handover and RACH-less Handover are applicable to RedCap NTN.

Proposal 1: RAN4 to define RRM requirements for (e)RedCap UEs with NR NTN on Conditional Handover and RACH-less Handover.

Reduced UE complexity

#### Issue 6-3-1: The principle for defining the requirements for 2Rx/1Rx (e)Redcap UE with FR1-NTN Online agreement <Agreement> - For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx. - RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE, if needed. - As baseline, follow the same principle for (e)RedCap UE in TN network. - For 2Rx (e)RedCap UEs with FR1-NTN: Reuse the existing requirements for NTN as a baseline. - Meanwhile, consider the following difference between 2Rx (e)RedCap and non-RedCap UE: - The number of searchers - Others if any #### Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ? <Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

Based on Rel-17 and Rel-18 progress, the main characteristic for RedCap UE is the reduced UE complexity. RedCap UE supports 1 or 2 UE Rx branches and HD-FDD or FD-FDD operation. Based on last meeting progress, it was agreed to follow the same principle for (e)RedCap UE in TN network for (e)RedCap UEs with FR1-NTN.

Besides some additional clarification on available samples for HD-FDD operation, the same RRM requirements as for normal NR UE are reused for Rel-17 RedCap with 2Rx, while the relaxed measurement requirements are defined for Rel-17 RedCap with 1Rx. The relaxation method including extending the number of measurement samples and relaxing the measurement accuracy. RAN4 has also agreed to reuse the existing requirements for NTN as a baseline for 2Rx (e)RedCap UEs with FR1-NTN. So, from our perspective, when defining RRM requirements for 1RX (e)RedCap UEs with FR1-NTN, similar principle as 1RX RedCap in TN network could be taken.

- Proposal 2: RAN4 to define RRM requirements for 1 RX (e)RedCap with FR1-NTN by relaxing requirements of 2 RX RedCap with FR1-NTN, such as extending the number of measurement samples or relaxing the measurement accuracy.

HD-FDD

For HD-FDD, it was agreed that the TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point. Currently, RAN1 is still working on evaluating the applicability of the existing handling rules for HD collisions cases, considering the TA mismatch between actual TA used by UE and assumed TA at the gNB. RAN4 can waiting RAN1’s final decision to check whether there is new RRM impact.

Proposal 3: RAN4 to check whether existing HD-FDD related requirements can be reused or not after RAN1 reaching further conclusions.

eDRX

The Rel-17 RedCap WI introduced extended DRX cycles for RRC Idle state (up to 10485.76 seconds) and RRC Inactive state (up to 10.24 seconds). Furthermore, the Rel-18 eRedCap WI introduced enhanced eDRX cycle length in RRC Inactive state beyond 10.24 seconds, up to the same eDRX cycle length as for RRC Idle state (up to 10485.76 seconds). During discussion in the (e)RedCap WI, the eDRX feature is introduced as an optional feature for both RedCap and non-RedCap UEs. As the eDRX feature is not specific for (e)RedCap UE, we think RAN4 should discuss whether to support eDRX feature for NR NTN.

In previous Rel-17 and Rel-18 NTN WI, cell reselection requirements are defined based on DRX cycle. It as agreed that the UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment, considering the limited serving time in earth-moving LEO scenario.

So, from our perspective, RAN4 can discuss and specify the requirements related to eDRX for GEO and LEO deployments separately. At least for earth-moving LEO development, we think the eDRX feature is not applicable.

Proposal 4: RAN4 to define measurement requirements for eDRX feature in GEO scenario.

RRM relaxation

RRM measurement relaxation for neighbour cells is introduced as an optional feature for RedCap UEs that can be enabled by the network in both RRC idle/inactive states and RRC connected state. From RRM perspective, the spec impact only exists in RRC idle/inactive states. The UE is allowed to further relax neighbour-cell RRM measurements (compared to existing Rel-16 relaxation functionality) when an RSRP/RSRQ-based stationarity criterion is met for a period of time, or when both the stationarity criterion and a not-at-cell-edge criterion are met.

Based on the current TS 38.133, NTN has already supported Rel-16 relaxation functionality.

#### 4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO. #### 4.2C.2.8 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.10 apply provided that target cell’s satellite is GEO.

If we follow the same logic as in Rel-17 NTN discussion, the requirements in clause 4.2B.2.9 and 4.2B.2.10 for RRM measurement relaxation for Redcap could apply provided that target cell’s satellite is GEO.

Proposal 5: RAN4 to define relaxed measurement requirements for RedCap UE configured with Rel-17 relaxation criteria in GEO scenario.

- Conclusion

Proposal 1: RAN4 to define RRM requirements for (e)RedCap UEs with NR NTN on Conditional Handover and RACH-less Handover.

- Proposal 2: RAN4 to define RRM requirements for 1 RX (e)RedCap with FR1-NTN by relaxing requirements of 2 RX RedCap with FR1-NTN, such as extending the number of measurement samples or relaxing the measurement accuracy.

Proposal 3: RAN4 to check whether existing HD-FDD related requirements can be reused or not after RAN1 reaching further conclusions.

Proposal 4: RAN4 to define measurement requirements for eDRX feature in GEO scenario.

Proposal 5: RAN4 to define relaxed measurement requirements for RedCap UE configured with Rel-17 relaxation criteria in GEO scenario.

- Reference

- R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT