# 3GPP TSG-RAN WG4 Meeting #112bis				                    R4-2415069

# Hefei, Anhui, China, 14th - 18th October, 2024

Source: 	Xiaomi

Title: 	Discussion on other RRM core requirements in NTN for NR Phase 3

Agenda Item:	6.26.4.2

Document for:	Discussion

# 1Introduction

In the last RAN4#112 meeting, an initial discussion on RRM requirements for Rel-19 NR NTN phase3 occurred and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on other RRM core requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

NR-NTN downlink coverage enhancement

#### Issue 2-2-1: RRM impact of DTX/DRX <Way Forward> : Further discuss the following proposal - Proposal 1 (Apple): - RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES. - Proposal 2 (Samsung): - All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs. - Proposal 3 (Ericsson): - RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of DRX periodicity. #### Issue 2-3-1: RRM impact of dynamic and flexible power sharing <Way Forward> : Further discuss the following proposal - Proposal 1 (LG): - After RAN1 concludes the solution for dynamic and flexible power sharing among satellite beams or different satellite beam patterns/size, RAN4 should check whether the outcome from RAN1 is affect RRM requirements. - Proposal 2 (Xiaomi): - RAN4 to study the RRM impact of satellite beams activation/deactivation scheme after RAN1/2 reaching more concrete conclusions.

In Rel-19, satellite may be unable to have all its beams active with the nominal EIRP density per beam at a given time, in order to balance the DL coverage and satellite payload. To ensure that all UEs can be served across the satellite foot print while maximizing the overall satellite throughput, the nominal EIRP density may separate by several beams as multiple beams are activated simultaneously.

Based on current progress, RAN1 is conduction evaluation considering three different beam states:

Agreement For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

The scheme of beams activation/deactivation is still under discussion in RAN1 and RAN2. For example, one of the methods currently considering is the cell DTX/DRX like mechanism.

We understand that the beams activation/deactivation would have RRM impact. Currently, measurement requirements in both IDLE/INACTIVE mode and CONNECTED mode are basically based on SSB or DRX configuration. If cell DTX/DRX like mechanism is used to indicate beam status, the cell DTX/DRX configuration shall also be considered in the measurement delay requirements. Also, we think that the satellite beam transition between different states may have impact on mobility requirements and the corresponding UE behavior should be further studied. However, since the conclusions from RAN1 is not sufficient yet, we think those RRM impact can be FFS.

Proposal 1: RAN4 to study the RRM impact of satellite beams activation/deactivation scheme, e.g. cell DTX/DRX like mechanism, after RAN1/2 reaching more concrete conclusions.

Based on the latest revised WID, SSB channel enhancement focusing on SSB periodicity extension will be considered in RAN1, while the final decision will be given by RAN1 at the latest by RAN#106. The concrete method for SSB periodicity extension depends on the subsequent RAN1 discussion. In last RAN1 discussion, the following agreements were reached.

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

We can see that the SSB periodicity extension is still under discussion as one potential enhancement with no final decision. The reception of SSB is the basics of measurement. UE conducting SSB measurement to get timing/frequency/AGC information of the serving cell as well as the target cell. The SSB periodicity extension clearly has RRM impact. However, the RAN4 study is pending on RAN1 finial decision.

Proposal 2: RAN4 to study the RRM impact of SSB periodicity enhancement waiting for further progress in RAN1.

Support of regenerative payload

#### Issue 5-2-1: Timing requirements for regenerative payload <Way Forward> - The common understanding is that NTA,adjcommon is configured by network. - Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

RAN4 reached initial agreement on the timing requirement for regeneration payload as captured above. In this meeting, one LS from RAN2 on Common TA configuration for regenerative payload was received in [2], the detailed context is duplicated below:

# 1. Overall Description: In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0. # 2. Actions: # To RAN1 and RAN4: # RAN2 kindly requests RAN1 and RAN4 to provide feedback on above question.

In our understanding, the component of common TA is to pre-compensate for the delay from the satellite to the RP of uplink synchronization. And in a regenerative payload scenario, the RP of uplink synchronization can always on the satellite. So, it is reasonable to configure common TA = 0, of which we see no technical concern.

Proposal 3: RAN4 to confirm that it is feasible to configure common TA = 0 as the minimum possible value in a regenerative payload scenario.

#### Issue 5-3-2: RRM requirements for RRC Connection Mobility Control with regenerative payload <Way Forward> : Further discuss the following proposal - Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress. - Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress. - FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

Based on the latest agreements, RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed. So, considering the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload, we think there would be no need to further update.

Proposal 4: No need to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload.

- Conclusion

Proposal 1: RAN4 to study the RRM impact of satellite beams activation/deactivation scheme, e.g. cell DTX/DRX like mechanism, after RAN1/2 reaching more concrete conclusions.

Proposal 2: RAN4 to study the RRM impact of SSB periodicity enhancement waiting for further progress in RAN1.

Proposal 3: RAN4 to confirm that it is feasible to configure common TA = 0 as the minimum possible value in a regenerative payload scenario.

Proposal 4: No need to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload.

- Reference

- R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- R4-2414914, LS on common TA in a regenerative payload scenario, RAN2