3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415070

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for beam management

Agenda Item:	6.17.4.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability issues for beam management.

- Channel modelling - Measurement error impact - Performance metric - Ground truth

- Discussion

      - 2.1 Channel modelling

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

For TDL channel, since it can’t reflect the spatial information, we suggest to consider CDL mode.

We understand the intention of discussing the issue is that there is probe limitation in chamber. Number of angles of arrival (AOA) will have impact on probe number in chamber. In current OTA setup, maximum supported probe number is 2 in 38.810 and maximum supported probe number is 6 in 38.827.

For CDL model, there are several clusters and for each cluster, there are 20 rays. In order to reduce the total number of AOA, for each cluster, only one ray is considered.

Proposal 1: For simplified CDL model, for each cluster, only one ray is considered.

Additionally, the number of clusters can be further reduced using the following options:

- Angle Offset Threshold:

  - If the AOA offset between two clusters is smaller than a threshold, these clusters can be merged into one. The AOA offset threshold may depend on the angle resolution of the RX antenna and test setup limitations, which can be further discussed in RAN4.

- Power Threshold:

  - Remove clusters with low power. The power threshold can be further discussed.

- Combination of Angle and Power Thresholds:

  - Apply both angle and power thresholds to reduce the number of clusters.

Here, we would like to apply both angle threshold and power threshold to reduce cluster number. Suppose the power threshold is -15dB. Suppose the AOA offset threshold is set as +-15 degree. Since the current relative angular offset between the directions of 2 active probe is 30° as defined in 38.133.

Table A.3.15.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | 30°, 60°, 90° and 120° |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |
| 5 | FFS |
| 6 | 30°, 60°, 90°, 120° and 150° |
| 7 | FFS |

Take CDL-A as example, by combing in both angle and power threshold, the cluster number can be reduced from 23 to 7. As shown in the following table, cluster # {1,11}, {2,3,4}, {5,6,7}, {8,15}, {9,17} are within similar AOA angle, which can be combined. Cluster #{10,12,14,18-23} can be removed due to low power.

Table 7.7.1-1. CDL-A

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 |  | -13.4 |  | -178.1 |  | 51.3 |  | 50.2 |  | 125.4 |
| 2 | 0.3819 |  | 0 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 3 | 0.4025 |  | -2.2 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 4 | 0.5868 |  | -4 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 5 | 0.4610 |  | -6 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 6 | 0.5375 |  | -8.2 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 7 | 0.6708 |  | -9.9 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 8 | 0.5750 |  | -10.5 |  | 121.5 |  | -1.8 |  | 150.2 |  | 47.1 |
| 9 | 0.7618 |  | -7.5 |  | -81.7 |  | -41.9 |  | 55.2 |  | 56 |
| 10 | 1.5375 |  | -15.9 |  | 158.4 |  | 94.2 |  | 26.4 |  | 30.1 |
| 11 | 1.8978 |  | -6.6 |  | -83 |  | 51.9 |  | 126.4 |  | 58.8 |
| 12 | 2.2242 |  | -16.7 |  | 134.8 |  | -115.9 |  | 171.6 |  | 26 |
| 13 | 2.1718 |  | -12.4 |  | -153 |  | 26.6 |  | 151.4 |  | 49.2 |
| 14 | 2.4942 |  | -15.2 |  | -172 |  | 76.6 |  | 157.2 |  | 143.1 |
| 15 | 2.5119 |  | -10.8 |  | -129.9 |  | -7 |  | 47.2 |  | 117.4 |
| 16 | 3.0582 |  | -11.3 |  | -136 |  | -23 |  | 40.4 |  | 122.7 |
| 17 | 4.0810 |  | -12.7 |  | 165.4 |  | -47.2 |  | 43.3 |  | 123.2 |
| 18 | 4.4579 |  | -16.2 |  | 148.4 |  | 110.4 |  | 161.8 |  | 32.6 |
| 19 | 4.5695 |  | -18.3 |  | 132.7 |  | 144.5 |  | 10.8 |  | 27.2 |
| 20 | 4.7966 |  | -18.9 |  | -118.6 |  | 155.3 |  | 16.7 |  | 15.2 |
| 21 | 5.0066 |  | -16.6 |  | -154.1 |  | 102 |  | 171.7 |  | 146 |
| 22 | 5.3043 |  | -19.9 |  | 126.5 |  | -151.8 |  | 22.7 |  | 150.7 |
| 23 | 9.6586 |  | -29.7 |  | -56.2 |  | 55.2 |  | 144.9 |  | 156.1 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 5 |  | 11 |  | 3 |  | 3 |  | 10 |  |

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

      - 2.2 Measurement error impact

Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

There are two possible impacts due to measurement error:

- Impact on prediction accuracy requirement

  - For example, RSRP prediction accuracy may degrade, an extra X dB margin is needed, where X is decided by evaluation results.

- Impact on performance metric selection

  - As analysed before, If RSRP can be predicted with high accuracy and the RSRP accuracy requirement is defined very tightly, then the beam prediction accuracy requirement may be skipped. However, if measurement error will degrade RSRP prediction accuracy, RSRP accuracy metric alone may not be enough. Then both RSRP and beam prediction metric may be needed.

Proposal 3: Measurement error may have impact on prediction accuracy requirement and performance metric selection.

There have been some discussions in RAN1 about measurement error impact on accuracy degradation. In RAN1, the measurement error is modeled as a Gaussian distribution for RF and BB, with error ranges of 2dB, 4dB, and 6dB. Generally, prediction accuracy degrades as the measurement error range increases. However, the RSRP prediction degradation is calculated based on different assumptions, leading to a lack of aligned comparisons.

For instance:

- Some companies compare prediction degradation based on input data differences while the label is the same, e.g., training with L1-RSRP measurements without error but testing with L1-RSRP measurements with error. Labels for both training and test are ideal L1-RSRP measurement without error.

- Other companies compare prediction degradation based on label differences and the input data are both measurement with error, e.g., during training, L1-RSRP measurements with error is labeled, but during testing, ideal L1-RSRP measurements without error is labeled.

Observation 1: In RAN1, the measurement error is modeled as Gaussian distribution for RF and BB, where the error range is 2dB, 4dB and 6dB.

Observation 2: In RAN1, the RSRP prediction degradation is calculated based on different assumptions and there is no aligned comparison. It’s difficult for RAN4 to directly use the results.

Summary of training and testing assumption

In general, the input data for training can be measurement with or without error. The input data for test will be measurement with error. The label for training and test can be measurement with or without error.

We summarize 6 cases for training and test under different assumptions to better understand if there is any difference. Here, test label refers to ground truth for assessing prediction performance. Since it’s not decided whether SNR for measurement of ground truth is high or medium, we still consider two cases for the ground truth of test: with and without measurement error.

For case 1~3, the label for training and test is aligned. Whether there is performance degradation depends on whether measurement error for input data is aligned.

- For case 1, the input data for training and test are different, i.e. L1-RSRP measurement with and without error respectively.

- For case 2~3, the input data for both training and test are L1-RSRP measurement with error. However, we still note “FFS input data is aligned” in the table.

  - Whether the SNR conditions for the two measurement assumptions are the same

  - Whether error model is the same, e.g. error is modeled as Gaussian distribution in training. It’s FFS what error distribution is in actual measurements for testing.

For case 4~5, the label for training and test is mismatched. Performance degradation is expected.

| Assumptions | Training data for input | Training Label | Test data for input | Test Label (for assessing model performance) | Difference between training and Test |
| --- | --- | --- | --- | --- | --- |
| Case 1 | Ideal L1-RSRP without measurement errors | Ideal L1-RSRP without measurement errors | L1-RSRP with measurement errors | Ideal L1-RSRP without measurement errors | Label is aligned Input data is mismatched |
| Case 2 | L1-RSRP with measurement errors | Ideal L1-RSRP without measurement errors | L1-RSRP with measurement errors | Ideal L1-RSRP without measurement errors | Label is aligned FFS input data is aligned, e.g. SNR, error distribution |
| Case 3 | L1-RSRP with measurement errors | L1-RSRP with measurement errors | L1-RSRP with measurement errors | L1-RSRP with measurement errors | Label is aligned FFS input data is aligned, e.g. SNR, error distribution |
| Case 4 | L1-RSRP with measurement errors | L1-RSRP with measurement errors | L1-RSRP with measurement errors | Ideal L1-RSRP without measurement errors | Label is mismatched FFS input data is aligned, e.g. SNR, error distribution |
| Case 5 | L1-RSRP with measurement errors | L1-RSRP without measurement errors | L1-RSRP with measurement errors | L1-RSRP with measurement errors | Label is mismatched FFS input data is aligned, e.g. SNR, error distribution |
| Case 6 | Ideal L1-RSRP without measurement errors | Ideal L1-RSRP without measurement errors | L1-RSRP with measurement errors | L1-RSRP with measurement errors | Label is mismatched Input data is mismatched |

Observation 3: When label for training and test is not aligned, Performance degradation is expected.

Observation 4: When label for training and test is aligned, whether there will be performance degradation depends on whether measurement error for input data is aligned or not. Measurement error difference may be impacted by SNR and error statistic distribution assumption.

Therefore, if RAN4 would like to consider extra margin for accuracy requirement due to measurement error, it is better to first align on the basic assumption for training and testing first. Then performance evaluation can be conducted to analyse the impact of measurement error if needed.

Proposal 4: RAN4 need to first align the assumptions of input data and labels for training and test first, then further analyze performance degradation due to impact of measurement error if needed.

## 2.3 Performance metric

The report contents for inference are agreed in RAN1 as below:

Agreement RAN1#116bis For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. RAN1#118 For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

Until latest RAN1 agreement, the report content for inference will include:

- Beam information

  - The beam information in the inference report refers to the resource set for Set A

- Predicted RSRP

  - Support differential RSRP report among multiple beams

From our understanding, the beam information is similar to the legacy beam index and is presented in the form of an RS ID configured in a resource set. UE can also report predicted RSRP when configured. Therefore, RAN4 can start to discuss the refinement of RSRP accuracy definition.

Observation 5: According to the latest RAN1 agreement, the inference report content can include beam information and predicted RSRP.

Observation 6: The beam information is similar to the legacy beam index reporting and is presented in the form of an RS ID configured in a resource set.

Proposal 5: RAN4 to start to discuss the refinement of RSRP accuracy definition.

In RAN4#111 WF, it’s suggested to discuss the refinement of RSRP accuracy definition for option 1. Besides, we also see some ambiguity for option 3.

### 2.3.1 Definition refinement for Option 1

For option 1, RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843[1]:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

In last meeting, it’s agreed that ideal L1-RSRP is measured RSRP under high SNR. Therefore, RSRP prediction accuracy focus about the RSRP difference between predicted and measured RSRP for the same beam.

Option 1 is similar as legacy absolute RSRP measurement accuracy definition. In legacy L1-RSRP test metric, the absolute RSRP difference is the delta between the measured L1-RSRP and ideal RSRP for the same beam index. During Rel-15 L1-RSRP accuracy performance discussion, the simulation results is also provided by assuming that RSRP difference is corresponding to the same beam index. No best beam selection functionality is tested.

Besides, Relative L1-RSRP accuracy is also defined for differential L1-RSRP reports in legacy. In RAN1 #117, it agrees about differential L1-RSRP reporting. RAN4 also needs to discuss relative L1-RSRP accuracy requirement.

Proposal 6: For test metric option 1, refine the definition:

- Option 1: Absolute RSRP prediction accuracy is the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and measured L1-RSRP of the same beam. Relative RSRP accuracy is the RSRP difference between the predicted L1-RSRP of different beams.

### 2.3.2 Definition refinement for Option 3

For option 3, the current wording is a little ambiguous. it’s not clear what does “maximum RSRP among top-K predicted beams” refers to.

- Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

There are two interpretations for “maximum RSRP among top-K predicted beams” for option 3:

- Interpretation 1: RSRP difference between Measured RSRP of best predicted beam and Measured RSRP of the strongest beam

- Interpretation 2: RSRP difference between Predicted RSRP of best predicted beam and Measured RSRP of the strongest beam

Different interpretation of RSRP may lead to different test purpose.

For Interpretation 1, option 3 will only test beam prediction accuracy in terms of RSRP difference. It will not check RSRP prediction performance and it only check whether beam selection is correct. measured RSRP for best predicted beam and best measured beam will be used for comparison. If that is the motivation, then the upper bound is not necessary. However, the current wording needs to be further refined.

For Interpretation 2, option 3 will try to verify both beam prediction and RSRP prediction together. However, it currently defines only the lower bound of the difference between predicted RSRP and measured RSRP, without addressing the upper bound. Therefore, we recommend modifying Option 3 to account for both the upper and lower bounds as detailed in [2].

From our understanding, option 3 is more similar as the test metric for beam prediction defined in 38.843 as below:

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam"

In above definition, measured L1-RSRP for both best predicted beam and best measured beam is compared. Then the main motivation is just to verify beam prediction accuracy. Therefore, we suggest to refine the wording of option 3 to indicate that the motivation is for beam prediction accuracy.

Proposal 7: For test metric option 3, refine the definition:

- Option 3: successful rate for the correct beam prediction which is considered as measured L1-RSRP of top-1 predicted beams is larger than the measured L1-RSRP of the Top-1 measured beam – x dB.

### 2.3.3 Test metric selection

In legacy non-AI Beam management, both beam index and RSRP can be included in L1 report, however, RAN4 only defines requirement for RSRP accuracy.

Observation 7: In legacy non-AI BM, both beam index and RSRP can be included in L1 report, however, RAN4 only defines requirement for RSRP accuracy.

For AI based beam management, both predicted beam index and predicted RSRP can be included in L1 report, similar as legacy. RAN4 needs to discuss how to select test metric between these options.

For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam.

While both option 2 and 3 can be classified into beam prediction accuracy.

In RAN1, the beam prediction accuracy can be defined from two different aspects:

- Type 1: Based on RSRP difference between predicted Top-1 beam and Genie Top-1 beam

- Type 2: Based on beam index difference between predicted Top-1/K beam and Genie Top-1/K beam

Therefore, both option 2 and 3 can be classified into beam prediction accuracy. Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Whether beam index accuracy requirement can be skipped depends on how accurate absolute RSRP prediction can achieve. Considering an ideal 2dB RSRP difference between the top two beams, if RSRP prediction is sufficiently accurate (e.g., ±1dB), it aids UE in identifying the optimal beam. Thus, RSRP prediction can assess both beam and RSRP performance. Even if UE can’t identify the best beam, it can also help to identify sub-optimal beam which is quite close to optimal beam. Conversely, lower accuracy (e.g., 5dB) may not suffice for beam prediction verification.

If RSRP can be predicted with high accuracy and the RSRP accuracy requirement (option 1) is defined very tightly, then the beam prediction accuracy requirement may be skipped.

Observation 8: For AI, whether beam accuracy metric can be skipped depends on how accurate absolute RSRP prediction can achieve.

      - 2.4 Ground truth

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

In last meeting, it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.1 test procedure

As shown in Fig.1, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.4.1 RX beam consistency impact

The L1-RSRP for a given TX beam can vary depending on the selected RX beam. Consequently, the optimal TX beam index is determined based on the L1-RSRP values. The assumption made about the RX beam affects both the value of the RSRP measurement and the precision of the beam selection metric.

Step 1- Step 3

For step 1, TE configures resource sets for measurement of set B and prediction report for set A.

For step 2 and 3, UE will measurement L1-RSRP of set B and predict L1-RSRP and/or best TX beam index of set A. L1-RSRSP and/or best TX beam index will be derived based on certain RX beam assumption. Since in TR, two kinds of RX assumptions are supported:

Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

For Option A, the output of AI model can be predicted L1-RSRP with full RX beam sweeping. For option B, the output of AI model can be predicted L1-RSRP with specific RX beams. Similarly, the input of AI model can also be measured L1-RSRP based on specific RX beams or full RX beam sweeping.

Observation 9: From TR, predicted best TX beam index and/or L1-RSRP can be based on full RX beam sweeping or specific RX beam.

Therefore, there are 2 kinds of possible UE implementation for RX beam assumption for the input and output of AI model, shown as below:

Fig.2 different RX beam assumption for prediction

For Case 1, when L1-RSRP measurements of Set B based on a specific RX beam are used as input, the output for predicted L1-RSRP of Set A is also based on the same specific RX beam. For case 2, when L1-RSRP measurements of Set B based on full RX beam sweeping are used as input, the output for predicted L1-RSRP of Set A can also be based on full RX beam sweeping. During full RX beam sweeping, the L1-RSRP for a given TX beam is determined based on the best RX beams. The best TX beam index will then be chosen based on the value of L1-RSRP of all TX beams.

As a result, the predicted L1-RSRP for a TX beam will vary depending on whether specific RX beams or full RX beam sweeping is assumed. For best TX beam index, it may also vary depending on whether specific RX beams or full RX beam sweeping is assumed.

Observation 10: Whether specific RX beams or full RX beam sweeping is assumed may lead to different best predicted TX beam index and predicted L1-RSRP value.

Step 5

For T2 in step 5, UE will measure set A to get L1-RSRP ground truth. For this step, it’s just legacy non-AI based L1-RSRP measurement. If the L1-RSRP configuration and reporting can re-use legacy rule, UE will measure for beams in set A based on RX beam sweeping if SSB or CSI with repetition ON is configured.

In legacy non-AI L1-RSRP measurement, there are 8 RX beams in RX beam sweeping set and the selection of these 8 beams depends on the UE's implementation. For instance, the UE might divide the full spatial scope into 8 equal parts, as shown in Fig.3(a) or narrow down the spatial scope based on L3 measurements, as illustrated in Fig. 3(a). In fact, even RX beams occupy full spatial scope, there may still be angle offset between two RX beam sets. For example, the angle of 8 RX beams in RX beam set 1 may be {0, 45, 90, 135, 180, 225, 270, 315} degree. The angle of 8 RX beams in RX beam set 2 may be {15, 60, 105, 150, 195, 240, 285, 330} degree.

- RX beams occupy full spatial scope         (b)  RX beams occupy partial spatial scope

Fig.3 RX beam spatial scope

Observation 11: In legacy non-AI based L1-RSRP measurement, when RX beam sweeping is applied, it’s up to UE implementation to choose RX beam angle.

RX beam mismatch for prediction and measurement for set A

In summary, without RX beam information indication for measurement of set A in step 5, it’s possible that UE choose different RX beams for measurement and prediction. There may be mismatch for predicted L1-RSRP/best beam index and measured L1-RSRP/best beam index for set A:

- Case 1: For prediction, specific RX beam is assumed while for measurement, RX beam sweeping is assumed.

- Case 2: For both prediction and measurement, RX sweeping beam is assumed. However, there may still be angle offset between two RX beam sweeping sets.

RAN4 needs to discuss how to keep the RX beam Consistency between prediction and measurement for set A to guarantee correct comparation.

Observation 12: If different RX beam is assumed for prediction and measurement, the L1-RSRP and/or best TX beam index may be different in two steps, which will lead to wrong performance evaluation.

There are several possible solutions. For example, NW/TE can also configure UE to report RX beam index for beam prediction of set A in step 4. Then NW/TE configures RX beam index for the measurement of set A in step 5. TE can also configure a RX beam association relationship between beam report and beam measurement for set A, e.g. indicate report ID. or NW/TE just send one configuration command including both step 1 and step 5, then UE will apply the same RX assumption for different steps.

Proposal 8: RAN4 to discuss how to keep the RX beam Consistency between prediction and measurement for set A to correctly evaluate prediction performance.

### 2.4.2 SNR impact

The ground truth for L1-RSRP in testing is defined by measuring L1-RSRP with a specified SNR threshold. To reduce measurement errors, a high SNR is essential. A higher SNR helps minimize errors in the measurement process. RAN4 should discuss how to select an appropriate SNR to ensure measurement quality. This SNR level can be determined through simulation. We recommend selecting the appropriate SNR level based on the L1-RSRP delta CDF curve, similar to the method used in Rel-15 to establish the accuracy requirements for measured L1-RSRP.

RSRP delta is defined as below:

RSRP delta= measured RSRP in Time duration 2 – ideal RSRP in Time duration 2

The procedure to determine the SNR level is outlined below:

- Set the RSRP delta threshold to XdB (e.g., 0.5 dB).

- Calculate and plot the RSRP delta CDF for different SNR levels and configurations. Configurations may include:

  - Channel type

  - Doppler

  - Number of reference signals (RS) in the time domain

  - Density of RS in the frequency domain

- Select the SNR level so that the maximum RSRP delta values (corresponding to the 5% and 95% CDF points) are below the XdB threshold.

Proposal 9: Suggest to use RSRP delta CDF-based simulation to determine the appropriate SNR level for deriving ground truth.

### 2.4.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 10: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 11: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.4.4 Multiple-AoA test setup impact

For multiple-AOA test setup, as pointed early by TE vendor, multiple directions might be addressed by the DUT with different antennas, so that RF baseline system assumptions are not valid and FF-QZ-MU configuration needs further investigation. it’s more challenging to obtain accurate RSRP measurement as ground truth. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

Proposal 12: It’s more challenging to obtain accurate RSRP measurement in multiple-AOA test setup. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

### 2.4.5 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 13: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.4(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index. In this way, UE can only honestly report the best measured beam in T2.

Fig.4 (a) beam index and RS index mapping relation for set A in T1

Fig.4 (b) beam index and RS index mapping relation for set A for T2

Proposal 13: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For simplified CDL model, for each cluster, only one ray is considered.

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

Proposal 3: Measurement error may have impact on prediction accuracy requirement and performance metric selection.

Observation 1: In RAN1, the measurement error is modeled as Gaussian distribution for RF and BB, where the error range is 2dB, 4dB and 6dB.

Observation 2: In RAN1, the RSRP prediction degradation is calculated based on different assumptions and there is no aligned comparison. It’s difficult for RAN4 to directly use the results.

Observation 3: When label for training and test is not aligned, Performance degradation is expected.

Observation 4: When label for training and test is aligned, whether there will be performance degradation depends on whether measurement error for input data is aligned or not. Measurement error difference may be impacted by SNR and error statistic distribution assumption.

Proposal 4: RAN4 need to first align the assumptions of input data and labels for training and test first, then further analyze performance degradation due to impact of measurement error if needed.

Observation 5: According to the latest RAN1 agreement, the inference report content can include beam information and predicted RSRP.

Observation 6: The beam information is similar to the legacy beam index reporting and is presented in the form of an RS ID configured in a resource set.

Proposal 5: RAN4 to start to discuss the refinement of RSRP accuracy definition.

Proposal 6: For test metric option 1, refine the definition:

- Option 1: Absolute RSRP prediction accuracy is the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and measured L1-RSRP of the same beam. Relative RSRP accuracy is the RSRP difference between the predicted L1-RSRP of different beams.

Proposal 7: For test metric option 3, refine the definition:

- Option 3: successful rate for the correct beam prediction which is considered as measured L1-RSRP of top-1 predicted beams is larger than the measured L1-RSRP of the Top-1 measured beam – x dB.

Observation 7: In legacy non-AI BM, both beam index and RSRP can be included in L1 report, however, RAN4 only defines requirement for RSRP accuracy and RAN4 doesn’t define requirement for beam accuracy.

Observation 8: For AI, Whether beam accuracy metric can be skipped depends on how accurate absolute RSRP prediction can achieve.

Observation 9: From TR, predicted best TX beam index and/or L1-RSRP can be based on full RX beam sweeping or specific RX beam.

Observation 10: Whether specific RX beams or full RX beam sweeping is assumed may lead to different best predicted TX beam index and predicted L1-RSRP value.

Observation 11: In legacy non-AI based L1-RSRP measurement, when RX beam sweeping is applied, it’s up to UE implementation to choose RX beam angle.

Observation 12: If different RX beam is assumed for prediction and measurement, the L1-RSRP and/or best TX beam index may be different in two steps, which will lead to wrong performance evaluation.

Proposal 8: RAN4 to discuss how to keep the RX beam Consistency between prediction and measurement for set A to correctly evaluate prediction performance.

Proposal 9: Suggest to use RSRP delta CDF-based simulation to determine the appropriate SNR level for deriving ground truth.

Proposal 10: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 11: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Proposal 12: It’s more challenging to obtain accurate RSRP measurement in multiple-AOA test setup. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

Observation 13: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

Proposal 13: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2]  R4-2404573, Discussion on testability and interoperability issues for beam management, Xiaomi