3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415071

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on core requirement for AI beam management

Agenda Item:	6.17.4.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will core part requirement for AI based beam management.

- Impact on L1-RSRP measurement - Impact on TCI state activation - Performance monitoring

- Discussion

      - 2.1 Impact on L1-RSRP measurement

RX beam sweeping factor

In legacy for FR2, RX beam sweeping is considered for SSB based measurement and CSI-RS with repletion ON.

Measurement period will be scaled by RX beam sweeping factor, e.g. N=8 for SSB.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

For AI beam management, two kinds of RX beam are assumed in TR 38.843:

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams.

For option B, the best beam can be predicted by specific RX beam(s). It means that UE can use subset RX beam to predict best TX beam and UE don’t need to sweep full RX beams. Therefore, it may have impact on RX beam sweeping factor in measurement period. Since RX beam sweeping will apply for spatial domain and time domain beam prediction, the impact to BM case-1 and BM case-2 is common.

Proposal 1: For both BM case-1 and BM case-2, RAN4 to discuss whether there is impact on measurement period due to specific RX beam assumption.

BM-case 2

For BM-case 2, RAN1 agrees that NW can configure UE to predict future N L1-RSRP instance.

RAN1 #118 Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted.

Besides, TR defined several types for time-domain prediction:

Case A: based on number of measurements/RSs and prediction time. An example is shown in Figure 6.3.1-2. - where T2 is the time duration for beam prediction - where Mt is the number of time instances for measurement as AI/ML inputs with a periodicity of Tper - where Pt is the number of time instance(s) for prediction with a periodicity of Tper in T2 Case B: based on a periodicity T of the required reference signals for measurements to achieve a certain beam prediction accuracy. An example is shown in Figure 6.3.1-3. - For AI, every T=Y ms, reference signals for measurements are needed - In this case, - For Set B = Set A, the RS overhead reduction is 1-X/Y. - For Set B (N beams) is a subset of Set A (M beams), the RS overhead reduction is [1-XN/(YM)].

The figures for case A and case B from TR are shown below:

Figure 6.3.1-2: Example for Case A

Figure 6.3.1-3: Example for Case B

The main difference of the two cases is the length of measurement periodicity and prediction periodicity.

  - Case A: the periodicity for history measurement instance is same as that of predicted future time instance.

  - Case B: the periodicity for history measurement instance is larger than that of predicted future time instance.

From our observation, at least case A will have impact on UE measurement requirement. In legacy, measurements on all instance are needed. Measurement period is defined for each instance periodically. While when AI is applied, for case A, some measurement instance can be skipped and UE can acquire predicted results for these time slots. The legacy measurement period requirement can’t reflect such benefits introduced by AI. Suppose that there are totally 10 instances. In legacy, the total measurement time is 10*TRS. While when AI is applied, suppose that AI can measure for 5 instance and predict for another 5 instances, the total measurement time will be reduced.

Since RAN1 introduce the concept of measurement time and prediction time, RAN4 needs to discuss how to define new requirement for measurement time and prediction time for BM-case 2 either. If measurement time and prediction time is defined, RAN4 can further discuss whether measurement restriction and scheduling restriction can be relaxed in prediction time.

Proposal 2: For BM-case 2, RAN4 to discuss impact on measurement requirement, e.g. measurement time, prediction time, measurement restriction and scheduling restriction, etc.

      - 2.2 TCI state activation

UE will measure for set B first and then predict and report best TX beam index in set A, then NW can configure TCI state activation. There are two possibilities for the reported best predicted TX beam index:

- Case 1: best predicted TX beam is in set B.

- Case 2: best predicted TX beam is in set A but not in set B.

For case 1, UE knows the corresponding best RX beam since UE performs RX beam sweeping for all TX beams in set B. For case 2, UE may know or not know the best RX beam. It will depend on UE implementation. If the input of AI model and output of AI model are based on specific RX beam, UE can know the corresponding RX beam. If the input of AI model and output of AI model are based on RX beam sweeping, UE can first predict the L1-RSRP based on different TX-RX pairs, then UE sort L1-RSRP and report the best beam index. Then it’s possible for UE to know the corresponding best RX beam. If UE directly predict best TX beam index, it’s possible that UE didn’t know the RX beam index.

Observation 1: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Whether UE knows the RX beam or not will have impact on TCI state activation delay. Current known condition only considers whether UE has sent the report within a period and it didn’t consider whether UE has known the RX beam or not. We prefer to define UE capability to indicate whether UE can know the RX beam or not. Then the L1 reporting part in known condition for TCI state will be updated to:

The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command. Where, - the target TCI state in L1-RSRP report has been measured, or - the target TCI state in L1-RSRP report is predicted and UE support [RX beam known] capability

Proposal 3: RAN4 to discuss known TCI state condition for AI based beam indication.

      - 2.3 Performance monitoring

RAN1#118 Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) RAN1#117 Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For UE-assisted performance monitoring, RAN1 will study how to design the evaluation time, evaluation criteria and L1 reporting. For example, RAN1 will study whether evaluation window is needed for evaluation period and whether to support event triggered reporting for L1 reporting.

Observation 2: For UE-assisted performance monitoring, RAN1 will study how to design evaluation time, evaluation criteria and L1 reporting.

UE needs to monitor the predicted beam quality by measurement and send report when criteria is satisfied, which is similar as radio link monitoring. The following requirement for performance monitoring may be needed:

- Evaluation period

- L1 reporting

- Measurement restriction

- Scheduling availability

Proposal 4: RAN4 to define requirement for UE assistant performance monitoring. The detail needs more input from RAN1.

For other LCM related requirement, we suggest to wait for more conclusions from other WGs.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For both BM case-1 and BM case-2, RAN4 to discuss whether there is impact on measurement period due to specific RX beam assumption.

Proposal 2: For BM-case 2, RAN4 to discuss impact on measurement requirement, e.g. measurement time, prediction time, measurement restriction and scheduling restriction, etc.

Observation 1: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Proposal 3: RAN4 to discuss known TCI state condition for AI based beam indication.

Observation 2: For UE-assisted performance monitoring, RAN1 will study how to design evaluation time, evaluation criteria and L1 reporting.

Proposal 4: RAN4 to define requirement for UE assistant performance monitoring. The detail needs more input from RAN1.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface