3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415072

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for CSI compression

Agenda Item:	6.17.2.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability and interoperability issues for CSI compression.

- Discussion

      - 2.1 Simulation assumption and results

Our simulation assumptions are as below:

| Parameter/Configurations | Values |
| --- | --- |
| Channel/data-set related |  |
| UE antenna element assumptions | Ideal omni-directional antenna elements following RAN1 assumptions |
| Sub-band channel averaging | Average of covariance channel matrices per PRBs in the sub-band 1 sample per PRB is selected |
| Any other pre-processing | Apply phase normalization |
| Training dataset size | 600K |
| Model-related |  |
| Normalization before quantization in the encoder | For the results with quantization, to use a sigmoid function in the encoder for normalization |
| Use and configuration of drop-out | Not to use drop-out layers |
| Optimizer | Adam |
| Learning rate | 0.0005 |
| Batch size | 128 |
| Number of epochs | 200 |
| Weight initialization | default |
| Kernel Parameters: - Dilation - Padding Type The value of groups (VG) | default |

The simulation results are as below:

| case | SGCS Layer1 | SGCS of Rel-16 eType II | Improvement of CNN model over Rel-16 eType II (C-D)/D * 100 |
| --- | --- | --- | --- |
| Enc(ResNet)-Dec(ResNet) | 0.682 | 0.676 | 0.88 |

For each user, one slot sample is assumed. Here we plot the CDF curve of SGCS across different users.

      - 2.2 Standardization process for option 3

In last meeting, companies discussed standardization process for option 3[1].

Issue 4-1: Option 3 next steps Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed Select one or more decoder for further analysis - selection criteria to be discussed Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Several steps are involved and needs further discussion.

Simulation alignment

To determine what constitutes good alignment, RAN4 can consider the following options:

- Deviation-Based Alignment: Calculate the average SGCS value and determine the deviation of SGCS values from different companies. If the deviation is within a specified threshold, the simulation results are considered aligned. RAN4 needs to discuss the maximum allowable deviation for SGCS.

- Threshold-Based Alignment: If the minimum SGCS values provided by most companies exceed an accuracy threshold, the simulation results are considered aligned.

The simulation results are aligned if:

- Option 1: The deviation of SGCS from the averaged SGCS is within a threshold.

- Option 2: The SGCS values from most companies are larger than a threshold.

Proposal 1: RAN4 should discuss and establish the criteria for simulation alignment.

Handling Misalignment

If the simulation results using company-specific data cannot be aligned, RAN4 can consider setting up a common data set by mixing data from different companies. Companies can then train their encoder/decoder models based on this common data set to further align the results.

Proposal 2: If alignment cannot be achieved using company-specific data, RAN4 should discuss whether and how to combine data from different companies to create a common data set for training models.

Choosing a Reference Decoder

If the simulation results can be aligned, the next step is to select a reference decoder. It’s hard to choose the reference decoder for the first round. Therefore, we suggest that multiple decoders can be chosen as candidate first and then the reference decoder can be selected from these candidate decoders with some conditions. We propose the steps as below:

- Candidate Decoders: Multiple reference decoders can be set as candidates. Ideally, all interested companies can provide their decoders.

- Training and Evaluation: Each company trains its own encoders to fit multiple candidate decoders and calculates the prediction accuracy for each pair.

- Selection: Companies select decoders with SGCS values larger than a threshold and provide these decoders to RAN4.

- Final Choice: RAN4 chooses the decoder selected by most companies, indicating better generalization and compatibility with more encoders to achieve good performance.

Proposal 3: Multiple decoders can be chosen as candidates first and then the final decoder can be selected from these candidates with good generalization.

      - 2.3 File/data formats

For data formats, we are fine with Numpy or Matlab. In addition to choosing the data format, it is crucial to align the datasets between companies. This includes standardizing the raw data generation methods, such as how to generate raw channel H. Furthermore, if pre-processing is involved, the procedures for obtaining eigenvectors must also be clearly defined.

For the sample number, it will be equal to:

- Number of drops* Number of UE* Number of slot

For each sample, it will include the following dimension:

- Number of TX port

- Number of subband

- Number of layer

- Number of IQ

Then the total data format will be:

- [Number of sample, Number of TX port, Number of subband, Number of layer, Number of IQ]

We are open to further discuss model format. For ONNX, it’s a solution when company use different model types. There are some possible issues due to ONNX which may need further study. For example, there may be compatibility issues. ONNX may not support all operations from PyTorch or TensorFlow, especially custom or recent operations. Need to ensure compatible versions of ONNX, PyTorch, and TensorFlow to avoid issues. Besides,
When converted to ONNX, advanced features or specific layer implementations may not be translated perfectly, affecting performance or behavior. Testing may be needed to ensure the converted model's behavior matches the original on different platforms.

Proposal 4: For data formats, Numpy or Matlab is fine.

Proposal 5: For model format, RAN4 to further discuss the feasibility of ONNX.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: RAN4 should discuss and establish the criteria for simulation alignment.

Proposal 2: If alignment cannot be achieved using company-specific data, RAN4 should discuss whether and how to combine data from different companies to create a common data set for training models.

Proposal 3: Multiple decoders can be chosen as candidates first and then the final decoder can be selected from these candidates with good generalization.

Proposal 4: For data formats, Numpy or Matlab is fine.

Proposal 5: For model format, RAN4 to further discuss the feasibility of ONNX.

- Reference

- R4-2414323, WF on NR ALML air interface, Qualcomm