3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415075

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on UE-initiated/event-driven beam management on Rel-19 MIMO phase 5

Agenda Item:	6.19.3.1

Document for:	Discussion

# 1 Introduction

In last meeting, it’s agreed to define L1 event trigged measurement reporting delay.

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement - Further discuss: - Whether to support both mode A and mode B - Whether to differentiate mode A and mode B - Other related aspects

In this contribution, we will mainly discuss the RRM requirement of UE-initiated/event-driven beam management for Rel-19 MIMO phase 5.

- Discussion

2.1 Legacy event triggered L3 reporting

L3 event triggered reporting may be used as starting point. The starting point and ending point for report delay is defined as below:

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signaling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

In L3 report, the starting point of report is an event that will trigger a L3 report. Here, the event is that channel condition of serving cell or neighbor cell has changed. The ending point of legacy L3 event triggered reporting is that UE starts to transmit the measurement report over the air interface. From our understanding, the ending point can also be interpreted as the occasion of first available UL resource to carry UL L3 report.

For reporting delay, the maximum delay is defined as one measurement period for known cell to guarantee that UE can obtain at least one RRM result to compare with criteria and trigger reporting. Besides, no UL resource waiting time and L3 related processing is considered in reporting delay. For example, TTT(time to trigger) and L3 filtering is not considered.

Observation 1: For legacy event triggered L3 reporting, the starting point and ending point is defined as:

- the starting point of reporting is the time when channel condition of cell is changed and UE needs to evaluate for L3 report.

- The ending point of reporting is the time when UE starts to send L3 report

Observation 2: For legacy event triggered L3 reporting, the maximum delay is defined as one measurement period for neighbour cell to guarantee that UE can obtain at least one L3 result to compare with criteria.

Observation 3: For legacy event triggered L3 reporting, No L3 related processing and no UL resource waiting time are included in reporting delay.

2.2 Starting and ending point for event-triggered L1 reporting

For L1 event-driven/UE-initiated reporting, similar as L3, the main motivation is to verify that when beam quality changes, UE can detect the variation timely and report L1-RSRP to NW within a required time based on different event triggering criteria. Then the definition for starting point and ending point for L3 event triggered reporting can be re-used for L1.

As shown in Fig.1, the total timeline mainly includes two parts:

- Needed measurement time for multiple beams

- UL resource waiting time

Fig.1 Time line for event triggered L1 reporting

Proposal 1: For L1 event-driven/UE-initiated reporting, the starting point is an event that beam quality of current beam or other beams has changed. From that point on, the UE needs to evaluate for L1 reporting through measurement.

Proposal 2: For L1 event-driven/UE-initiated reporting, the ending point is the time when UE starts to send L1 report.

2.3 Needed measurement time

Until now, the supported triggering event agreed in RAN1 include:

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signaling - UE may only indicate a single candidate value or not support Event-7.

Take event 2 as example, suppose that current beam is from serving cell and new beam is from cell with different PCI(CDP). Measurement periods for two beams are different, e.g. TSSB_serving and TSSB_CDP. As shown in Fig.1, if follow the legacy L3 requirement, total measurement time is defined based one measurement period of TSSB_CDP.  However, since TSSB_CDP < TSSB_serving, when UE obtain L1-RSRP of new beam, UE doesn’t obtain L1-RSRP from current beam. UE can’t compare with criteria and trigger L1 report.

Fig.2 Different measurement period for different beams in inter-cell mTRP

While for event 1, UE only to measure current beams. Measurement time only needs to consider one beam.

For event 7, UE needs to measure for beams with activated TCI states. Total measurement time also needs to consider the measurement period of multiple beams.

Proposal 3: If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time.

2.4 Impact of Triggering window

From RAN1, for event-2, a window including M instance(s) for new beam will be defined for triggering L1 reporting. Here, one instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam.

[117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In legacy L3 reporting, only one measurement period is defined in reporting delay to get rid of impact from high levels. While for event triggered L1 reporting, the impact of triggering window in physical layer needs to be considered.

Fig.2 Triggering window

Proposal 4: For measurement time, RAN4 needs to consider the impact of triggering window defined in RAN1.

2.5 UL resource waiting time

RAN1 defined two modes to configure UL resource to carry L1 report. For both mode-A and mode-B, UE needs to transmit PUCCH first. Then for mode-A, UE can acquire first dynamic UL resource after DCI. For Mode-B, UE can acquire first UL by pre-configuration. The two modes are plotted as below:

Fig.3 Two modes for UL resource acquiring

In legacy L3 reporting, UL resource waiting time is not included in total L3 reporting delay length as the main intension is to verify the UE capability to report the variation of channel quality within required time duration. While for UL resource waiting time, the delay is dependent on NW configuration. Besides, since L3 measurement period is long, in most cases, UL resource waiting time will be shorter than measurement period. UE will not acquire new L3 results before first UL resource is available. L3-RSRP result which triggered the reporting will be included in the L3 report. There is no impact if UL resource waiting time is not included.

Fig.4 L3 measurement time is larger than UL resource waiting time

However, there will some difference for L1 event triggered reporting.

Different from L3 measurement, L1 measurement is much fast. Depends on different configuration for L1 measurement period and UL resource availability, it’s possible that L1 measurement period is shorter than UL resource waiting time. Then UE will acquire several new L1-RSRP result before first UL resource is available, as shown in Fig.4.

Fig.4 L1 Measurement period is shorter than UL resource waiting time

For instance, if the UE has obtained several new L1 results after L1 reporting has been triggered,

  - Which L1 result will be included in the L1 report if the event trigger criteria still apply

  - What happens if the new L1 results change and no longer satisfy the event trigger criteria

Therefore, RAN4 needs to consider to impact of UL resource waiting time for L1 event triggered reporting. We don’t mean to specify the detail length of UL resource waiting time. But we need to consider the impact of UL waiting time on L1 measurement and L1 reporting.

Proposal 5: RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting.

Proposal 6: RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time.

2.6 L1-RSRP Accuracy requirement

Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward> - Option 1: No new accuracy requirements for existing L1 measurements are required. - Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

For legacy L3 event triggered reporting, there is no new requirement for measurement accuracy. Similar, for L1 event triggered reporting, there is also no new requirement for measurement accuracy.

Proposal 7: No new accuracy requirements for existing L1 measurements are required.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: For legacy event triggered L3 reporting, the starting point and ending point is defined as:

- the starting point of reporting is the time when channel condition of cell is changed and UE needs to evaluate for L3 report.

- The ending point of reporting is the time when UE starts to send L3 report

Observation 2: For legacy event triggered L3 reporting, the maximum delay is defined as one measurement period for neighbour cell to guarantee that UE can obtain at least one L3 result to compare with criteria.

Observation 3: For legacy event triggered L3 reporting, No L3 related processing and no UL resource waiting time are included in reporting delay.

Proposal 1: For L1 event-driven/UE-initiated reporting, the starting point is an event that beam quality of current beam or other beams has changed. From that point on, the UE needs to evaluate for L1 reporting through measurement.

Proposal 2: For L1 event-driven/UE-initiated reporting, the ending point is the time when UE starts to send L1 report.

Proposal 3: If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time.

Proposal 4: For measurement time, RAN4 needs to consider the impact of triggering window defined in RAN1.

Proposal 5: RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting.

Proposal 6: RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time.

Proposal 7: No new accuracy requirements for existing L1 measurements are required.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung