3GPP TSG-RAN WG4 Meeting #112bis                                                                                                      R4-2415078

Hefei, Anhui, China, 14th –18th October, 2024

Source: 	CATT

Title: 	Discussion on spatial channel modelling methodology for demodulation performance requirements

Agenda item:	6.14.2

Document for:	Approval

- Introduction

In RAN4#112 meeting, a WF[1] for [112][327] FS_NR_demod_SCM was agreed and remaining issues on spatial channel modelling methodology needs further discussion.

In this contribution, we provide our views on spatial channel modelling methodology for demodulation.

- Discussion

2.1 TDL Based Methodologies

Tap Delay Line Parameters

As per the WF [1], the issue concerning tap delay line parameters is shown as below:

Tap Delay Line Parameters Agreement: The following Legacy TDL models for comparison: - Option 1: TDLC 300-100 ULA Low - Option 2: TDLA 30-10 ULA Low - FFS: TDLC 300-100 medA - FFS: TDLC 300-100 medB - FFS: TDLB 100

The PDSCH Rank 8 demodulation in TS 38.101-4 is copied as below:

Table 5.2.4.1.1-7: Minimum performance for Rank 8

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.1-3.8 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.5 |

Referring to above PDSCH Rank 8 demodulation in TS 38.101-4, the TDLA30-10 8x8 ULA Low is for 64QAM. So we prefer to use Option 2 for TDL for comparison.

Observation 1: Use Option 2 TDLA 30-10 ULA Low for comparison.

TDL Model Extensions

As per the WF [1], the issue concerning TDL model extensions is shown as below:

TDL Model Extensions Way Forward: Open issue needs further discussion, 2 contributions provided during RAN4 #112, interested companies to review and provide views at next meeting.

Referring to sub-clause 7.7.4 of TR 38.901, the TDL models can be generated from CDL models by spatial filtering with assuming antenna pattern at both Tx and Rx. The assuming antenna pattern can be ideal isotropic antenna, non-isotropic antennas or array antennas. In sub-clause 7.7.4.1 of TR 38.901, the isotropic antenna pattern, rectangular mask pattern, and simplified antenna pattern give in ITU-R M.2135 are defined. In sub-clause 7.7.4.2 of TR 38.901, the generation procedure for TDL generated from CDL are defined, From equation 7.7-4 of TR 38.901 as following, the TDL cluster powerfor cluster n can be calculated with CDL cluster power for cluster n, assuming rx antenna pattern ,and assuming tx antenna pattern .

From our understanding, the assuming rx antenna pattern ,and assuming tx antenna pattern are single antenna element pattern.

The TDL models described in Clause 7.7.2 of TR 38.901 are generated from the CDL models assuming ideal isotropic antennas at both Tx and Rx. From Table 7.3-1 of TR 38.901, the element response pattern is normalized 3D radiation power pattern, which is calculated from vertical cut of the radiation power pattern and horizontal cut of the radiation power pattern.

Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) |  |
| Horizontal cut of the radiation power pattern (dB) |  |
| 3D radiation power pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

So, for TDL model extensions, it is reasonable to define the radiation power pattern of a single antenna element for spatial filtering.

Observation 2: Define the radiation power pattern of a single antenna element for spatial filtering for TDL model extensions.

For TDL model extensions for MIMO simulations, the correlation matrix is needed, and correlation matrix is dependent on array pattern. The sub-clause 7.7.5.2 of TR 38.901 define how to apply a correlation matrix to TDL model extensions as following:

The TDLs and the spatial-filtered TDLs can be used with the correlation matrices for MIMO link-level simulations. Typical correlation parameters can be derived from 1) delay & angular scaled CDLs with antenna array assumptions, 2) system-level model with antenna array assumptions, or 3) by selecting extreme cases, e.g. uncorrelated, highly correlated etc. For example, these following options can be considered: 1) Zero correlation (IID channel coefficients) can be used for any number of antenna elements 2) The correlation matrix construction method from TS36.101/104 [15][16] can be used for linear and planar (single- or dual-polarized) arrays. - Other correlation parameters α, β, γ than those specified in TS36.101/104 [15][16] and extensions to larger antenna arrays can be considered. For typical scenarios, α and β will be in the range 0-1 - A representative set of values is {0,0.7,0.9,0.99} Note: This approach can be applied to TDLs derived from spatially filtered CDLs to emulate hybrid BF system Note: Other methodologies could also be developed, e.g. - extending the TS36.101/104 [15][16] procedure to planar arrays or more elements - using CDLs in combination with array assumptions to derive per-tap correlation matrices as in [17].

So, it is reasonable to use method in sub-clause 7.7.5.2 of TR 38.901 to generate correlation matrix for TDL model extensions.

Observation 3: Use method in sub-clause 7.7.5.2 of TR 38.901 to generate correlation matrix for TDL model extensions.

- Conclusion

This contribution provides analysis on spatial channel modelling methodology for demodulation performance requirements, and the following observations are concluded as follows:

Observation 1: Use Option 2 TDLA 30-10 ULA Low for comparison.

Observation 2: Define the radiation power pattern of a single antenna element for spatial filtering for TDL model extensions.

Observation 3: Use method in sub-clause 7.7.5.2 of TR 38.901 to generate correlation matrix for TDL model extensions.

- References

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Moderator (Nokia), RAN4#112

- RP-241610, Study on spatial channel model for demodulation performance requirements, Nokia, BT Plc, etc., RAN#104