3GPP TSG-RAN WG4 Meeting #112bis                                                                                                      R4-2415079

Hefei, Anhui, China, 14th –18th October, 2024

Source: 	CATT

Title: 	Discussion on coexistence evaluation for Ku-band for NR NTN

Agenda item:	6.9.2

Document for:	Approval

- Introduction

In RAN4#112 meeting, a WF[1] on NTN Ku band Coexistence study was agreed and remaining issues on Ku-band coexistence assumptions in WF require further discussion.

In this contribution, we provide our views on coexistence evaluation for Ku-band for NR NTN.

- Discussion

The Ka-band satellite antenna parameters for 20GHz DL and 30GHz UL in Table 6.1.1.1-1 of TR 38.821 are copied as following table:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

The antenna gain is dependent on 3dB beamwidth, and 3dB beamwidth is dependent on frequency(λ) and antenna diameter (D) as following equation.

2θ-3dB=2×Arcsin1.616×λπD                                                                                                                       (Eq.1)

For satellite antenna parameter, we can use 58.5 dBi for 20GHz and 0.1768degree 3dB beamwidth as reference, the antenna gain for other frequency and antenna diameter can be calculated by following Eq.2.

Gf1=Gf0-20log10⁡2θ-3dBf12θ-3dBf0                                                                                                                        (Eq.2)

Where,

Gf0=58.5

2θ-3dBf0=0.1768 degree, which is calculated by Eq.1 with frequency is 20GHz and antenna diameter is 5m.

2θ-3dBf1 can be calculated by Eq.1 for other frequency and antenna diameter.

We can design Ku-band satellite antenna parameters for co-existence by Eq.2 and Eq.1 shown in the following table for 11GHz and 14GHz.

| Frequency range | Frequency (GHz) | Antenna diameter （m) | 3dB beamwidth (degree) | Gain (dBi) |
| --- | --- | --- | --- | --- |
| Ka-band | 20 | 5 | 0.1768 | 58.5 |
|  | 30 | 3.33 | 0.177 | 58.5 |
|  | 20 | 0.5 | 1.7684 | 38.5 |
|  | 30 | 0.333 | 1.7702 | 38.5 |
| Ku-band | 11 | 5.455 | 0.2947 | 54.1 |
|  | 14 | 4.29 | 0.2944 | 54.1 |
|  | 11 | 0.5456 | 2.9468 | 34.1 |
|  | 14 | 0.429 | 2.9446 | 34.1 |

Based on above calculated satellite antenna gain, the satellite antenna parameter in following table can be used for Ku-band coexistence simulation.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 5.455 m | 0.5456 m | 0.5456 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 54.1 dBi | 34.1dBi | 34.1dBi |
| 3dB beamwidth |  | 0.2947 deg | 2.9468 deg | 2.9468deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL ) | 4.29 m | 0.429 m | 0.429 m |
| G/T |  | 27 dB K-1 | 7 dB K-1 | 7 dB K-1 |
| Satellite RX max Gain |  | 54.1 dBi | 34.1 dBi | 34.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Base on above analysis, we propose,

Proposal 1: Use the Satellite antenna parameter in following table for Ku-band coexistence simulation.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 5.455 m | 0.5456 m | 0.5456 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 54.1 dBi | 34.1dBi | 34.1dBi |
| 3dB beamwidth |  | 0.2947 deg | 2.9468 deg | 2.9468deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL ) | 4.29 m | 0.429 m | 0.429 m |
| G/T |  | 27 dB K-1 | 7 dB K-1 | 7 dB K-1 |
| Satellite RX max Gain |  | 54.1 dBi | 34.1 dBi | 34.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

The Ka-band VSAT antenna parameters for 20GHz DL and 30GHz UL in Table 6.1.1.1-3 of TR 38.821 are copied as following table:

| Characteristics | VSAT |
| --- | --- |
| Frequency band | Ka band(30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx antenna gain | 39.7 dBi |
| Tx antenna gain | 43.2 dBi |

For satellite antenna parameter, we can use 43.2 dBi for 30GHz and 0.9824degree 3dB beamwidth as reference, the antenna gain for other frequency and antenna diameter can be calculated by following Eq.3.

Gf1=Gf0-20log10⁡2θ-3dBf12θ-3dBf0                                                                                                                        (Eq.3)

Where,

Gf0=43.2

2θ-3dBf0=0.9824 degree, which is calculated by Eq.1 with frequency is 30GHz and antenna diameter is 0.6m.

2θ-3dBf1 can be calculated by Eq.1 for other frequency and antenna diameter.

We can design Ku-band VSAT antenna parameters for co-existence by Eq.3 and Eq.1 shown in the following table for 11GHz and 14GHz.

| Frequency range | Frequency (GHz) | Antenna diameter （m) | 3dB beamwidth (degree) | Gain (dBi) |
| --- | --- | --- | --- | --- |
| Ka-band | 30 | 0.6 | 0.9824 | 43.2 |
|  | 20 | 0.6 | 1.4737 | 39.7 |
| Ku-band | 14 | 0.6 | 2.1053 | 36.6 |
|  | 11 | 0.6 | 2.6795 | 34.5 |

Based on above calculated VSAT antenna gain, the VSAT antenna parameter in following table can be used for Ku-band coexistence simulation.

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 11 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.5 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi |
| NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna |  |

Proposal 2: Use the VSAT antenna parameter in following table for Ku-band coexistence simulation.

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 11 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.5 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi |
| NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna |  |

- Conclusion

This contribution provides analysis on coexistence evaluation parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Use the Satellite antenna parameter in following table for Ku-band coexistence simulation.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 5.455 m | 0.5456 m | 0.5456 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 54.1 dBi | 34.1dBi | 34.1dBi |
| 3dB beamwidth |  | 0.2947 deg | 2.9468 deg | 2.9468deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL ) | 4.29 m | 0.429 m | 0.429 m |
| G/T |  | 27 dB K-1 | 7 dB K-1 | 7 dB K-1 |
| Satellite RX max Gain |  | 54.1 dBi | 34.1 dBi | 34.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Proposal 2: Use the VSAT antenna parameter in following table for Ku-band coexistence simulation.

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 11 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.5 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi |
| NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna |  |

- References

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

- R4-2411507,Ku Band Co-existence, Intelsat, RAN4#112