3GPP TSG-RAN WG4 Meeting #112bis                                                                                                      R4-2415084

Hefei, Anhui, China, 14th –18th October, 2024

Source: 	CATT

Title: 	Discussion on EESS protection for BS in mmWave in NR

Agenda item:	5.21.3

Document for:	Approval

- Introduction

The New WID [1] on mmWave UE spurious emission will start in RAN4#112bis meeting, the impacts of EESS on UE and BS specification need to be discussed.

In this contribution, we provide our views on impacts of EESS protection for n257 and n258 on BS specification.

- Discussion

EESS protection requirement

As per the WID [1], the objectives concerning EESS protection requirement for BS is shown as below:

The objective of this WI includes the following aspects: - For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

For EESS protection for n257 and n258, the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.104:

- 9.7.4.3.4.1	Protection of Earth Exploration Satellite Service

- 9.7.5.3.3.1	Limits for protection of Earth Exploration Satellite Service

The additional OBUE limits for EESS protection in sub-clause 9.7.4.3.4.1 of TS38.104 is copied as below:

9.7.4.3.4.1 Protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of unwanted emission shall not exceed the limits in table 9.7.4.3.4.1-1. Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; -9 dBm (Note 2); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027..

The additional spurious limits for EESS protection in sub-clause 9.7.5.3.3.1 of TS38.104 is copied as below:

9.7.5.3.3.1 Limits for protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of any spurious emissions shall not exceed the limits in Table 9.7.5.3.3.1-1. Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027..

According to WID[1], to support the commercial deployment, EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

We can add the additional OBUE limits for EESS protection in European Union as following table.

9.7.4.3.4.1 Protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of unwanted emission shall not exceed the limits in table 9.7.4.3.4.1-1. Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; -9 dBm (Note 2); 200 MHz. 23.6 – 24 GHz; -3 dBm (Note 3); 200 MHz. 23.6 – 24 GHz; -5 dBm (Note 4); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 except in European Union. NOTE 2: This limit applies to BS brought into use after 1 September 2027 except in European Union. NOTE 3: This limit applies to BS brought into use on or before 1 January 2024 in European Union. NOTE 4: This limit applies to BS brought into use after 1 January 2024 in European Union..

Observation 1: Add the additional OBUE limits for EESS protection in European Union as following table.

9.7.4.3.4.1 Protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of unwanted emission shall not exceed the limits in table 9.7.4.3.4.1-1. Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; -9 dBm (Note 2); 200 MHz. 23.6 – 24 GHz; -3 dBm (Note 3); 200 MHz. 23.6 – 24 GHz; -5 dBm (Note 4); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 except in European Union. NOTE 2: This limit applies to BS brought into use after 1 September 2027 except in European Union. NOTE 3: This limit applies to BS brought into use on or before 1 January 2024 in European Union. NOTE 4: This limit applies to BS brought into use after 1 January 2024 in European Union..

We can add additional spurious limits for EESS protection in European Union as following table.

9.7.5.3.3.1 Limits for protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of any spurious emissions shall not exceed the limits in Table 9.7.5.3.3.1-1. Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 3. 23.6 – 24 GHz; -5 dBm; 200 MHz; Note 4. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 except in European Union. NOTE 2: This limit applies to BS brought into use after 1 September 2027 except in European Union. NOTE 3: This limit applies to BS brought into use on or before 1 January 2024 in European Union. NOTE 4: This limit applies to BS brought into use after 1 January 2024 in European Union..

Observation 2: Add additional spurious limits for EESS protection in European Union as following table.

9.7.5.3.3.1 Limits for protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of any spurious emissions shall not exceed the limits in Table 9.7.5.3.3.1-1. Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 3. 23.6 – 24 GHz; -5 dBm; 200 MHz; Note 4. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 except in European Union. NOTE 2: This limit applies to BS brought into use after 1 September 2027 except in European Union. NOTE 3: This limit applies to BS brought into use on or before 1 January 2024 in European Union. NOTE 4: This limit applies to BS brought into use after 1 January 2024 in European Union..

- Conclusion

This contribution provides analysis on impacts of EESS protection for n257 and n258 on BS specification, and the following observations are concluded as follows:

Observation 1: Add the additional OBUE limits for EESS protection in European Union as following table.

9.7.4.3.4.1 Protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of unwanted emission shall not exceed the limits in table 9.7.4.3.4.1-1. Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; -9 dBm (Note 2); 200 MHz. 23.6 – 24 GHz; -3 dBm (Note 3); 200 MHz. 23.6 – 24 GHz; -5 dBm (Note 4); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 except in European Union. NOTE 2: This limit applies to BS brought into use after 1 September 2027 except in European Union. NOTE 3: This limit applies to BS brought into use on or before 1 January 2024 in European Union. NOTE 4: This limit applies to BS brought into use after 1 January 2024 in European Union..

Observation 2: Add additional spurious limits for EESS protection in European Union as following table.

9.7.5.3.3.1 Limits for protection of Earth Exploration Satellite Service For BS operating in the frequency range 24.25 – 27.5 GHz, the power of any spurious emissions shall not exceed the limits in Table 9.7.5.3.3.1-1. Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 3. 23.6 – 24 GHz; -5 dBm; 200 MHz; Note 4. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 except in European Union. NOTE 2: This limit applies to BS brought into use after 1 September 2027 except in European Union. NOTE 3: This limit applies to BS brought into use on or before 1 January 2024 in European Union. NOTE 4: This limit applies to BS brought into use after 1 January 2024 in European Union..

- References

- RP-242371, New WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE, RAN#105