3GPP TSG-RAN WG4 Meeting #112-bis                                                                                       R4-2415089

Hefei, CN, Oct. 14 – Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on UE RF requirements for the NTN new S-band

Agenda item:	5.17.2

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on NR NTN support new S-band UE RF requirement was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE RF requirements for introducing the NTN new S-band.

- Discussion

2.1 General aspect

The agreements of UE-to-UE coexistence between TN and NTN system has been made during previous meeting, as shown below:

#### Issue 1-2-5: Applicable TN Bands for Coexistence Agreement: - Focus on coexistence of S-band UL with B2/n2 and B25/n25 DL. - Capture the clarifications on the co-existence issues with B70/n70 and B66/n66 and that there is no 3GPP solution for them in the TR #### Issue 1-2-6: TN-NTN UE-to-UE Coexistence Offline Agreement: - Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering: - Realistic usage scenarios - consider UE antenna gain for smartphone of -5.5dBi - UE-to-UE separations distance - probability of the UE-to-UE coexistence scenarios occurring - Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work. - Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710) - Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted

As discussed at the last meeting, for the new NTN S-band, the UL portion is close to TN band n2 DL and TN band n25 DL. Meanwhile, it overlaps with TN bands n70 and n66. As the overlap issue between the new S band and band n70/n66 could not be resolved for now, RAN4 should initially focus on coexistence issue of TN band n2 and n25. Typically, the spurious emission requirement for UE coexistence would be applied to NTN UE to address this issue, so we believe it is reasonable to reuse this approach.

Observation 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the existing UE coexistence requirement could be considered.

Referring to the discussion of intended regions and countries, the agreements achieved as follow:

#### Issue 1-2-3: Intended Regions and Countries Agreement: Clarify that the new NTN S-band is only targeting north America.

However, for the NTN UE spec TS 38.101-5, there should be a discussion on how to indicate that the NTN new S-band is for North America only. Referring to TN operating bands n34 and n77, a note can be supplemented to the clause 5.2 operating band to clarify the intended region of the specific band. Therefore, we believed that RAN4 should discuss how to reflect the new NTN S-band is only targeting North America.

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, RAN4 should discuss how to reflect the regional requirements of the new S-band, e.g. by adding note in clause 5.2 operating band.

2.2 UE RF requirements

According to the work item objective, the new NR-NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz should support UE power class 3. Hence the maximum output power requirements could be defined as follow:

Table 2.2-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [n252] | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the maximum output power should be defined as Table 2.2-1.

In addition, for NTN UE RF requirements, not all RF requirements are band agnostic. There are certain RF requirements are affected by regional regulations, as shown in the table below:

Table 2.2-2: NTN UE RF requirements affected by regional regulations

| Section | Requirement | Note |
| --- | --- | --- |
| Tx requirements |  |  |
| 6.2.3 | UE additional maximum output power reduction | Depends on regional regulation requirements |
| 6.5.2.3 | Additional spectrum emission mask | Depends on regional regulation requirements |
| 6.5.3.2 | Spurious emissions for UE co-existence | Depends on regional regulation requirements |
| 6.5.3.3 | Additional spurious emissions | Depends on regional regulation requirements |
| Rx requirements |  |  |
| 7.3 | Reference sensitivity | Depends on regional requirements |

As for A-MPR, additional spurious emissions, additional spectrum emission mask and REFSENS for NTN new S-band, since the frequency range of new NTN S-band is same as the frequency band of LTE band 23, we believed that the requirements mentioned above could be consider to follow the similar approach s of LTE band 23, correspondingly. In addition, LTE band 23 is a very mature design and has been widely used. Adopting the LTE band 23 approaches will reduce costs and benefits the entire ecosystem.

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the A-MPR, additional spurious emissions, additional spectrum emission mask and REFSENS requirements could consider to follow the similar approach of the requirements from LTE band 23, correspondingly.

As for the spurious emissions for UE co-existence requirements of NTN new S-band, existing UE co-existence requirements in TS 38.101-5 could be considered, as described above. Therefore, we believed that the -50 dBm / MHz for UE co-existence requirements could be reused.

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the -50 dBm / MHz UE coexistence requirement could be considered.

- Conclusion

This contribution provides our initial views on UE requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Observation 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the existing UE coexistence requirement could be considered.

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, RAN4 should discuss how to reflect the regional requirements of the new S-band, e.g. by adding note in clause 5.2 operating band.

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the maximum output power should be defined as Table 2.2-1.

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the A-MPR, additional spurious emissions, additional spectrum emission mask and REFSENS requirements could consider to follow the similar approach of the requirements from LTE band 23, correspondingly.

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the -50 dBm / MHz UE coexistence requirement could be considered.

- References

- R4-2414287, WF on Rel-19 FR1 NTN bands, Inmarsat, RAN4 #112