3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415093

Hefei, CN, Oct. 14 – Oct. 19, 2024

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	6.8.2.2.1

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on UE Tx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Tx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The agreement on UE Tx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

Issue 3-1-1: General considerations - Agreement - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements. Issue 3-1-2: Consideration of band(s) Agreement: - Use 2GHz for the co-existence study - WI can be completed when the band specific requirements for one pair of {n256,256}and {n255, 255} are completed in this WI - Specify the band specific requirements for other potential NTN bands in the separate spectrum WI Issue 3-1-3: Consideration of power classes - Agreement - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed Issue 3-1-4: Consideration of regulation(s) - Agreement - Applicable regulations should be taken into account for requirements discussions. - If any identified corrections to be made for PC3, those should be considered in another work item, it is not in the current scope of this WI. Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - For PC2 IoT NTN, at least 1Tx shall be considered - For other PCs (both IoT NTN and NR NTN), FFS Issue 3-2-1: NR-NTN HPUE feasibility for different power classes Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes - Agreement of Issue 3-2-1 to Issue 3-2-3: - For PC2, it is feasible to support both handheld and non-handheld devices. - FFS other power classes for applicable device type. Issue 3-3-1: Maximum output power - Agreement - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes.

Since PC2 (NR NTN) HPUE will consider both 1 Tx and 2 Tx architectures, some requirements can be defined in the same manner as for TN HPUE. For instance, TS 38.101-1 defines the MPR and A-MPR requirements for 1 Tx HPUE and 2 Tx HPUE respectively, and the same approach may be considered for NR NTN HPUE.

Proposal 1: Different UE RF requirements such as MPR and A-MPR requirements may need to be specified to support NTN HPUE 1Tx and 2Tx architectures.

For TN HPUE that support higher power levels such as PC 1.5, only non-handheld UEs will be deployed. However, similar to PC 2 handheld HPUE, the 2Tx PC 1.5 handheld UE is also possible since the state-of-the-art RF front-end implementations already made some progress (such as deployed by using 2 Tx RF components with 26 dBm capability). For PC 1.5 non-handheld UE, the implementation could use more Tx chains. However, as mentioned above, there are no significant challenges to adopting 2 Tx implementation for non-handheld PC 1.5 HPUE. Therefore, 2 Tx architecture could be considered for both handheld and non-handheld NR NTN HPUE.

Proposal 2: If NR NTN PC2 HPUE with 1Tx architecture is commercially mature and widely supported, then for NR NTN PC 1.5 HPUE, the 2 Tx architecture could be considered for both handheld and non-handheld.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: Different UE RF requirements such as MPR and A-MPR requirements may need to be specified to support NTN HPUE 1Tx and 2Tx architectures.

Proposal 2: If NR NTN PC2 HPUE with 1Tx architecture is commercially mature and widely supported, then for NR NTN PC 1.5 HPUE, the 2 Tx architecture could be considered for both handheld and non-handheld.

- References

- R4-2414275, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #112