3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415094

Hefei, CN, Oct. 14 – Oct. 19, 2024

Source: 	CATT

Title: 	Discussion on UE Rx requirement for NTN support HPUE

Agenda item:	6.8.2.2.2

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on UE Rx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Rx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The agreement on UE Rx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

Issue 4-1: General considerations - Agreement - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class. Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point Issue 4-3: RSD for IoT-NTN - Agreement: - For NB-Iot based Iot-NTN, no RSD needed for HPUE - For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE - For eMTC based Iot-NTN with full duplex, further discuss RSD impact

For NR NTN supporting HPUE, most of the Rx requirements in TS 38.101-5 can be applied to NTN HPUE as well, as mentioned in the agreements of previous meetings. However, referring to the TN related discussion in the past release, higher UE Tx power will cause the sensitivity degradation. REFSENS may get affected due to leakage in the own Tx chain of UE, which will be exacerbated by higher power levels. For TN HPUE, the reference sensitive degradation (RSD) requirement was introduced, which could be introduced for NTN HPUE as well.

Observation 1: The reference sensitive degradation (RSD) requirement could be introduced for NTN HPUE.

Referring to the RSD requirements for TN PC2 HPUE in TS 38.101-1, different requirements were defined for UE with Tx diversity and UE without Tx diversity, as shown below:

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n1; 0; 0; 0; 0; 0; 0; -; 0; 0; 0. n2; 0.8; 0.9; 1.1; 1.2; 1.3; 2.7; 2.8; 3.5. n3; 0.5; 0.5; 0.5; 0.5; 0.6; 0.8; 1.1; 1.5; 2.3; 2.8. n7; 0.5; 0.5; 0.5; 0.5; 0.5; 0.5; 0.5; 0.5; 2.0. n8; 0.5; 0.7; 0.8; 2.3; 2.8; 3.2; 3.1. n13; 0.8; 0.9. n14; 0.6; 0.8. n25; 0.8; 0.8; 0.9; 1.1; 1.3; 2.7; 2.8; 3.5; 3.7. n66; 0; 0; 0; 0; 0; 0; 0; 0; 0. n71; 0.5; 0.9; 0.9; 2.2; 2.42 2.53; 2.52 2.43; 2.92 3.13. n70; 0; 0; 0; 0; 0. n85; 0.6; 1.0; 2.6. NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW.. Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n1; 0; 0; 0; 0; 0; 0; -; 0; 0; 0. n2; 1.2; 1.2; 1.3; 1.2; 1.2; 5.8; 6.0; 6.5. n3; 1.4; 1.5; 1.5; 1.5; 1.6; 1.7; 2.8; 5; 5.5; 6.0. n7; 0.9; 1.0; 1.0; 1.0; 1.1; 1.1; 1.1; 1.1; 5.3. n8; 1.3; 1.4; 2.1; 5.8; 6.1; 6.5; 7.0. n13; 1.2; 1.3. n14; 1.1; 1.3. n25; 1.5; 1.5; 1.6; 1.6; 1.7; 6.0; 6.2; 6.7; 7.1. n66; 0; 0; 0; 0; 0; 0; 0; 0; 0. n71; 1.1; 1.1; 1.7; 5.5; 5.92 6.93; 6.22 7.23; 6.52 7.33. n70; 0; 0; 0; 0; 0. n85; 1.2; 1.4; 6.4. NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW..

Since FR1 NR NTN UE and FR1 NR TN UE are implemented in a similar approach, the study of TN HPUE operating in single band FDD mode could be reused for NR NTN HPUE. Referring to the RSD study TR 38.861 and TR 38.896, the 2Tx architecture causes more severe interference compared to the 1Tx architecture, but the larger Tx-Rx separation prevents potential interference from the Tx chain. For those HPUE operating at the frequency band with larger Tx-Rx separation, the 2Tx architecture does not introduce severe interference compared to the 1Tx architecture, and therefore the RSD value is 0 dB, such as HPUE for band n1. Considering that the NR NTN PC2 HPUE operated in band n256 and NR TN PC2 HPUE operated in band n1 has similar Tx-Rx separation, we believed that for NR NTN PC2 HPUE in band n256, the RSD requirements could be 0 dB.

Proposal 1: For NR NTN PC 2 HPUE, the RSD value of band n256 could be 0 dB.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Observation 1: The reference sensitive degradation (RSD) requirement could be introduced for NTN HPUE.

Proposal 1: For NR NTN PC 2 HPUE, the RSD value of band n256 could be 0 dB.

- References

- R4-2414275, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #112