3GPP TSG-RAN WG4 Meeting #112bis	                                                                                       R4-2415099

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	CATT

Title: 	Discussion on AIoT device RF requirements

Agenda item:	6.21.3.2

Document for:	Discussion

- Introduction

This contribution reviews the agreements in last RAN4 meeting, some different views are proposed. For the remaining issues, we provide our further analysis and views.

- Discussion

  - Tx requirements

The following is the agreement in last RAN4 meeting.

Agreement in RAN4#112:

- Requirements potentially needed:

  - Transmit OFF power for device 2b

  - Power control requirement for device 2b

  - ON/OFF time mask for device 2b

  - Frequency error for device 2b

  - In band emissions (IBE) for device 2b

  - Carrier leakage for device 2b

  - [SEM]

  - [ACLR]

  - [Spurious emissions]

For the above agreements, the SEM, ACLR and spurious emissions are kept in brackets. They should be revisited especially for device 1 and device 2a. SEM, ACLR and spurious emission requirements are referred from NR UE requirements. They’re related to the system requirements, some implementation guidance are also implied in the requirements. For example, SEM is highly decided by the digital filter transient domain performance, ACLR is decided by the digital filter adjacent channel rejection performance and the PA linearity, spurious emission is related to many aspects such as LO harmonic, RF component linearity, RF filter performance, etc. When the three requirements can’t be met, NR UE can find the way to improve the performance. However, for AIoT passive devices, it’s not necessary to define three requirements but only one SEM or unwanted emission is sufficient to limit the emission outside the used channel because the passive devices backscatter CW signal implementation is much simpler than NR UE Tx path. The emission performance is decided mainly by the theory emission level of the reflection signal but can’t be improved much if the emission requirement is tight. Therefore, we propose to only define unwanted emission requirement for device 1 and device 2a.

Proposal 1: For Device 1 and Device 2a, only unwanted emission requirement is defined. No separate requirements such as SEM, ACLR and spurious emissions are defined.

For the following FFS requirements,

- FFS whether requirements are needed or not:

  - Maximum output power

  - EVM

  - Transmit OFF power for device 1/2a

  - Transmit time mask

  - Minimum output power

  - Frequency error for device 2a

  - In band emissions (IBE) for device 2a

  - Carrier leakage for device 2a

  - Occupied bandwidth

  - Transmit intermodulation

The analysis should be separated for passive and active devices. For passive devices, our understanding is that only radiated power is needed, other requirements are not needed. For the radiated power, our understanding is that passive devices reflect the input power to the device, the output power is the input power minus the backscattering loss, no power control is used for the UL signal, therefore no need to separate the requirements for maximum output power and the minimum output power.

Proposal 2: Radiated power is defined for Device 1 and Device 2a. No need to separate maximum output power or minimum output power.

For the other FFS requirements for device 1 and device 2a, our understanding is that if there’s no strong motivation or technical argument, no need to define other requirements.

Proposal 3: If there’s no strong motivation or technical argument, not need to define other Tx requirements for device 1 and device 2a.

For Device 2b, it seems all NR UE/NB-IoT UE requirements are on the table. However, as RAN1 system design is still in the very preliminary stage, we propose to have the tentative agreement in RAN4 for the RF requirements that NB-IoT UE RF requirements can be the reference or starting point because many details are need to be discussed to decide the requirements.

Observation: For device 2b, only tentative agreements can be reached in SI stage because the system design is not fixed. RAN4 may also need to revisit the RF architecture agreed in RAN1.

Proposal 4: The RF requirement agreement for device 2b can be revisited in WI phase.

  - Rx requirements

The following is the agreement in RAN4#112,

Agreement in RAN4#112:

- Requirements potentially needed:

  - Reference sensitivity

Proposal 5: No Rx requirements requirement for device 1 and device 2a.

For other Rx requirements for device 2b, we have the same view in proposal 4 that the agreements should be careful that all of them should be able to be revisited in WI phase.

- Conclusion

This contribution provides our further analysis for the remaining issues for AIoT device requirements. We have the following observations and proposals,

Proposal 1: For Device 1 and Device 2a, only unwanted emission requirement is defined. No separate requirements such as SEM, ACLR and spurious emissions are defined.

Proposal 2: Radiated power is defined for Device 1 and Device 2a. No need to separate maximum output power or minimum output power.

Proposal 3: If there’s no strong motivation or technical argument, not need to define other Tx requirements for device 1 and device 2a.

Observation: For device 2b, only tentative agreements can be reached in SI stage because the system design is not fixed. RAN4 may also need to revisit the RF architecture agreed in RAN1.

Proposal 4: The RF requirement agreement for device 2b can be revisited in WI phase.

Proposal 5: No Rx requirements requirement for device 1 and device 2a.

- References

[1] R4-2414305, “WF on RF requirements for A-IoT”, RAN4#112, Huawei

[2] R4-2411072, “Discussion on AIoT device RF requirement, RAN4#112, CATT