3GPP TSG-RAN WG4 Meeting #112bis	                                                                                       R4-2415101

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	CATT

Title: 	Discussion on U6GHz EEIRP mask requirement test

Agenda item:	6.11.4

Document for:	Discussion

- Introduction

There’re still some open issues for the U6GHz EEIRP mask conformance test according to the WF [1] in last RAN4 meeting. This contribution provides our views on the declarations and the RF channels.

- Discussion

  - Declarations

For the declarations, there’re some agreements in RAN4#111 and RAN4#112.

Agreement in RAN4##111 [1],

Table 2.2.2-1: [CAR] declarations

| Parameter | Description |
| --- | --- |
| min | Minimum angle along  axis |
| max | Maximum angle along  axis |
| min | Minimum angle along  axis |
| max | Maximum angle along  axis |

- NOTE 1: further discuss the definition of min   max min max

- NOTE 2: reusing the existing declaration is not precluded.

Agreement in RAN4#112 [2],

To declare the supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing

For the CAR declaration agreement in RAN4#111, there’re different views in last RAN4 meeting that CAR declaration or reusing current beam steering directions declaration are not decided. They’re also related to how to test the EEIRP mask requirement. For the CAR declaration approach, the test beams distribution and the weights for the beams can be defined in the spec considering the test time and test accuracy. A uniformly distributed beam set is identified as a compromise for some acceptable accuracy in the contribution [3], the issue may be that if this beam set aligns with the real BS operation. For using beam steering directions declaration approach, it seems the beams declaration can be more close to the real BS operation [4], there’s also some issue that how to distribute the uniform beams if the current 5 peak beam directions are reused. There can be another argument that if the whole set of test beams can be declared or be defined in the specification with some criterions. With all of the analysis and discussion, our understanding is that a totally new declaration may not be very necessary. Reusing current beam declaration approach may be possible but with some additional clarifications on how to adapt to the new EERIP requirement for the test beam set. It’s also possible to add more beam declarations to help to decide the test beam set.

Observation: Reusing current beam declaration approach may be possible but with some additional clarifications on how to adapt to the new EERIP requirement for the test beam set.

  - RF channels for the conformance testing

For the following issue,

Issue 4-4: RF channels for the conformance testing

  - Proposal 1: B, T [Nokia]

  - Proposal 2: M. [Huawei]

  - Proposal 3: T [Ericsson]

Our understanding is that top range of the 6425 – 7075 MHz range can be tested because the emission of this range Tx signal is the most difficult case.

Proposal: Top range of the 6425 – 7075 MHz range is tested for the EEIRP requirement test.

- Conclusion

This contribution provides our views on the declaration and the RF channels, we have the following observation and proposal.

Observation: Reusing current beam declaration approach may be possible but with some additional clarifications on how to adapt to the new EERIP requirement for the test beam set.

Proposal: Top range of the 6425 – 7075 MHz range is tested for the EEIRP requirement test.

- References

[1] R4-2413509, “Way Forward for [112][305] NR_BS_RF”, ZTE, RAN4#112b

[2] R4-2413405, “Topic summary for [112][305] NR_BS_RF”, ZTE, RAN4#112b

[3] R4-2413220, “Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104”, Ericsson, RAN4#112b

[4] R4-2413276, “Expected EIRP conformance – Test Vectors, MU budget and Conformance test method”, Huawei, RAN4#112b