3GPP TSG-RAN WG4 Meeting # 112bis                                                          R4-2415108

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on modification of existing Tx requirements for SBFD operation

Source: 	CATT

Agenda item:	6.20.2.2

Document for:	Discussion

- Background

WF [1] of RAN4#112 concluded that further studies are needed for some TX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Tx RF Requirement

  - ACLR and OBUE

Although this requirement is called OBUE, the intention is to define some new and more stringent requirement at the 1st adjacent channel frequency range.

The motivation is to enable two SBFD-capable BSs to be deployed co-location, and the operating frequency band of the two BSs is adjacent. A preliminary for the link analysis is provided in Table 1.

For FR1 aggressor WA BS, the output power is 49dBm, bandwidth of channel is 100MHz, if assume the degradation of victim BS sensitivity is 1dB, the required total isolation between the two BSs is 144dB, that is, the sum of ACLR, couple loss of co-existence and the rejection of RF BPF used after PA must larger than 144dB.

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim BS sensitivity(dB) | [1] |
| Required total isolation(dB) | 144 |
| 1st ACLR(dB) | [45] |
| Coupling loss of co-location (dB) | [30] |
| Rejection of RF BPF used after PA (dB) | 69 |

Couple loss of co-location is depended with operating frequency and distance between two BSs. According to [2], coupling loss increases with the increase of BS operating frequency. Existing 30 dB coupling loss is suitable for frequency range 0.45 – 2 GHz. For higher operating frequencies, other antenna port-to-port isolation should be considered.  RF BPF used after PA must be high power filter, and rejection of BPF is depended with operating frequency, passband width and filter type. For large channel bandwidths, the 1st ACLR will not improve much.

If ACLR is 45dB, coupling loss of co-location is 30dB，rejection from RF BPF at victim BS UL operating frequency range must be larger than 69dB, if 69 dB rejection can’t be achieved, couple loss of co-location may be improved by increase distance between two BSs, and the frequency impact on coupling loss can also be accounted for.

Observation 1: For WA BS, if victim BS DESENS is 1dB and CL for co-location BS is 30 dB, RF BPF rejection performance should be larger than 69dB which is a stringent requirement for some RF filters.

Proposal 1: We propose to consider the frequency impact on coupling loss of co-location.

  - Transmitter signal quality

Joint measurement requires a new complex test model, and the start time position of EVM test is very critical when switching between SBFD slot and non-SBFD slot, and the test results will be affected by this time point, so it would be beneficial to measure the mean EVM of SBFD and non-SBFD slots separately.

Proposal 2: Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

EVM is a Tx requirement, when calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account, and data from SBFD ULs needn’t to be included.

Proposal 3: When calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account.

- Summary

In this contribution, we have the following proposals and observations.

Observation 1: For WA BS, if victim BS DESENS is 1dB and CL for co-location BS is 30 dB, RF BPF rejection performance should be larger than 69dB which is a stringent requirement for some RF filters.

Proposal 1: We propose to consider the frequency impact on coupling loss of co-location.

Proposal 2: Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Proposal 3: When calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account.

- Reference

[1] R4-2413515, “Way Forward Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, HiSilicon, RAN4#112

[2] R4-1706766, “Co-location isolation between different frequency bands”, Ericsson, RAN4 NR#2