3GPP TSG-RAN WG4 Meeting # 112bis                                                       R4-2415111

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	                 Discussion on A-IoT BS requirements

Source: 	CATT

Agenda item:	6.21.3.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS Tx, Rx and CW requirements according to the WF [1].

- A-IoT BS RF requirements analysis

2.1 Tx requirements

In A-IoT system, there is no operation on RE, but RB is treated as the minimum processing unit, so RE power control dynamic range requirement is not needed.

The signal resources of R2D can be several RBs, so total power dynamic range requirement is necessary.

In the current BS Tx requirements, transmit OFF power requirement only applies to TDD operation, if the spectrum of R2D and D2R/CW are same and CW node is outside the topology, the requirement is necessary, otherwise, the requirement is not needed, so whether the frequencies of R2D and CW/D2R can be the same need to be considered.

A-IoT BS doesn’t transmit R2D signal all the time and the transmission needs to be turned on and off, so transmitter transient period is necessary.

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. If the operating bands of A-IoT BS R2D signal and legacy BS DL are adjacent and the two BS are also co-located, the transmitter intermodulation requirement is necessary, otherwise, it is not necessary, and we can discuss necessary of this requirement based on the simulation results.

Table 1: A-IoT BS potential Tx applicable requirements

| No. | Requirement list |  | Requirement necessary? |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WF[1] | CATT |  |
| 1 | BS output power |  | Yes | Yes |  |
| 2 | Power dynamic range | RE power control dynamic range | FFS | No |  |
|  |  | Total power dynamic range | FFS | Yes |  |
| 4 | Transmit ON/OFF power | Transmitter OFF power | FFS | FFS |  |
| 5 |  | Transmitter transient period | FFS | Yes |  |
| 6 | Transmitted signal quality | Frequency error | Yes | Yes |  |
| 7 |  | EVM | Yes | Yes |  |
| 8 |  | TAE | NA | NA |  |
| 9 | Unwanted emissions | Occupied bandwidth | Yes | Yes |  |
| 10 |  | ACLR | Yes | Yes |  |
| 11 |  | Operating band unwanted emissions | Yes | Yes |  |
| 12 |  | Transmitter spurious emissions | Yes | Yes |  |
| 13 | Transmitter intermodulation |  | FFS | FFS |  |

2.2 Rx requirements

If A-IoT system coexists with NR system, and their operating frequency bands are adjacent, in-band blocking requirement is necessary, and if the distances between NR UEs and A-IoT BSs are relatively small, narrow-band blocking, receiver intermodulation and narrowband intermodulation requirements may be needed.

The dynamic range requirement is related to the maximum co-channel input power, and the UL signals from other devices and the leakage power of NR BS Tx output to the adjacent RB which is A-IoT BS operating RB are interference signals. The interference power value is related to the A-IoT UL RB configuration, and whether the NR BS and A-IoT BS are co-location is also important, we can discuss this requirement after RAN1 has decided the UL RB configuration and RAN4 has completed the simulation.

ICS is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density, but device 1 and device 2a are located at fix positions, and the received signal levels from these devices may not be higher, so the ICS requirement may not be required.

Table 2: A-IoT BS potential Rx requirements

| No. | Requirement list | Requirement necessary? |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | WF[1] | CATT |  |
| 1 | Reference sensitivity level | Yes | Yes |  |
| 2 | Dynamic range | FFS | FFS |  |
| 3 | ICS | FFS | No |  |
| 4 | ACS | Yes | Yes |  |
| 5 | In-band blocking | FFS | Yes |  |
| 6 | Narrow-band blocking | FFS | Yes |  |
| 7 | Out-band blocking | FFS | Yes |  |
| 8 | Receiver spurious emissions | Yes | Yes |  |
| 9 | Receiver intermodulation | FFS | Yes |  |
| 10 | Narrowband intermodulation | FFS | Yes |  |

2.3 CW cancellation

For CW signal，output power and spurious emissions are necessary. Frequency error of CW can affect the frequency error of the D2R signal, which ultimately affects the quality of the signal received by A-IoT BS, so it is needed. According to modulation scheme of R2D signal, frequency error requirement is sufficient, phase noise requirement may not be needed.

Table 3: A-IoT BS CW requirements

| No. | Requirement list | Requirement necessary? |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | WF[1] | CATT |  |
| 1 | Output power | Yes | Yes |  |
| 2 | Spurious emissions | FFS | Yes | CW harmonics need to be considered |
| 3 | Frequency error |  | Yes |  |
| 43 | Phase noise | FFS | No |  |

- Summary

This contribution provides our further analysis for A-IoT BS requirements. We have the following observations.

Proposal 1: For the A-IoT BS Tx RF requirements, views from our side are provided in Table 1.

Proposal 2: For the A-IoT Rx BS RF requirements, views from our side are provided in Table 2.

Proposal 3: For the A-IoT BS CW requirements, views from our side are provided in Table 3.

Reference

[1] R4-2414305, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#112