3GPP TSG-RAN WG4 Meeting # 112bis                                                         R4-2415112

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	                 Discussion on A-IoT intermediate UE requirements

Source: 	CATT

Agenda item:	6.21.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for AIoT intermediate UE Tx, Rx and CW requirements according to the WF [1].

- A-IoT intermediate UE requirements analysis

2.1 Tx requirements

Power control is not used for AIoT DL, intermediate UEs need to transmit at maximum output power so minimum output power requirement is not necessary.

Transmit OFF power requirements apply only to TDD operation, for D2T2, the R2D signal uses the FDD UL spectrum, if the CW node is outside the topology and the CW uses the FDD DL spectrum, then the frequency ranges of the R2D signal and CW are different, and this requirement does not need to be defined. Even if the CW node is inside the topology and CW uses the FDD UL spectrum, whether the frequencies of R2D and CW/D2R can be the same need to be considered.

A-IoT BS doesn’t transmit R2D signal all the time and the transmission needs to be turned on and off, so ON/OFF time mask is necessary.

UE transmitting in close vicinity of each other can produce intermodulation products, which can fall into the UE, or BS receive band as an unwanted interfering signal, so transmit intermodulation is needed.

Table 1: A-IoT intermediate UE potential Tx applicable requirements

| No. | Requirement list |  |  | Requirement necessary? |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | WF[1] | CATT |  |
| 1 | Transmit power | UE maximum output power |  | Yes | Yes |  |
| 2 | Output power dynamics | Minimum output power |  | FFS | No |  |
| 3 |  | Transmit OFF power |  | FFS | FFS |  |
| 4 |  | ON/OFF time mask |  | FFS | Yes |  |
| 5 | Transmit signal quality | Frequency error |  | Yes | Yes |  |
| 6 |  | EVM |  | Yes | Yes |  |
| 7 |  | In-band emissions |  | FFS | No |  |
| 8 | Output RF spectrum emissions | Occupied bandwidth |  | Yes | Yes |  |
| 9 |  | Out of band emission | Spectrum emission mask | Yes | Yes |  |
| 10 |  |  | ACLR | Yes | Yes |  |
| 11 |  | Spurious emissions |  | Yes | Yes |  |
| 12 | Transmit intermodulation |  |  | Yes | Yes |  |

2.2 Rx requirements

If A-IoT system coexists with NR system, and their operating frequency bands are adjacent, in-band blocking, ACS, narrow-band blocking and receiver intermodulation requirements may be needed.

ICS is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density, but device 1 and device 2a are located at fix positions, and the received signal levels from these devices may not be higher, so the ICS requirement may not be required..

Spurious response is NR UE requirement, and it is FFS for intermediate UEs, the necessary of this requirement can be discussed in WI phase.

Table 2: A-IoT intermediate UE potential Rx requirements

| No. | Requirement list | Requirement necessary? |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | WF[1] | CATT |  |
| 1 | Reference sensitivity power level | Yes | Yes |  |
| 2 | Maximum input level | Yes | Yes |  |
| 3 | ICS | NA | No |  |
| 4 | ACS | FFS | Yes |  |
| 5 | In-band blocking | FFS | Yes |  |
| 6 | Out-of-band blocking | FFS | Yes |  |
| 7 | Narrow band blocking | FFS | Yes |  |
| 8 | Spurious response | FFS | FFS |  |
| 9 | Intermodulation characteristics | Yes | Yes |  |
| 10 | Spurious emissions | FFS | Yes |  |

2.3 CW cancellation

For CW signal, output power and spurious emissions are necessary. Frequency error of CW can affect the frequency error of the D2R signal, which ultimately affects the quality of the signal received by intermediate UE, so it is needed. According to modulation scheme of R2D signal, frequency error requirement is sufficient, phase noise requirement may not be needed.

Table 3: A-IoT intermediate UE CW requirements

| No. | Requirement list | Requirement necessary? |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | WF[1] | CATT |  |
| 1 | Output power | Yes | Yes |  |
| 2 | Spurious emissions | FFS | Yes | CW harmonics need to be considered |
| 3 | Frequency error |  | Yes |  |
| 4 | Phase noise |  | No |  |

- Summary

This contribution provides our further analysis for AIoT intermediate UE requirements. We have the following observations.

Proposal 1: For the A-IoT intermediate UE Tx RF requirements, views from our side are provided in Table 1.

Proposal 2: For the A-IoT intermediate UE Rx RF requirements, views from our side are provided in Table 2.

Proposal 3: For the A-IoT intermediate UE CW requirements, views from our side are provided in Table 3.

Reference

[1] R4-2414305, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#112