3GPP TSG-RAN WG4 Meeting # 112bis                                                        R4-2415113

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	UE RF requirements for new NR bands n87 and n88

Source: 	CATT

Agenda item:	5.15.2

Document for:	Approval

- Background

In the WF[1] of  RAN4#112 meeting, UE RF requirements of NR bands n87 and n88 need to be analysed whether aligned with the regulation from in ECC/Dec/(16)02 and ECC/Dec/(19)02, we provide our views in this paper.

- BS RF Requirements

Agreed changes of UE RF requirements for n87 and n88 in WF [1] are listed as following,

2.2	UE RF requirements

Agreed changes are listed in Table 2.2-1 for both bands separately. As a reference LTE band 31 and 72 requirements are listed.

Table 2.2-1: Changes for UE specification 38.101-1 due to introduction of bands n87 and n88

| Requirement | B87 in 36.101 | n87 proposal | B88 in 36.101 | n88 proposal |
| --- | --- | --- | --- | --- |
| Operating bands | 410 MHz – 415 MHz 420 MH z– 425 MHz | Re-use LTE | 412 MHz – 417 MHz 422 MH z– 427 MHz | Re-use LTE |
| Channel bandwidths | 1.4, 3 and 5 MHz | 3 and 5 MHz | 1.4, 3 and 5 MHz | 3 and 5 MHz |
| EARFCN / NR-ARFCN | LTE band specific | Needs n87 specific | LTE band specific | Needs n88 specific |
| Sync-raster | Not in LTE | Needs n87 specific | Not in LTE | Needs n88 specific |
| TX–RX frequency separation | 10 MHz | 10 MHz | 10 MHz | 10 MHz |
| UE maximum output power | PC1 and PC3 | PC1 and PC3 | PC1 and PC3 | PC1 and PC3 |
| MPR | Not band but LTE specific | PC3 MPR is not band specific. PC1 reuse MPR for PC1 for bands other than Band n14 | Not band but LTE specific | PC3 MPR is not band specific. PC1 reuse MPR for PC1 for bands other than Band n14 |
| A-MPR | No A-MPR | Not needed | No A-MPR | Not needed |
| Output power dynamics | Not band specific | Not band specific | Not band specific | Not band specific |
| Transmit signal quality | Not band specific | Not band specific | Not band specific | Not band specific |

| Occupied bandwidth | Not band specific | Not band specific | Not band specific | Not band specific |
| --- | --- | --- | --- | --- |
| Spectrum emission mask | Not band specific | Not band specific | Not band specific | Not band specific |
| ACLR | Not band specific | NR ACLR is not a band specific. UTRAACLR is not applicable. | Not band specific | NR ACLR is not a band specific. UTRAACLR is not applicable. |
| General spurious emissions | Not band specific | Not band specific | Not band specific | Not band specific |
| Spurious emissions for UE co-existence | Is band specific | LTE requirement is a baseline | Is band specific | LTE requirement is a baseline |
| Transmit intermodulation | Not band specific | Not band specific | Not band specific | Not band specific |
| REFSENS power level | -95.7 for 3 MHz -93.5 for 5 MHz | Re-use LTE REFSENS as NRB is same for LTE and NR | -95.7 for 3 MHz -93.5 for 5 MHz | Re-use LTE REFSENS as NRB is same for LTE and NR |
| REFSENS UL allocation | 5 RB’s with Note 4 restriction | Re-use LTE specification with note 4 content | As an error not specified, maintenance CRs are provided into this meeting 5 RB’s with Note 4 restriction | Re-use LTE specification with note 4 content |
| Max input level | Not band specific | Not band specific | Not band specific | Not band specific |
| ACS | Not band specific | Not band specific | Not band specific | Not band specific |
| IBB | Is band specific | Add n87 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table | Is band specific | Add n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table |
| OBB | Is band specific | Add n87 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table | Is band specific | Add n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table |
| NBB | Not band specific | Add n87 into NBB table | Not band specific | Add n88 into NBB table |
| Spurious response | Not band specific | Is frequency specific but no change is needed | Not band specific | Is frequency specific but no change is needed |
| Wideband intermodulation | Not band specific | Is frequency specific but no change is needed | Not band specific | Is frequency specific but no change is needed |

| Spurious emissions | Not band specific | Not band specific | Not band specific | Not band specific |
| --- | --- | --- | --- | --- |

With the above table being the baseline, reassure that the UE RF requirements are aligned with the regulation captured in:

- Table 6 and table 7 of Annex 3 in ECC/Dec/(16)02, and

- Table 5 and table 6 of Annex 2 in ECC/Dec/(19)02.

In Annex 3 in ECC/Dec/(16)02[3], table 6 includes BB-PPDR UE channel band and maximum mean in-block power,  table 7 is BB-PPDR UE maximum unwanted emission levels, the details are listed as following,

In Annex 2 in ECC/Dec/(19)02[4], table 5 includes channel band and maximum mean in-block power,  table 6 is UE maximum unwanted emission levels, the details are listed as following,

The objectives of WID don’t include 1.4MHz channel width and higher UE power requirement, so these two features in [3] and [4] do not need to be taken into account.

Comparing the spectrum emission masks of 3MHz and 5MHz channel widths in TS38.101-1[2] with the UE maximum unwanted emission levels in [3] and [4], the other levels are aligned except for level of ±0-1 MHz frequency offset from channel edge for 5MHz channel width. The level in TS 38.101-1 is -13dBm/50KHz, i.e. -15.2dBm/30KHz, which is 0.2dB stricter than the levels in [3] and [4].

Observation 1: For±0-1MHz frequency offset from channel edge for 5MHz channel width, the level in TS 38.101-1 is 0.2dB stricter than the level in [3] and [4].

Proposal 1: For±0-1 MHz frequency offset from channel edge for 5MHz channel width, the 0.2 dB strict value of TS 38-101-1 is small and can be ignored for spectrum emission masks.

Proposal 2: Spectrum emission masks of 3MHz and 5MHz channel widths in TS 38.101-1 are aligned with UE maximum unwanted emission levels in ECC/Dec/(16)02 and ECC/Dec/(19)02.

System parameters in R4-2411945 were agreed in WF [1], we also provide a draft CR [5], and it introduces system parameters and UE RF requirements for the two new bands to TS 38.101-1.

- Summary

In this contribution, we have the following observation and proposals,

Observation 1: For±0-1MHz frequency offset from channel edge for 5MHz channel width, the level in TS 38.101-1 is 0.2dB stricter than the level in [3] and [4].

Proposal 1: For±0-1 MHz frequency offset from channel edge for 5MHz channel width, the 0.2 dB strict value of TS 38-101-1 is small and can be ignored for spectrum emission masks.

Proposal 2: Spectrum emission masks of 3MHz and 5MHz channel widths in TS 38.101-1 are aligned with UE maximum unwanted emission levels in ECC/Dec/(16)02 and ECC/Dec/(19)02.

- Reference

[1] R4-2414272, “WF on introduction of NR bands n87 and n88”, Nokia, RAN4#112

[2] TS38.101-1

[3] ECC/Dec/(16)02 	Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems, https://docdb.cept.org/document/941

[4] ECC/Dec/(19)02 	Land mobile systems in the frequency ranges 68-87.5 MHz, 146-174 MHz, 406.1-410 MHz, 410-430 MHz, 440-450 MHz and 450-470 MHz, https://docdb.cept.org/document/9680

[5] R4-2415114, “draft CR to TS 38.101-1 on introduction NR bands n87 and n88”, CATT, RAN4#112bis