3GPP TSG-RAN WG4 Meeting # 112bis                                             R4-2415116

# Hefei, Anhui, China, 14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft CR | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133- Introduction of NR band n68 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_band_n68-Core |  |  |  |  |  | Date: |  |  | 2024-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add NR band n68 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RF requirements for new band n68 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new NR band n68 won’t be correctly supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of Change >

3.5.2	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3.5.2-1.

Table 3.5.2-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  | NR CCA10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | n1, n18, n24, n70, n744, n91, n92, n93, n94, n100, n109 | NR_TDD_FR1_A | n34, n389, n39, n40, n50, n51, n53, n54, n101 | NR_SDL_FR1_A | n67, n75, n76 | NR_CCA_FR1_A | - |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | n387 | NR_SDL_FR1_B | - | NR_CCA_FR1_B | - |
| C | NR_FDD_FR1_C | n30 | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - | NR_CCA_FR1_C | - |
| D | NR_FDD_FR1_D | n28, n68 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - | NR_CCA_FR1_D | - |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41, n90 | NR_SDL_FR1_E | - | NR_CCA_FR1_E | - |
| F | NR_FDD_FR1_F | n266 | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - | NR_CCA_FR1_F | - |
| G | NR_FDD_FR1_G | n3, n8, n12, n13, n14, n20, n71, n85, n10511, n106 | NR_TDD_FR1_G | n104 | NR_SDL_FR1_G | n29 | NR_CCA_FR1_G | - |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - | NR_CCA_FR1_H | - |
| I | NR_FDD_FR1_I | - | NR_TDD_FR1_I |  | NR_SDL_FR1_I | - | NR_CCA_FR1_I | n46 |
| J | NR_FDD_FR1_J | - | NR_TDD_FR1_J | n478 | NR_SDL_FR1_J | - | NR_CCA_FR1_J | n96, n102 |
| K | NR_FDD_FR1_K | - | NR_TDD_FR1_K | - | NR_SDL_FR1_K | - | NR_CCA_FR1_K | - |
| L | NR_FDD_FR1_L | - | NR_TDD_FR1_L | - | NR_SDL_FR1_L | - | NR_CCA_FR1_L | - |
| M | NR_FDD_FR1_M | - | NR_TDD_FR1_M | - | NR_SDL_FR1_M | - | NR_CCA_FR1_M | - |
| N | NR_FDD_FR1_N | n31, n72 | NR_TDD_FR1_N | - | NR_SDL_FR1_N | - | NR_CCA_FR1_N | - |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is only used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 8: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 9: When this band is only used for WAN service. NOTE 10: Operating bands where operation on carrier frequencies with CCA is supported. NOTE 11: The minimum Io condition is reduced by 0.5 dB when the downlink channel overlap the 612-617 MHz frequency range and the channel bandwidth is 5 MHz. |  |  |  |  |  |  |  |  |

< End of Change >