3GPP TSG-RAN WG4 Meeting #112bis	R4-2415127

Hefei, China, Oct 14th – 18th, 2024

Title: 	Discussion on impacts on UE RF requirements and DL performance

Agenda Item:	6.6.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4#112 has initiated the study for fragmented DL carriers [1] and a WF was approved where the consensus is mostly on some clarification on the scope [2]. Regarding impacts on UE RF requirements and DL performance, discussions were held, but no consensus was achieved, so these issues remain open for further discussion.

In this contribution, we will continue the discussion on the open issues and share our perspectives and proposals.

# 2 Discussion

Since the study is to identify the methods of reducing the number of UE Rx chains for intra-band non-contiguous CA with two CCs, the potential impacts on UE RF requirements should be restricted to Rx RF requirements only, which impacts are limited to the downlink between BS and the UE enabling this feature, hence no any relaxation on Tx RF requirements should be allowed.

If relaxing some transmission RF requirements, it may degrade network performance, leading to increased unwanted emissions or spurious signals, potentially interfering with other users or networks, dropped calls, reduced data throughput, or interference with neighbouring cells, hence it is important that the feature does not come at the cost of transmission stability and performance, which are critical for ensuring high-quality service for all users in the network.

Proposal 1: No relaxation on any Tx RF requirements should be assumed in this study item.

Regarding the combination of intra-band non-contiguous CA with MIMO, it is clear that the current specs does not support intra-band non-contiguous UL CA with UL MIMO, however, if single uplink or UL CA without UL-MIMO is configured, DL MIMO for the two non-contiguous CCs may be still feasible. However, for the sake of the simplicity, RAN4 may start with the case where DL MIMO is not enabled for the two non-contiguous CCs.

Proposal 2: RAN4 to start with the case where DL MIMO is not enabled for the two non-contiguous CCs and only single uplink or UL CA without UL-MIMO is configured.

# 3 Conclusion

In this contribution, we have the following proposals on impacts on UE RF requirements and DL performance:

Proposal 1: No relaxation on any Tx RF requirements should be assumed in this study item.

Proposal 2: RAN4 to start with the case where DL MIMO is not enabled for the two non-contiguous CCs and only single uplink or UL CA without UL-MIMO is configured.

# Reference

[1] RP-242404, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek

[2] R4-2414457, “WF on fragmented DL carriers study”, MediaTek