3GPP TSG-RAN WG4 Meeting #112bis	R4-2415130

Hefei, China, Oct 14th – 18th, 2024

Title: 	Further discussion on system parameters for LP-WUS

Agenda Item:	6.23.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, discussion on whether RAN4 needs to specify channel raster for LP-WUS still remains open as shown in the following [1]:

Issue 1-2-1: Channel raster for LP-WUR Agreements: - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS. - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

During the discussion, we realized that there are different understandings on the purpose of introducing channel raster for LP-WUS, hence in this contribution, we further explain the purpose in addition to the detailed channel raster entries for LP-WUS.

# 2 Discussion

Purpose of introducing channel raster

In general, there may be two main purposes of introducing channel raster:

- Indicating the possible positions of LP-WUS in the frequency domain

- Indicating the frequencies generated by frequency synthesizer at LP-WUR to be able to receive LP-WUS

For (1), LP-WUR may detect the actual LP-WUS based on LP-SS, or it can be notified or indicated directly in some other way, e.g., in RRC_CONNECTED state, the centre operation frequency of LP-WUS can be indicated via message over NR. However, it does not solve (2), since from specification perspective, there should be a set of frequencies which an LP-WUR must be able to generate via its frequency synthesizer, hence RAN4 needs to specify new channel raster for LP-WUS.

Observation 1: Even if the centre operation frequency of LP-WUS could be indicated from RAN1 or RAN2 perspective, the set of frequencies generated by an LP-WUR’s frequency synthesizer still needs to be specified.

Proposal 1: RAN4 to introduce new channel raster for LP-WUS.

New channel raster design for LP-WUS

As discussed in our previous paper [2], since LP-WUS is embedded in an NR carrier, its PRB grid should be aligned with that of NR signal for keeping orthogonality and minimizing the interference from implementation imperfection.

Observation 2: PRB grid should be aligned between LP-WUS and NR.

Proposal 2: RAN4 to ensure PRB grid alignment between LP-WUS and NR when designing new channel raster for LP-WUS.

For the placement of an LP-WUS within an NR carrier, it is natural that it should be flexible and no restriction should be specified, hence RAN4 needs to ensure an LP-WUS carrier can be flexibly allocated within the wider NR carrier when designing new channel raster for LP-WUS.

Proposal 3: RAN4 to ensure an LP-WUS carrier can be flexibly allocated within the wider NR carrier when designing new channel raster for LP-WUS.

It is obvious that for NR bands with SCS based channel raster, the existing NR channel raster can be re-used for LP-WUS.

Proposal 4: For an NR band with SCS based channel raster, the existing NR channel raster is re-used for LP-WUS within the NR band.

However, for an NR bands with 100kHz channel raster, the existing NR channel raster cannot ensure Proposal 2 and 3. We may look into different cases in order to design the required channel raster for LP-WUS.

Fig. 1 illustrates four different cases where an LP-WUS carrier is allocated within the PRB grid of the corresponding NR carrier (with 100kHz channel raster) based on the PRB numbers’ oddity of the two carriers.

- Fig. 1a indicates the case where the PRB number of the NR carrier is even while that of LP-WUS carrier is odd, i.e., Case 1.

- Fig. 1b indicates the case where the PRB number of the NR carrier is even while that of LP-WUS carrier is even, i.e., Case 2.

- Fig. 1c indicates the case where the PRB number of the NR carrier is odd while that of LP-WUS carrier is odd, i.e., Case 3.

- Fig. 1d indicates the case where the PRB number of the NR carrier is odd while that of LP-WUS carrier is even, i.e., Case 4.

Fig. 1, channel raster for WUS carrier

As shown in Fig. 1, the distance D between the LP-WUS carrier and NR carrier can be expressed as:

- q * SCS * 12 + 6 * SCS for Case 1 and 4;

- q * SCS * 12 for Case 2 and 3.

Where q is an integer.

The channel raster of LP-WUR, labelled as cr(t),  can be calculated as:

cr(t) = n*100kHz + q*SCS*12 + m*6*SCS           (1)

where n, q is an integer, and m = 0 or 1.

Let’s take 15kHz SCS as an example. The Eq. (1) is further expressed as:

cr(t) = n*100kHz + q*180kHz + m*90kHz           (2)

Actually, Eq. (2) can be simplified as:

cr(t) = n*100kHz +p * 90kHz           (3)

where p is an integer between -272 and 272.

The final set indicated by Eq. (3) is {10kHz, 20kHz, 30kHz, …, m*10kHz, …}, which means cr(t) = t * 10k. Therefore, a channel raster with 10kHz can ensure the flexible allocation of an LP-WUS carrier within a wider NR carrier with 100kHz channel raster.

Proposal 5: RAN4 to introduce 10kHz channel raster for LP-WUR for an NR band with 100kHz channel raster to ensure an LP-WUS carrier can be flexibly allocated within an NR carrier in that NR band.

# 3 Conclusion

In this contribution, we have the following observations and proposals on system parameters for LP-WUS:

Observation 1: Even if the centre operation frequency of LP-WUS could be indicated from RAN1 or RAN2 perspective, the set of frequencies generated by an LP-WUR’s frequency synthesizer still needs to be specified.

Observation 2: PRB grid should be aligned between LP-WUS and NR.

Proposal 1: RAN4 to introduce new channel raster for LP-WUS.

Proposal 2: RAN4 to ensure PRB grid alignment between LP-WUS and NR when designing new channel raster for LP-WUS.

Proposal 3: RAN4 to ensure an LP-WUS carrier can be flexibly allocated within the wider NR carrier when designing new channel raster for LP-WUS.

Proposal 4: For an NR band with SCS based channel raster, the existing NR channel raster is re-used for LP-WUS within the NR band.

Proposal 5: RAN4 to introduce 10kHz channel raster for LP-WUR for an NR band with 100kHz channel raster to ensure an LP-WUS carrier can be flexibly allocated within an NR carrier in that NR band.

# Reference

[1] R4-2414309, “WF on LP-WUS UE RF requirements”, vivo

[2] R4-2411905, “Further discussion on system parameters for LP-WUS”, CATT