3GPP TSG-RAN WG4 Meeting #112bis	R4-2415131

Hefei, China, Oct 14th – 18th, 2024

Title: 	On Rx requirements of REFSENS, ASCS and ACS for LP-WUS

Agenda Item:	6.23.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, the discussion on REFSENS, ASCS and ACS for LP-WUS continues, and some basic agreements have been made for further studies [1]. It appears that SNR value should be determined at first which is the basis for many Rx requirements. In addition, there is also an open issue whether a single SNR value for both OOK-based and OFDM-based receiver:

Issue 2-1-4: Target SNR value Agreements: - RAN4 further discuss target SNR value for LP-WUS - FFS single SNR for OOK-based and OFDM-based receiver

In this contribution, we provide preliminary simulation results for SNR value and also present our views on the open issue and defining REFSENS, ASCS and ACS.

# 2 Discussion

Simulated SNR value

With the simulation assumptions agreed in previous meetings, we simulated OOK-based receiver at first and for the sake of simplicity we keep sending LP-WUS hence only obtain statistics on miss detection, and not on false alarm. The simulation result is illustrated in Fig. 1 below, and the required SNR at 1% miss detection is around -3.3dB.

Fig. 1, Missed detection rate Vs SNR for AWGN with OOK-based receiver

Proposal 1: RAN4 to set the required SNR value as -3.3 dB for OOK-based receiver at 1% miss detection.

There is also preliminary result for the required SNR for OFDM-based receiver at 1% miss detection, which is between -7dB and -8dB [2]. Considering the potential significant difference for the required SNR value between OOK-based and OFDM-based receiver, it may be a good choice to introduce two SNR values for OOK-based and OFDM-based receiver respectively.

Proposal 2: RAN4 to introduce two different SNR values for OOK-based and OFDM-based receiver respectively.

MDR value for REFSENS test case

One issue was raised that a different MDR value for REFSENS test case could be used if considering a compromise between the performance and test complexity [1]:

MDR value for REFSENS test case Agreements: FFS the MDR for conformance testing should be aligned with the value for RF requirement.

In our view, if a different MDR value, e.g., 5%, from that for specifying REFSENS is selected, i.e., 1%, then a fixed SNR offset between these two MDR values should be specified as well, regardless of whether it is accounted for in the test tolerance. However, since this offset may vary depending on different implementations, it would be more appropriate to use the same MDR value.

Proposal 3: MDR for conformance testing should be aligned with the value for RF requirement.

# 3 Conclusion

In this contribution, we have the following proposals for Rx requirements for LP-WUS:

Proposal 1: RAN4 to set the required SNR value as -3.3 dB for OOK-based receiver at 1% miss detection.

Proposal 2: RAN4 to introduce two different SNR values for OOK-based and OFDM-based receiver respectively.

Proposal 3: MDR for conformance testing should be aligned with the value for RF requirement.

# Reference

[1] R4-2414309, “WF on LP-WUS UE RF requirements”, vivo

[2] R4-2413223, “On UE Rx requirements for the LPWUR”, Qualcomm