3GPP TSG-RAN WG4 Meeting #112bis	R4-2415134

Hefei, China, Oct 14th – 18th, 2024

Title: 	On BS RF requirements for LP-WUS

Agenda Item:	6.23.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, discussions continued on BS RF requirements for LP-WUS with some progress, and a WF was agreed which mainly identified open issues and potential options:

Issue 1-1: Manufacture declaration on LP-WUS power boosting - WF - FFS on following options: - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level. - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification. Issue 1-2: Concept of LP-WUS power boosting - WF - FFS on following options: - Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs. - Option 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs). - Option 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs). - Adopt following definition rather than introduce EPRE ratio. - Option 3: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). Issue 1-3: Whether to preclude small CBW for consideration of LP-WUS power boosting - WF - FFS on following options: - Option 1: Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting. - Option 1-1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting. - Option 1-2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded. - Option 1-3: other option does not excluded. - Option 2: No need to preclude any CBW, only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement. Issue 1-4: On applicable BS type for LP-WUS - WF - FFS whether all available BS types can be considered for LP-WUS due to the support of LP-WUS is optional and the power boosting level could be up to manufacturer declaration. - FFS whether only conductive based BS RF requirements should be defined for LP-WUS: Issue 1-5: On minimum value for LP-WUS power boosting - WF - FFS on the following options: - Option 1: If power boosting will be defined as EPRE ratio, use 3dB as minimum requirement. - Option 2: If power boosting will be defined as dynamic range, based on existing specification for OOK-1 waveform, the core requirement for the dynamic range should be limited to 3dB. - Option 3: Not to introduce minimum value for LP-WUS power boosting. Issue 1-6: Whether a cap for LP-WUS power boosting should be considered - WF - FFS on the following options: - Option 1: It should be considered as 3dB. - Option 2: It should be considered as 6dB - Option 3: Not to introduce maximum value for LP-WUS power boosting. Issue 1-8: Transmitted signal quality - WF - Option 1: Legacy transmitted signal quality requirements apply, no impact when BS transmitting LP-WUS signal. - Option 2: The OOK-based EVM should be defined for LP-WUS. FFS on how to specify this requirement with consideration of RAN1 waveform design. Issue 1-9: Multi-band operation - WF - FFS on multi-band operation.

In this contribution, we present our views on these open issues.

# 2 Discussion

Manufacture declaration on LP-WUS power boosting

The primary difference between Option 1 and Option 2 is that Option 1 implicitly treats power boosting for LP-WUS as an optional feature, while Option 2 as a mandatory feature if LP-WUS is supported. Moreover, Option 2 provides a way for an infra vendor to declare its superiority over the minimum requirements specified in specs. In order to exploit most of LP-WUS in reality, we think Option 2 is a reasonable to go.

Proposal 1: For manufacture declaration on LP-WUS power boosting, RAN4 to specify minimum power boosting level in core specs together with manufacturer declaration in the conformance test specification, i.e., Option 2, for the sake of making most of LP-WUS in reality.

Concept of LP-WUS power boosting

Our papers submitted to the previous meetings have already clearly explained the difference between power boosting and power dynamic range mathematically [2][3], where power dynamic range is calculated by the following equation where δLP is the power dynamic range and α is the EPRE ratio:

δLP=NRB∙αNLP_RB∙α+(NRB-NLP_RB)                                (1)

And from power dynamic range to EPRE ratio:

α=δLP∙(NRB-NLP_RB)NRB-δLP∙NLP_RB                                       (2)

For a certain channel bandwidth and power dynamic range values, α could be a negative value according to Equ. (2) , which is not applicable [3], e.g., for power dynamic range +9dB, and channel bandwidth 10MHz and 20MHz, α is calculated as a negative value.

Observation 1: If using power dynamic range, a negative EPRE ratio may be required for some power dynamic range and channel bandwidth, and a restriction should be clearily specified, hence using directly EPRE ratio is more convenient.

Proposal 2: RAN4 to introduce and specify power boosting represented by EPRE ratio between LP-WUS and NR REs excluding LP-WUS.

Since power dynamic range and power boosting has different meaning, it is better not to mix the use of power dynamic range and power boosting, therefore, Option 1 is and accurate definition of power boosting.

Proposal 3: RAN4 to take Option 1 as the definition of power boosting for LP-WUS.

small CBW for consideration of LP-WUS power boosting

The power degradation ∆ could be calculated as Equ. (3) and illustrated as Fig. 1 where is associated with a combination of both EPRE ratio and channel bandwidth:

∆=1+(α-1)NLP_RBNRB                                       (3)

Fig.1, power degradation Vs channel bandwidth for different EPRE ratios

From the above equation and figure, it could be observed that for a power degradation less than the same value, multiple combinations of EPRE ratio and channel bandwidth could be applied.

Observation 2: For a power degradation less than the same value, multiple combinations of EPRE ratio and channel bandwidth could be applied.

Moreover, the power degradation is a suitable common control metric for the impact of LP-WUS power boosting on existing network performance for different EPRE ratio and channel bandwidth configurations, while only channel bandwidth itself cannot achieve this goal.

Observation 3: Power degradation is a more suitable common control metric than channel bandwidth alone for the impact of LP-WUS power boosting on existing network performance for different EPRE ratio and channel bandwidth configurations.

Therefore, we propose to introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting, i.e., Option 1-1.

Proposal 4: RAN4 to introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting, i.e., Option 1-1.

Applicable BS type for LP-WUS

Considering deployment flexibility, it’s better to enable the potential benefits of applying LP-WUS in different scenarios, hence there should be no restriction on applicable BS types.

Propsoal 5: RAN4 not to set any restriction on applicable BS types for LP-WUS, i.e., support all BS types.

And in this release, RAN4 can only define conductive requirements for BS for LP-WUS.

Proposal 6: RAN4 to only define conductived based BS RF requirements for LP-WUS in Release-19.

Minimum value for LP-WUS power boosting

Since LP-WUS is an optional feature, and to be able to declare its support, BS shall ensure a minimum level of LP-WUS power boosting should be specified. A minimum value of 3 dB can be considered as a starting point.

Proposal 7: RAN4 to introduce 3 dB as the minimum requirements for LP-WUS power boosting.

Maximum value for LP-WUS power boosting

Regarding the maximum value for LP-WUS power boosting, it is highly related to implementation and BS types, and since anyway BS needs to declare this support, the maximum level for LP-WUS power boosting can be declared.

Proposal 8: RAN4 not to introduce maximum requirements for LP-WUS power boosting.

Transmitted signal quality

Obviously, RAN4 can wait until RAN1 has completed the LP-WUS signal design, and then further discuss how to specify requirements for transmitted signal quality for LP-WUS.

Proposal 9: RAN4 to wait until RAN1 has completed LP-WUS signal design for specifying requirements for transmitted signal quality.

Multi-band operation

Considering there are still many open issues on the single band operation, it’s better to wait until the single band operation is finalized.

Proposal 10: Postpone the discussion on multi-band operation for LP-WUS until the single band operation is finalized.

# 3 Conclusion

In this contribution, we have the following observations and proposals for BS RF requirements for LP-WUS:

Observation 1: If using power dynamic range, a negative EPRE ratio may be required for some power dynamic range and channel bandwidth, and a restriction should be clearily specified, hence using directly EPRE ratio is more convenient.

Observation 2: For a power degradation less than the same value, multiple combinations of EPRE ratio and channel bandwidth could be applied.

Observation 3: Power degradation is a more suitable common control metric than channel bandwidth alone for the impact of LP-WUS power boosting on existing network performance for different EPRE ratio and channel bandwidth configurations.

Proposal 1: For manufacture declaration on LP-WUS power boosting, RAN4 to specify minimum power boosting level in core specs together with manufacturer declaration in the conformance test specification, i.e., Option 2, for the sake of making most of LP-WUS in reality.

Proposal 2: RAN4 to introduce and specify power boosting represented by EPRE ratio between LP-WUS and NR REs excluding LP-WUS.

Proposal 3: RAN4 to take Option 1 as the definition of power boosting for LP-WUS.

Proposal 4: RAN4 to introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting, i.e., Option 1-1.

Propsoal 5: RAN4 not to set any restriction on applicable BS types for LP-WUS, i.e., support all BS types.

Proposal 6: RAN4 to only define conductived based BS RF requirements for LP-WUS in Release-19.

Proposal 7: RAN4 to introduce 3 dB as the minimum requirements for LP-WUS power boosting.

Proposal 8: RAN4 not to introduce maximum requirements for LP-WUS power boosting.

Proposal 9: RAN4 to wait until RAN1 has completed LP-WUS signal design for specifying requirements for transmitted signal quality.

Proposal 10: Postpone the discussion on multi-band operation for LP-WUS until the single band operation is finalized.

# Reference

[1] R4-2413512, “Way Forward for [112][309] NR_LPWUS”, Huawei

[2] R4-2407547, “On BS RF requirements for LP-WUS/LP-WUR”, CATT

[3] R4-2411094, “Further discussion on BS RF requirements for LP-WUS”, CATT