3GPP TSG-RAN WG4 Meeting #112bis	R4-2415140

Hefei, China, Oct 14th – 18th, 2024

Title: 	Further discussion on notes usage in UE RF specifications

Agenda Item:	8.1.1.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, notes usage were discussed as well for possible specification improvement, and several approaches are proposed for further elaboration [1]:

## Sub-topic 1-3: Notes usage Proposals: - Option 1: Proposal 6 in R4-2411237 (HW) RAN4 should discuss necessity of more practical drafting rule for NOTEs handling in a table for the future specifications. - Option 2: Proposal 2 in R4-2413320: Do not use notes in tables for requirements that apply every cell/line in the table. Use text above the table instead - Option 3: Proposal 6a in R4-2412482: Use “NOTE x” (with x as a number) for specific items in a table (like with superscript number) and use “REMARK” (numbering not necessary) for note general to the table. - Option 4: Proposal 6b in R4-2412482:: Use “NOTE x” (with x as a number) for specific items in a table (like with superscript number) and move general notes to the main body of the text with the table to which it applies to clearly mentioned. Agreement: - Use Option 1, Option 2 and Option 4 as baseline, and further work on how to implement them. - Companies are encouraged to provide draft examples for option 1, 2, and 4 in the next meeting. - Discuss the changes for the existing tables case by case.

In this contribution, we discuss the potential aligned way of handling notes and provide examples respectively.

# 2 Discussion

Note indication

In current specifications, a note “x” is indicated either via explicit notation like “ (Note x)”, or a superscript “text x”. For example,

- Explicit notation “Note x”: in Table 5.4.3.3-1 which specifies applicable SS raster entries per operating band for above 3 MHz channel bandwidth, explicit notations are used.

- Superscript: in Table 5.2-1 for NR operating bands in FR1, superscripts are widely used.

For consistency consideration, it is better to stick to the same style for note indication, and explicit note indication instead of superscripting if following drafting rules, hence RAN4 may consider to modify all note indication via superscripts to explicit note notation. For example, for Table 5.2-1, it could be revised as the following table (partially shown):

| n96 (Note 13, 14) | 5925 MHz – 7125 MHz |  | 5925 MHz – 7125 MHz | TDD |
| --- | --- | --- | --- | --- |
| n9715 (Note 15) | 2300 MHz – 2400 MHz |  | N/A | SUL |
| n98 (Note 15) | 1880 MHz – 1920 MHz |  | N/A | SUL |
| n99 (Note 22) | 1626.5 MHz – 1660.5 MHz |  | N/A | SUL |
| n100 (Note 21) | 874.4 MHz – 880 MHz |  | 919.4 MHz – 925 MHz | FDD |
| n101 (Note 21) | 1900 MHz – 1910 MHz |  | 1900 MHz – 1910 MHz | TDD |
| n102 (Note 13, 14) | 5925 MHz – 6425 MHz |  | 5925 MHz – 6425 MHz | TDD |
| n104 (Note 17, 18) | 6425 MHz – 7125 MHz |  | 6425 MHz – 7125 MHz | TDD |

Proposal 1: RAN4 to modify all note indication via superscripts to explicit note notation.

Common/general note in one table

In current UE RF specs, it is also commonly seen that a table may have a general note which is applicable to each row in the whole table. In this case, this general note should be moved to the main body texts just before the table.

Proposal 2: General or common notes in a table which is applicable to every row in the table should be moved to the main body just before the table.

One example is illustrated below for Table 5.2C-1 for SUL band combinations:

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n802 | n1, n80 |
| SUL_n1-n812 | n1, n81 |
| SUL_n1-n892 | n1, n89 |
| SUL_n3-n842 | n3, n84 |
| SUL_n5-n842 | n5, n84 |
| SUL_n8-n842 | n8, n84 |
| SUL_n24-n992 | n24, n99 |
| SUL_n41-n802 | n41, n80 |
| SUL_n41-n812 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n952 | n41, n95 |
| SUL_n41-n972 | n41, n97 |
| SUL_n41-n982 | n41, n98 |
| SUL_n41-n992 | n41, n99 |
| SUL_n48-n992 | n48, n99 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n77-n992 | n77, n99 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n78-n892 | n78, n89 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n842 | n79, n84 |
| SUL_n79-n952 | n79, n95 |
| SUL_n79-n972 | n79, n97 |
| SUL_n79-n982 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

In this example, NOTE 1 is a general note hence it should be moved to the main body texts. Furthermore, NOTE 2 is seen in every row in this table, thus it also belongs to a common note, then both notes can be moved to the main body before the table:

-------------------------------------------- Example starts -----------------------------------------------------------------

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us.

For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n80 | n1, n80 |
| SUL_n1-n81 | n1, n81 |
| SUL_n1-n89 | n1, n89 |
| SUL_n3-n84 | n3, n84 |
| SUL_n5-n84 | n5, n84 |
| SUL_n8-n84 | n8, n84 |
| SUL_n24-n99 | n24, n99 |
| SUL_n41-n80 | n41, n80 |
| SUL_n41-n81 | n41, n81 |
| SUL_n41-n83 | n41, n83 |
| SUL_n41-n95 | n41, n95 |
| SUL_n41-n97 | n41, n97 |
| SUL_n41-n98 | n41, n98 |
| SUL_n41-n99 | n41, n99 |
| SUL_n48-n99 | n48, n99 |
| SUL_n77-n80 | n77, n80 |
| SUL_n77-n84 | n77, n84 |
| SUL_n77-n99 | n77, n99 |
| SUL_n78-n80 | n78, n80 |
| SUL_n78-n81 | n78, n81 |
| SUL_n78-n82 | n78, n82 |
| SUL_n78-n83 | n78, n83 |
| SUL_n78-n84 | n78, n84 |
| SUL_n78-n86 | n78, n86 |
| SUL_n78-n89 | n78, n89 |
| SUL_n79-n80 | n79, n80 |
| SUL_n79-n81 | n79, n81 |
| SUL_n79-n83 | n79, n83 |
| SUL_n79-n84 | n79, n84 |
| SUL_n79-n95 | n79, n95 |
| SUL_n79-n97 | n79, n97 |
| SUL_n79-n98 | n79, n98 |

-------------- Example ends ------------------

# 3 Conclusion

In this contribution, we have the following proposals, and we also illustrate these proposals with some specific examples:

Proposal 1: RAN4 to modify all note indication via superscripts to explicit note notation.

Proposal 2: General or common notes in a table which is applicable to every row in the table should be moved to the main body just before the table.

# Reference

[1] R4-2414430, “WF on UE RF Specification quality improvement”, Qualcomm