3GPP TSG-RAN WG4 Meeting #112bis	R4-2415141

Hefei, China, Oct 14th – 18th, 2024

Title: 	On potential table modifications for UE RF specs

Agenda Item:	8.1.1.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, discussions on UE RF specs improvement continued and a WF was agreed [1]. In addition to identified wording change, other aspects such as table modifications are also open.

In this contribution, we continue to discuss two types of table modification: EN-DC configuration tables and NR CA ΔTIB,c and ΔRIB,c tables.

# 2 Discussion

## 2.1 Modifying configuration tables for EN-DC

For intra-band EN-DC configuration tables, e.g., the below table indicates channel bandwidths for E-UTRA and NR carrier respectively:

The table inherits LTE’s style to indicate component carriers in order of increasing carrier frequency, hence, to allow all permutation of LTE and NR carriers, the table has two symmetric table cells for LTE carrier as illustrated in red circles and blue circles as two examples. The redundant symmetric table cells actually invalidate the need of order of increasing carrier frequency, therefore, we propose to remove the columns/rows associated with the symmetric table cell and clarify without consideration of order of increasing carrier frequency. The below table is the one under this proposal:

Table – 1: Simplification of configuration tables of EN-DC

| E-UTRA – NR configuration /Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers without considering order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) |  |  |  |
| DC_(n)3AA | DC_(n)3AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
| DC_(n)7AA | DC_(n)7AA4 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| DC_(n)41AA |  | DC_(n)41AA |  | 20 |  | 40, 60, 80,100 |  |  |  | 120 |  | 0 |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |

Proposal 1: For configuration EN-DC tables, do not consider order of increasing carrier frequency for LTE and NR carriers and remove redundant symmetric table cells for each BCS as illustrated in Table - 1.

## 2.2 Template-based approach for ΔTIB,c and ΔRIB,c tables for NR CA

In our view, long tables with many pages in the current RF specs brings bad readibility, and the template-based approach may be helpful for improving tables, e.g., for ΔTIB,c and ΔRIB,c tables for NR CA. The template-based approach can be applied perfectly for simplifying ΔTIB,c and ΔRIB,c tables resulting in much less pages and better readability without altering any core requirements at the same time.

Take two-band tables as one example for ΔTIB,c :

And we can introduce a template: {a, b} representing ΔTIB,c =a for band nX and ΔTIB,c = b for band nY for CA band combination CA_nX-nY, then the table can have two or more band combinations for each row as shown in Table – 2 below:

Table – 2 Simplification of two-band ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |
| --- | --- | --- | --- | --- | --- |
|  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |
| CA_n1-n3 | {0.3, 0.3} | CA_n1-n5 | {0.3, 0.3} | CA_n1-n7 | {0.5, 0.6} |
| CA_n1-n8 | {0.3, 0.3} |  |  |  |  |

And the lexicographic order is still kept row by row. Considering there are many pages for such tables, the improvement is not trivial.

Proposal 2: RAN4 to introduce the template-based approach for simplifying ΔTIB,c and ΔRIB,c tables, which keeps both lexicographic order of band combinations and readability as shown in Table – 2.

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |
| --- | --- | --- | --- | --- | --- |
|  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |
| CA_n1-n3 | {0.3, 0.3} | CA_n1-n5 | {0.3, 0.3} | CA_n1-n7 | {0.5, 0.6} |
| CA_n1-n8 | {0.3, 0.3} |  |  |  |  |

# 3 Conclusion

In this contribution, we have the following proposals for potential table modifications for UE RF specs:

Proposal 1: For configuration EN-DC tables, do not consider order of increasing carrier frequency for LTE and NR carriers and remove redundant symmetric table cells for each BCS as illustrated in Table - 1.

Table – 1: Simplification of configuration tables of EN-DC

| E-UTRA – NR configuration /Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers without considering order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) |  |  |  |
| DC_(n)3AA | DC_(n)3AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
| DC_(n)7AA | DC_(n)7AA4 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| DC_(n)41AA |  | DC_(n)41AA |  | 20 |  | 40, 60, 80,100 |  |  |  | 120 |  | 0 |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |

Proposal 2: RAN4 to introduce the template-based approach for simplifying ΔTIB,c and ΔRIB,c tables, which keeps both lexicographic order of band combinations and readability as shown in Table – 2.

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |
| --- | --- | --- | --- | --- | --- |
|  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |
| CA_n1-n3 | {0.3, 0.3} | CA_n1-n5 | {0.3, 0.3} | CA_n1-n7 | {0.5, 0.6} |
| CA_n1-n8 | {0.3, 0.3} |  |  |  |  |

# Reference

[1] R4-2414430, “WF on UE RF Specification quality improvement”, Qualcomm