3GPP TSG-RAN WG4 Meeting #112bis	R4-2415151

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.16.2

Source:	MediaTek Inc.

Title:	Discussion on UE requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

A WF for UE 8Rx MMSE-IRC receiver requirements was agreed in RAN4#112 meeting [1]. In this contribution, we provide our views on outstanding issues.

- Discussion

## 2.1 General

Test parameters for different devices with 8Rx - Not to define different set of requirements for FWA, vehicular and industrial devices PDSCH mapping type - Candidate options: - Option 1: Include both PDSCH type A and type B requirements. - Option 2: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only

RAN4 agreed not to differentiate test parameters for FWA, vehicular and industrial devices. Therefore, we suggest not to consider PDSCH mapping type B which only targets for industrial devices. Also, PDSCH mapping type B is a mandatory feature with UE capability signalling. We propose considering only PDSCH mapping type A, which is a mandatory feature without UE capability signalling.

Proposal 1: Define requirements for 8Rx MMSE-IRC receiver with PDSCD mapping type A only.

## 2.1 PDSCH requirements with inter cell interference

Interference modelling and power levels - Candidate options: - Option 1: Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4 - Option 1A: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 1B: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 2: RAN4 to discuss if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell Network Type and SCS/CBW - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) - Other options are not precluded

For the interference modelling and power levels, we prefer following the configurations used in Rel-17. Also, we are OK to consider only HomNet scenario for requirements as the receiver algorithm will not change with different interference power level. As for SCS and CBW, we support to reuse the setting of MMSE-IRC receiver requirements for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U (S=6D+4G+4U).

Proposal 2: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Proposal 3: Consider 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U (S=6D+4G+4U) to define requirements.

Propagation condition - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) - Option 2: Consider TDLA30-10 as the starting point Rank - Candidate options: - Option 1: Consider higher rank for 8Rx - Option 1A: RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value - Option 1B: Rank 2 and Rank 4 as the starting point - Option 1C: Rank 2 and Rank 4 and Rank 8 - Option 1D: Rank 4 - Option 1E: Consider 2 layers, 4 layers and 6 layers - Option 2: Same as Rel-17 2/4Rx, i.e., Rank 1 MCS - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., MCS 13 - Option 2: MCS 16 - Option 3: MCS 19 for rank 2, MCS 17 for rank 4 and 8 - Option 4: Define different parameters for FWA, Vehicular and Industrial deployments respectively Antenna and MIMO correlation - Candidate options: - Option 1: 2T8R ULA low - Option 2: 4T8R - Option 3: - 2T8R ULA medium B - 4T8R ULA low - 8T8R ULA low

As RAN4 agreed not to define different set of requirements for CPE/FWA/vehicle/industrial devices for 8Rx IRC receiver, we propose to use 2T8R ULA low as MIMO correlation matrix and antenna configuration.

Proposal 4: Use “2T8R ULA low” as MIMO correlation matrix and antenna configuration to define PDSCH requirements for 8Rx IRC receiver considering CPE/FWA/vehicle/industrial devices.

We provided initial simulation results in Table 1 based on the following assumptions:

- Test parameters of Table 5.2.2.1.15-2 in TS38.101-4 (INRs 7.77 dB and 2.29 dB for two interfering cells)

- MIMO correlation matrix and antenna configuration: 2T8R ULA low

- Rank: rank 1

- propagation condition: TDLC300-100 and TDLA30-10

Table 1. Initial evaluation results for 8Rx IRC receiver in the presence of inter-cell interference

| Scenario | propagation condition | MCS | SNR for 70% of Max T-put (MMSE-IRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC simplified receiver) | SNR difference between MMSE-IRC baseline and simplified receiver | Corresponding SINR for SNR (MMSE-IRC baseline receiver) |
| --- | --- | --- | --- | --- | --- | --- |
| Two interfering cells (INRs: 7.77 dB and 2.29 dB) | TDLA30-10 | MCS 13 | 2.5 | 3.6 | 1.1 | -6.7 |
|  |  | MCS 16 | 5.1 | 6.2 | 1.1 | -4.1 |
|  | TDLC300-100 | MCS 13 | 3.4 | 4.8 | 1.4 | -5.9 |
|  |  | MCS 16 | 6.2 | 7.5 | 1.3 | -3.1 |

From simulation results, it shows that the SINRs for MCS 13 are quite low for both TDLA30-10 and TDLC300-100. Therefore, we propose to consider MCS 16 to define requirements, where the corresponding SINR for SNR achieving 70% maximum throughput is around -3 dB for TDLC and -4 dB for TDLA. Besides, the gaps between 8Rx MMSE-IRC baseline receiver and 8Rx MMSE-IRC simplified receiver are over 1dB. Therefore, we suggest considering two sets of requirements as RAN4 did in Rel-18 8Rx WI.

Observation 1: With Rank 1 and MCS16, the corresponding SINR for SNR achieving 70% maximum throughput is around -3 dB for TDLC and -4 dB for TDLA, which are all above -6dB.

Proposal 5: Consider rank 1, MCS16 and TDLA30-10 as a starting point to define 8Rx MMSE-IRC requirements with inter-cell interference.

Observation 2: With Rank 1 and MCS16, the gaps between 8Rx MMSE-IRC baseline receiver and simplified receiver are over 1dB

Proposal 6: Consider two sets of PDSCH requirements for 8Rx MMSE-IRC receiver with inter-cell interference.

## 2.2 Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm - Candidate options: - Option 1: RAN4 to consider whether to introduce two CQI calculation assumptions to match two 8Rx receiver assumptions. - Baseline CQI calculation: UE perform CQI calculation with 8Rx joint processing - Simplified CQI calculation: UE perform CQI calculation with separate 4Rx processing - Other options not precluded Antenna configuration and MIMO correlation for the serving cell - Candidate options: - Option 1: 2T8R ULA Low - Option 2: Consider 4T8R Rank - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., rank 1 - Option 2: Rank 2 - Option 3: Rank 4

As the test metric may base on the throughput ratio of interference model and AWGN model, we suggest RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for two set of requirements. Otherwise, RAN4 can define two sets of requirements for different type of receiver. Regarding the antenna configuration and rank, we propose to use 2T8R ULA low and rank 1.

Proposal 7: RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for one requirements. Otherwise, RAN4 can define two sets of requirements for different type of receiver.

Proposal 8: Use 2T8R ULA low as the antenna configuration and MIMO correlation for the serving cell.

Proposal 9: Reuse the same configuration for Rel-17 2/4Rx, i.e., rank 1

## 2.3 PDSCH requirements with intra-cell inter-user interference

Network Type and SCS/CBW - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U - Other options are not precluded Interference modelling - Candidate options on MU-MIMO Beamforming Model: - Option 1: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 - Option 2: Include in a new clause of Annex B.6 of TS 38.101-4 - Candidate options on co-UE number: - Option 1: Same as Rel-17 2/4Rx requirements, i.e., 1 target UE + 1 co-UE - Option 2: Consider 1 target UE and more than 1 co-UEs Propagation condition - Candidate options: - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2 - Option 2: Consider TDLA30-10 Precoding method for the co-scheduled UE - Candidate options: - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests. - Option 2: Orthogonal precoding with the target UE, i.e., any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation - Candidate options: - Option 1: 2T8R ULA low - Option 2: Rank 4+2 with 8T8R - Option 3: - Rank 2+2 for 2T8R - Rank 4+4 for 4T8R - Rank 8+8 for 8T8R when 1 paired UE scheduled - Option 4: - Rank 1+1 for 2T8R - Rank 2+2 for 4T8R - Rank 2+6 or rank 6+6 for 8T8R - Option 5: - Rank 2+2+2 for 8T8R - Rank 4+4 for 4T8R - Option 6: - Rank 2+2 - Rank 4+4 MCS - Candidate options for the target UE: - Option 1: Same as Rel-17 2/4Rx requirements for MMSE-IRC, i.e., MCS 13 - Option 2: - MCS 19 Table 1 for Rank 2 - MCS 17 Table 1 for Rank 4 - MCS 17 Table 1 for Rank 8 - Option 3: Consider MCS 17 and MCS 13 - Option 4: MCS 19 for rank 2, MCS 13 for rank 4 Other parameters - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC - Other options are not precluded

Regarding SCS/CBW, we support to use the same setting as Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U. For MU-MIMO Beamforming model, we propose to reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4. Regarding the number of co-scheduling UE, we propose to use only one co-scheduled UE. The reason is that the receiver algorithm does not change with the number of co-scheduled UE as target UE does not know how number of co-scheduled UE.

Proposal 10: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U.

Proposal 11: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4.

Observation 3: The receiver algorithm for MMSE-IRC receiver does not change with the number of co-scheduled UE as target UE does not know how number of co-scheduled UE.

Proposal 12: Consider only one co-scheduled UE to define requirements for MU-MIMO.

We provide initial evaluation results in Table 4 for 8Rx PDSCH demodulation performance with inter-user intra-cell interference based on the following assumptions:

- Target UE

  - Antenna configuration and propagation condition: 4T8R ULA low and TDLA30-10

  - DMRS port: ports 1000 and 1001

  - MCS: MCS13

  - Precoder selection: Random precoder

- Co-scheduled UE

  - DMRS port: ports 1002 and 1003

  - MCS: random 16 QAM

  - Precoder selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Table 4. Initial evaluation results for 8Rx PDSCH demodulation performance with inter-user intra-cell interference

| SNR for 70% of Max T-put (MMSE-MRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC simplified receiver) | MMSE-IRC gain | SNR difference between MMSE-IRC baseline and simplified receiver |
| --- | --- | --- | --- | --- |
| Inf | 6.9 | 10.7 | Inf | 3.8 |

From the simulation results, it shows that MMSE-MRC receiver cannot achieve 70% of the maximum throughput. Therefore, the test setup/assumptions above are suitable to verify whether the UE implement MMSE-IRC receiver.

Proposal 13: Use the following settings to define requirements for MU-MIMO.

- Target UE

  - Antenna configuration and propagation condition: 4T8R ULA low and TDLA30-10

  - DMRS port: ports 1000 and 1001

  - MCS: MCS13

  - Precoder selection: Random precoder

- Co-scheduled UE

  - DMRS port: ports 1002 and 1003

  - MCS: random 16 QAM

  - Precoder selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Besides, from our initial evaluations, the gap between 8Rx MMSE-IRC baseline receiver and 8Rx MMSE-IRC simplified receiver is 3.8 dB. Therefore, we suggest considering two sets of requirements as what RAN4 did in Rel-18 8Rx WI.

Proposal 14: Consider two sets of requirements for 8Rx MMSE-IRC receiver for MU-MIMO.

In MU-MIMO scenario, the data of target UE and co-scheduled UE are multiplexed in spatial domain and transmitted in the same time-frequency domain. We propose to use same signal power and SNR assumptions as Rel-17 Rel-17 2/4Rx.

Proposal 15: Consider the following signal power and SNR assumptions to define requirements for scenario of MU-MIMO

- Signal power assumptions: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- SNR assumptions: SNR = (STargetUE+ SInterfUE)/N

## 2.4 Applicability rule and release independent

Test applicability - Candidate options: - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

We support new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. However, if RAN4 agreed to define two set of requirements for baseline and simplified, we need to introduce new applicability rules as what we have in Table 5.1.1.3-1 of TS38.101-4. Otherwise, we think there is no necessary to introduce new applicability rules. Also, we are willing to introduce applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

Table 5.1.1.3-1: Requirements applicability for optional UE features UE feature/capability [14]; Test type; Test list; Applicability notes. Baseline SU-MIMO 8Rx receiver; FR1 FDD; PDSCH; Clause 5.2.4.1.1(Tests 1-1,3-1). FR1 TDD; PDSCH; Clause 5.2.4.2.1(Tests 1-1,3-1). Simplified SU-MIMO 8Rx receiver; FR1 FDD; PDSCH; Clause 5.2.4.1.1(Tests 2-1,4-1). FR1 TDD; PDSCH; Clause 5.2.4.2.1(Tests 2-1,4-1). PBCH; Clause 5.4.2.1 (Test 3-1) Clause 5.4.3.1 (Test 3-1).

Proposal 16: We support new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. However, if RAN4 agreed to define two set of requirements for baseline and simplified, we need to introduce new applicability rules as what we have in Table 5.1.1.3-1 of TS38.101-4.

Proposal 17: Support defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

Release independent - Candidate options: - Option 1: The new requirements for 8Rx should be release independent from Rel-18 Other options are not precluded

As there are some many options/settings for requirements are under discussion. Based on the workplan [2], we have 5 meeting including this October meeting. We propose RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

Proposal 18: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

# Conclusion

In this contribution, we provide our views on UE demodulation requirements with 8Rx MMSE-IRC receiver. Our observations and proposals are captured below:

Proposal 1: Define requirements for 8Rx MMSE-IRC receiver with PDSCD mapping type A only.

Proposal 2: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Proposal 3: Consider 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U (S=6D+4G+4U) to define requirements.

Proposal 4: Use “2T8R ULA low” as MIMO correlation matrix and antenna configuration to define PDSCH requirements for 8Rx IRC receiver considering CPE/FWA/vehicle/industrial devices.

Observation 1: With Rank 1 and MCS16, the corresponding SINR for SNR achieving 70% maximum throughput is around -3 dB for TDLC and -4 dB for TDLA, which are all above -6dB.

Proposal 5: Consider rank 1, MCS16 and TDLA30-10 as a starting point to define 8Rx MMSE-IRC requirements with inter-cell interference.

Observation 2: With Rank 1 and MCS16, the gaps between 8Rx MMSE-IRC baseline receiver and simplified receiver are over 1dB

Proposal 6: Consider two sets of PDSCH requirements for 8Rx MMSE-IRC receiver with inter-cell interference.

Proposal 7: RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for one requirements. Otherwise, RAN4 can define two sets of requirements for different type of receiver.

Proposal 8: Use 2T8R ULA low as the antenna configuration and MIMO correlation for the serving cell.

Proposal 9: Reuse the same configuration for Rel-17 2/4Rx, i.e., rank 1

Proposal 10: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U.

Proposal 11: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4.

Observation 3: The receiver algorithm for MMSE-IRC receiver does not change with the number of co-scheduled UE as target UE does not know how number of co-scheduled UE.

Proposal 12: Consider only one co-scheduled UE to define requirements for MU-MIMO.

Proposal 13: Use the following settings to define requirements for MU-MIMO.

- Target UE

  - Antenna configuration and propagation condition: 4T8R ULA low and TDLA30-10

  - DMRS port: ports 1000 and 1001

  - MCS: MCS13

  - Precoder selection: Random precoder

- Co-scheduled UE

  - DMRS port: ports 1002 and 1003

  - MCS: random 16 QAM

  - Precoder selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Proposal 14: Consider two sets of requirements for 8Rx MMSE-IRC receiver for MU-MIMO.

Proposal 15: Consider the following signal power and SNR assumptions to define requirements for scenario of MU-MIMO

- Signal power assumptions: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- SNR assumptions: SNR = (STargetUE+ SInterfUE)/N

Proposal 16: We support new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. However, if RAN4 agreed to define two set of requirements for baseline and simplified, we need to introduce new applicability rules as what we have in Table 5.1.1.3-1 of TS38.101-4.

Proposal 17: Support defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed.

Proposal 18: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113.

# Reference

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telecom

- R4-2413569, Work plan for NR demodulation performance: Phase 5, China Telecom