3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415158

# Hefei , CN, Oct 14 – 18, 2024

Title: 	Discussion on other RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	6.26.4.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in RAN#102 [1], and the WID have been revised in RAN#105.The latest objectives of this WI other than (e)RedCap UE related are as following [2]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

This document will discuss other RRM requirements about NR NTN phase 3, and give our understanding and proposals.

- Discussion

Topic #1: Downlink coverage enhancements

- Sub-topic 1-1 SSB periodicity enhancement

RRM impact of SSB periodicity enhancement

The Way Forward on RRM impact of SSB periodicity enhancement in the last meeting is copied as follows [3]:

<Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, Apple, Xiaomi, Samsung, HW, QC): - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement. - Proposal 1a (Samsung): - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed. - Proposal 1b (Ericsson): RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity. - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

This issue is related to RAN1’s conclusions. In RAN1 #118 meeting, the agreements on NTN DL coverage enhancements at system level and link level are copied as follows:

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

From RRM requirements perspective, the legacy requirements are mainly based on DL RS including SSB or CSI-RS, almost RRM requirements are related to SSB periodicity.

According to the above, RAN1 agreed that extending the periodicity of the half frames with SSB assumed by UE during initial access, and RAN1 is still discussing the default value[s] with extended periodicity. Therefore, at least cell re-selection requirements in idle/inactive mode are related to the SSB periodicity extension assumed by UE during initial access. However, in RAN4 spec, there is no explicit SSB periodicity in initial access related requirements, and only the SMTC periodicity is used to describe it, so it seems that there is no RRM spec impact.

Observation 1: RAN1 agreed that extending the periodicity of the half frames with SSB assumed by UE during initial access, and RAN1 is still discussing the default value[s] with extended periodicity.

Observation 2: In RAN4 spec, there is no explicit SSB periodicity in initial access related requirements, and only the SMTC periodicity is used to describe it.

Proposal 1: Due to SSB periodicity extension assumed by UE during initial access, there may be no explicit RRM spec impact on initial access related requirements.

In addition, if the UE transitions between any two states when being configured with SSB periodicity or extending SSB periodicity, RAN4 may need to discuss whether and how to define such a transition requirement from one state to other state that UE need to meet.

Proposal 2: RAN4 need to discuss whether and how to define the transition requirement for the following case:

- The UE transitions between any two states when being configured with SSB periodicity or extending SSB periodicity.

- Sub-topic 1-2 Other RRM impacts

Beam switching related issues

In the last meeting, some proposals of beam switching are copied as below [3]:

<Way Forward> : Further discuss the following proposal - Proposal 1 (Ericsson): RAN4 to check if beam steering latency other than 0 may impact RRM requirements. - Proposal 2 (Ericsson): RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements. - Proposal 3 (Ericsson): If applicable, the requirements for quasi-earth_fixed cell and earth_ moving cell both shall be studied with respect to the beam hopping scheme.

For proposal 1, we think that proponent should clarify what beam steering latency is. If it is additional delay caused by setting the phase coefficient for different antenna elements, it may be defined from the SAN and we consider it is the delay of antenna switching for beam hopping. In our view, so the delay of antenna switching is only in the nanosecond range, so it's short enough to be ignored.

Proposal 3: The beam steering latency should be further clarified by proponent.

Proposal 4: The delay of antenna switching for beam hopping is short enough and can be ignored.

For proposal 2, we think that proponent also should clarify what timing offset/error (between beam switching timing and DL/UL framework) is. If the related issue is caused by the synchronization error between the gNB/gateway and satellite, since only FDD is considered for NTN, RAN4 will not consider it in NTN network.

Proposal 5: The timing offset/error (between beam switching timing and DL/UL framework) should be further clarified by proponent.

Proposal 6: If the timing offset/error (between beam switching timing and DL/UL framework) is caused by the synchronization error between the gNB/gateway and satellite, RAN4 will not consider it in NTN network (FDD).

Topic #2: Uplink Capacity/Cell Throughput Enhancement

The Way Forward on RRM impact for uplink capacity/cell throughput enhancement in the last meeting is copied as follows [3]:

#### Issue 3-1-1: RRM impact for uplink Capacity/Cell Throughput Enhancement < Way Forward > - No need for RAN4 to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

According to the current progress in other groups, we don’t identify some RRM related impacts, considering that the WI in this topic only captured the possible RF related impacts, we support that there is no RRM impact for uplink capacity/cell throughput enhancement.

Proposal 7: There is no RRM impact for uplink capacity/cell throughput enhancement.

Topic #3: Regenerative payload

- Sub-topic 3-1 Timing requirements

RAN2 LS

In RAN 2 LS, Question on common TA in a regenerative payload scenario was raised to RAN1 and RAN4, which is copied as follow [5]:

Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

In our view, in a regenerative payload scenario, it is reasonable to sticking to 0 as the minimum possible value for TA-Common. Considering that gNB will be located at the satellite, there is no need to consider the TA compensation for feeder link, so the NTA,adjcommon in clause 7.1C.2 in TS38.133 will be considered as 0. If the NTA,adjcommon is set to negative value, it don’t correspond to its real physical meaning.

Observation 3: There is no need to consider the TA compensation for feeder link, so the NTA,adjcommon in clause 7.1C.2 in TS38.133 will be considered as 0.

Proposal 8: Reply RAN2 that sticking to 0 as the minimum possible value for TA-Common is reasonable in regenerative payload scenario.

Secondly, even there may be some accuracy errors on the UE side when performing UL transmission, RAN4 still default that the UE needs to meet the UE UL transmit timing Te requirements defined in clause 7.1C.2 in TS 38.133 for UL initial transmission [4]. From RAN4’s perspective, Te requirements already took into account the worst case, and all of possible errors have been considered, which included the following errors [6]:

  - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs)

  - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors

  - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error

  - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error

  - Tr: TAC resolution error (from TS38.213)

  - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133)

  - Tf: an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm

  - Tm: a margin needed at gNB receiver to accommodate any additional impairments if needed.

  - If a non-zero value is assumed in the proposal for Tm, the source of the impairments shall be provided too.

According to the above principle, sufficient margin has been taken into account when specifying the Te requirements in RAN4.

Observation 4: In TS 38.133, Te requirements already took into account the worst case, and all of possible errors have been considered, enough margin has been taken into account when specifying the Te requirements in RAN4.

Proposal 9: Reply RAN2 that RAN4 default that the UE needs to meet the UE UL transmit timing Te requirements defined in clause 7.1C.2 in TS38.133 for UL initial transmission in any case.

Proposal 10: Reply RAN2 that RAN4 has already considered all of possible errors and enough margins when specifying the Te requirements in RAN4.

In addition, the values of Te_NTN are also almost smaller than half of CP length for the given SCS of UL channel/signal, except in the case of a 120 kHz SCS of uplink signals in FR2 for mobile VSAT is served by GSO. Therefore, it also means that the current Te requirements make it possible for the network side to demodulate the uplink information of different UEs through implementation in most cases, and there is no need to consider any errors when setting value for NTA,adjcommon.

Therefore, it is reasonable to only following its real physical meaning and set NTA,adjcommon as 0.

Observation 5: The values of Te_NTN are also almost smaller than half of CP length for the given SCS of UL channel/signal, except in the case of a 120 kHz SCS of uplink signals in FR2 for mobile VSAT is served by GSO.

Proposal 11: Reply RAN2 that the values of Te_NTN are also almost smaller than half of CP length for the given SCS of UL channel/signal and the current Te requirements make it possible for the network side to demodulate the uplink information of different UEs through implementation in most cases.

The RRM impact on RAN4 spec

In the last meeting, RAN4 also discussed the RRM impact of NTA,adjcommon for regenerative mode of NTN and the Way Forward is copied as follows [3]:

#### Issue 5-2-1: Timing requirements for regenerative payload <Way Forward> - The common understanding is that NTA,adjcommon is configured by network. - Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

Considering that RAN2 has sent the LS to RAN1 and RAN4 about the related issue [5], we believe that RAN4 just reply the LS from RAN2 about setting the value of NTA,adjcommon. Since the parameter was defined in RAN1 spec and described in RAN2 spec, so we think that there is no RRM impact on NTA,adjcommon in timing requirements in TS 38.133 for regenerative mode of NTN and the impact on other spec can be up to RAN1/2.

Proposal 12: There is no RRM impact on NTA,adjcommon in timing requirements in TS 38.133 for regenerative mode of NTN.

- RAN4 just reply the LS from RAN2 about setting the value of NTA,adjcommon.

- The impact on other spec can be up to RAN1/2.

- Sub-topic 3-2 Mobility requirements for regenerative payload

Satellite switching with resync

In the RAN #105 meeting, whether to support satellite switching with resync for regenerative payload was discussed. However, the related WF in RAN meeting was not pursued. Therefore, we believe that RAN4 doesn’t need to define the requirements of satellite switching with resync for regenerative payload, so the current requirements of satellite switching without PCI change is not applied for regenerative mode of NTN.

Proposal 13: RAN4 doesn’t need to define the requirements of satellite switching with resync for regenerative payload.

- The current requirements of satellite switching without PCI change is not applied for regenerative mode of NTN.

- Conclusion

This document discussed the issues for other RRM requirements and the following observations and proposals are presented:

Observation 1: RAN1 agreed that extending the periodicity of the half frames with SSB assumed by UE during initial access, and RAN1 is still discussing the default value[s] with extended periodicity.

Observation 2: In RAN4 spec, there is no explicit SSB periodicity in initial access related requirements, and only the SMTC periodicity is used to describe it.

Proposal 1: Due to SSB periodicity extension assumed by UE during initial access, there may be no explicit RRM spec impact on initial access related requirements.

Proposal 2: RAN4 need to discuss whether and how to define the transition requirement for the following case:

- The UE transitions between any two states when being configured with SSB periodicity or extending SSB periodicity.

Proposal 3: The beam steering latency should be further clarified by proponent.

Proposal 4: The delay of antenna switching for beam hopping is short enough and can be ignored.

Proposal 5: The timing offset/error (between beam switching timing and DL/UL framework) should be further clarified by proponent.

Proposal 6: If the timing offset/error (between beam switching timing and DL/UL framework) is caused by the synchronization error between the gNB/gateway and satellite, RAN4 will not consider it in NTN network (FDD).

Proposal 7: There is no RRM impact for uplink capacity/cell throughput enhancement.

Observation 3: There is no need to consider the TA compensation for feeder link, so the NTA,adjcommon in clause 7.1C.2 in TS38.133 will be considered as 0.

Proposal 8: Reply RAN2 that sticking to 0 as the minimum possible value for TA-Common is reasonable in regenerative payload scenario.

Observation 4: In TS 38.133, Te requirements already took into account the worst case, and all of possible errors have been considered, enough margin has been taken into account when specifying the Te requirements in RAN4.

Proposal 9: Reply RAN2 that RAN4 default that the UE needs to meet the UE UL transmit timing Te requirements defined in clause 7.1C.2 in TS38.133 for UL initial transmission in any case.

Proposal 10: Reply RAN2 that RAN4 has already considered all of possible errors and enough margins when specifying the Te requirements in RAN4.

Observation 5: The values of Te_NTN are also almost smaller than half of CP length for the given SCS of UL channel/signal, except in the case of a 120 kHz SCS of uplink signals in FR2 for mobile VSAT is served by GSO.

Proposal 11: Reply RAN2 that the values of Te_NTN are also almost smaller than half of CP length for the given SCS of UL channel/signal and the current Te requirements make it possible for the network side to demodulate the uplink information of different UEs through implementation in most cases.

Proposal 12: There is no RRM impact on NTA,adjcommon in timing requirements in TS 38.133 for regenerative mode of NTN.

- RAN4 just reply the LS from RAN2 about setting the value of NTA,adjcommon.

- The impact on other spec can be up to RAN1/2.

Proposal 13: RAN4 doesn’t need to define the requirements of satellite switching with resync for regenerative payload.

- The current requirements of satellite switching without PCI change is not applied for regenerative mode of NTN.

- Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- 3GPP TS 38.133, NR Requirements for support of radio resource management

- R2-2407623 LS on common TA in a regenerative payload scenario, RAN2 #127

- R4-2317374, RRM WF and CR work split for NR_NTN_enh, Qualcomm Incorporated