3GPP RAN WG4 Meeting #112bis	R4-2415166

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on Core requirements of SL Pos and CPP

Source: 	CATT

Agenda item:	4.2.1.2

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed the core requirements for SL and carrier phase positioning, and the agreements were captured in the WF [1]. In this paper, we will further discuss some leftovers.

Discussion

LS

In RAN4#111 meeting, a LS was sent to RAN1/2 to check whether any solution to inform measuring UE about the sync source change at transmitting UE. And a reply LS is received from RAN1 [2] where RAN1 confirmed that no solution is introduced to specifically inform the Rx UE about synchronization reference source change of the UE transmitting SL-PRS and RAN1 has no intention to introduce solutions for such cases. Therefore RAN4 can confirm that the defined measurement period requirements do not apply when the synchronization reference source changes at transmitting UE during measurements.

Proposal 1: RAN4 confirms that the defined measurement period requirements do not apply when the synchronization reference source changes at transmitting UE during measurements. No updates on spec are needed.

SL RSTD measurement delay requirements

In last RAN4 meeting, the measurement delay requirements for SL RSTD were discussed without any agreements made. The conclusion is copied below:

Conclusion: - No common understanding on RAN2 procedure for reporting behaviour. Companies check offline. - No consensus at the moment to revisit previous agreement.

The core of this issue is how to determine the start point and end point for one SL RSTD measurement and the answer will affect whether or not to revise the SL RSTD measurement delay requirements. The current formula in the spec is for a single link according to the following highlighted contents:

for each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities [Components 2 and 3 of FG 41-1-1], TSL RSTD,effect,s is defined as: TSL RSTD,effect,s=ts+1-ts , for s<S, where ts+1 and ts are the beginning of the first slot of SL-PRS sample s+1 and SL-PRS sample s, respectively, TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S, Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RSTD measurement, ΔSLproc is the processing time given by the UE capability in [Components 4 of FG 41-1-1].

For each SL RSTD measurement, at least two different measurement links are required. Therefore it seems the current requirements are not sufficient to define the whole measurement duration for a single RSTD measurement. A solution is to choose the earliest start point of different measurement links and latest end point of these measurement links. In order to explain the idea, possible cases with 4 samples are illustrated in Figure 1. Note that it is assumed that the start point of RSTD measurement delay and the end of last SL-PRES in Figure 1 are aligned with the boundary of the slots for simplification.

Figure 1 Illustration of SL RSTD measurement delay

During discussions in last meeting, it was also mentioned that whether reference UE can be different for different measurement report. In our understanding, according to RAN2 spec, only one reference UE is selected by measuring UE for each SL RSTD request and measurement report, while the reference UE can be different for different SL RSTD requests and reports.

Observation 1: The reference UE is selected by measuring UE for each SL RSTD request and measurement, while for different SL RSTD requests, measuring UE can select different reference UE.

Proposal 2: RAN4 to revise the SL RSTD measurement delay requirements to explicitly define the start point and the end point which are the earliest start point and the last end point among all links respectively for each SL RSTD measurement, where the reference UE is selected by the measuring UE for each SL RSTD request.

CPP measurement period requirements with aperiodic time window

During CPP discussions, RAN4 agreed to define measurement period requirements with aperiodic time window and a high-level principle was provided:

Agreements: - Define the requirement for one-shot window. - The requirement is defined by taking the time window length + processing time.

According to the agreements, the requirement is defined by taking the time window length + processing time. The time window length is configured by network via a LPP signalling. In regard to the processing time length, the available PRS resource length and UE processing capabilities should be taken into account.

For the available PRS resource length, a similar definition to  Lavailable_PRS,i can be defined as  Lavailable_PRS_onoShot,i:

Lavailable_PRS_onoShot,i is the time duration of available PRS in the positioning frequency layer i to be measured during the indicated one shot time window, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_onoShot,i, only the PRS resources unmuted and fully or partially overlapped with one shot time window are considered.

The total processing time length can be defined as follows, considering UE processing capabilities:

Tprocessing_onoShot,i=Lavailable_PRS_onoShot,iN*T

Where {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

Based on the above discussions, the measurement period requirements with aperiodic time window is defined as:

TRSCPD with RSTD,i=LoneShotTimeWindow+ Tprocessing_onoShot,i

Proposal 3: The measurement period requirements with one shot time window for DL RSCPD with RSTD can be defined as:

TRSCPD with RSTD,i=LoneShotTimeWindow+ Tprocessing_onoShot,i

Where

LoneShotTimeWindow  is length of the indicated one shot time window;

Tprocessing_onoShot,i  is the processing time duration and is defined as

Tprocessing_onoShot,i= Lavailable_PRS_onoShot,iN*T

Where

- Lavailable_PRS_onoShot,i is the time duration of available PRS in the positioning frequency layer i to be measured during the indicated one shot time window, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_onoShot,i, only the PRS resources unmuted and fully or partially overlapped with one shot time window are considered.

- Where {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

Conclusions

This paper discussed the core requirements for SL and carrier phase positioning, and some proposals are provided:

SL positioning:

Proposal 1: RAN4 confirms that the defined measurement period requirements do not apply when the synchronization reference source changes at transmitting UE during measurements. No updates on spec are needed.

Observation 1: The reference UE is selected by measuring UE for each SL RSTD request and measurement, while for different SL RSTD requests, measuring UE can select different reference UE.

Proposal 2: RAN4 to revise the SL RSTD measurement delay requirements to explicitly define the start point and the end point which are the earliest start point and the last end point among all links respectively for each SL RSTD measurement, where the reference UE is selected by the measuring UE for each SL RSTD request.

CPP:

Proposal 3: The measurement period requirements with one shot time window for DL RSCPD with RSTD can be defined as:

TRSCPD with RSTD=LoneShotTimeWindow+ Tprocessing_onoShot,i

Where

LoneShotTimeWindow  is length of the indicated one shot time window;

Tprocessing_onoShot,i  is the processing time duration and is defined as

Tprocessing_onoShot,i= Lavailable_PRS_onoShot,iN*T

Where

- Lavailable_PRS_onoShot,i is the time duration of available PRS in the positioning frequency layer i to be measured during the indicated one shot time window, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_onoShot,i, only the PRS resources unmuted and fully or partially overlapped with one shot time window are considered.

- Where {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

References

[1] R4-2414033, WF on SL positioning and carrier phase positioning, CATT, RAN4#112.

[2] R4-2414905 (R1-2407401), Reply LS on synchronization source change at the transmitting anchor UE in SL positioning, RAN1#118.