3GPP RAN WG4 Meeting #112bis	R4-2415168

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on Performance requirements of SL positioning

Source: 	CATT

Agenda item:	4.2.2.2

Document for:	Discussion

Introduction

In RAN4#112 meeting, fruitful progress regarding the performance requirements was achieved which was captured in the WF [1] and Rel-18 Positioning WI was successfully closed. However, there are some issues which need further discussions. In this contribution, we will discuss these performance issues.

Discussion

- Issue 1: SL-PRS signal level configurations

During the discussion on SL-PRS signal level in last RAN4 meeting, a related issue was brought up which is whether overlapping SL-PRS from different UEs can be used in TCs. The options are shown below:

Agreements:-RAN4#112 - Check the RAN1 specification and the Uu TCs in relation to overlapping SL-PRS from different UEs. Further discuss the options: - Option 1: overlapping SL-PRS can be used in TCs. - Option 2: overlapping SL-PRS should not be used in SL TCs.

In our understanding, the solution to this issue depends on if a multiplexing method, e.g., FDM or CDM based, is used. If no multiplexing is used, overlapping SL-PRS resources in one slot probably also overlap in other domains, such as frequency domain or code domain, and this will cause non-negligible interference and thereby lead to a poor positioning performance.

If we consider using overlapping SL-PRS resources in one slot with applicable multiplexing methods, this will lead us to the issue of type of resource pools, since different types of resource pools support different multiplexing according to RAN1 agreements. The related RAN1 agreements are copied below:

RAN1#agreeements: - TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - TDM-based multiplexing in a slot of SL PRS from different UEs is NOT supported for a shared resource pool. - Comb-based multiplexing of SL PRS resources from different UEs in a slot is NOT supported for shared resource pools.

Based on the agreements, if a dedicated resource pool is configured for a TC to which the SL-PRS resources from different UEs belong, the comb-based or FDM-based or CDM-based multiplexing can be considered to achieve time domain overlapping SL-PRS resources. While if a sharing resource pool is used, the comb-based multiplexing is not an option anymore. Also it should be noted that in the legacy TCs, CDM-based multiplexing is rarely used.

Furthermore, there are some relations between resource pool configurations and SL-PRS configurations which will affect the TC configurations:

Agreement- RAN1#114bis: For a dedicated resource pool, where SL PRS bandwidth is same as resource pool bandwidth, the following interpretation applies: SL PRS bandwidth corresponds to all PRBs of the resource pool bandwidth.

Since the SL-PRS bandwidth is the same as the resource pool bandwidth for a dedicated resource pool, if a dedicated resource pool is configured for a TC and RAN4 would like to use FDM-based multiplexing to configure time domain overlapping SL-PRS resources from different UEs, it means only one dedicated resource pool is not enough for this TC.

Based on the discussions above, applicable solutions of achieving time domain overlapping SL-PRS resources and the corresponding impacts are summarized in Table 1.

Table 1 Applicable solutions to achieve time domain overlapping SL-PRS

|  | Dedicated resource pool | Sharing resource pool |
| --- | --- | --- |
| Comb-based multiplexing | Usable. | Usable. |
| FDM-based multiplexing | Usable. Impacts: More than one dedicated resource pool needs to be configured. | Usable. |
| CDM-based multiplexing | Usable. Notes: CDM-based multiplexing is barely used in legacy positioning TCs. |  |

Besides, we also confirmed the configurations in legacy positioning TCs. Usually there are three cells are configured to transmit PRS signals, i.e., cell 1, 2 and 3, and cell 1 is the reference cell. In each configuration, only one PRS pattern is used for all three cells while cell 1 and cell 2 are configured with different PRS muting pattern which makes the PRSs from these two cells in a TDM manner. For cell 1 and cell 3, 1 RE offset is configured, which means comb-based multiplexing is used. This configuration methodology, in our opinion, can be directly applied for SL positioning TCs. i.e., anchor UE1 and anchor UE2 transmit SL-PRS in a TDM manner and SL-PRS resources from anchor UE1 and anchor UE3 can be comb-based multiplexed. In regards to the type of resource pool, either can be chosen with comb-based multiplexing used.

Observation 1: Overlapping SL-PRS can be used in TCs with comb-based multiplexing adopted.

Back to the issue about SL-PRS signal level, since the measurement accuracy requirements have been defined with agreed Es/Iot, it is straightforward and reasonable to use the same Es/Iot value in TCs. Otherwise we cannot verify the quality of measurement accuracy. In addition, some companies concern that correct Es/Iot for PSCCH should also be configured to avoid frequent PSCCH decoding failure, which thereby leads to SL-PRS being not measured. In order to deal with this concern on the possible PSCCH decoding failure, we are okay to use proposed Es/Iot for PSCCH, i.e., 3 dB. Regarding the SL-PRS configuration, we prefer to follow the legacy configuration method, i.e., using muting and RE offset to achieve non-overlapping PRS resources in TCs.

Proposal 1: Es/Iot for PSCCH with 3 dB can be used to avoid PSCCH decoding failure. And muting and RE offset can be configured to achieve non-overlapping SL-PRS resources.

- Issue 2: Other test case configurations

For other test case configurations, two further studies are left:

Agreements: - FFS: Shared and dedicated resource pools are randomly used among different TCs. - FFS: (symbol num, comb size): (4, 4) and (2, 4).

For the first issue, based on the discussions above, either type can be chosen without much difference in tests. Therefore we prefer not to explicitly configure the type of resource pools in SL positioning TCs.

For comb configuration, both configurations are already randomly used in SL test cases, and therefore RAN4 does not discuss this issue anymore.

Observation 2: Both comb configurations are already randomly used in SL test cases.

Proposal 2: RAN4 not to explicitly configure the type of resource pools in TCs.

Proposal 3: RAN4 does not discuss SL-PRS comb configuration for test cases anymore.

Conclusions

This paper discussed the performance requirements for SL positioning and provided the following proposals:

Observation 1: Overlapping SL-PRS can be used in TCs with comb-based multiplexing adopted.

Proposal 1: Es/Iot for PSCCH with 3 dB can be used to avoid PSCCH decoding failure. And muting and RE offset can be configured to achieve non-overlapping SL-PRS resources.

Observation 2: Both comb configurations are already randomly used in SL test cases.

Proposal 2: RAN4 not to explicitly configure the type of resource pools in TCs.

Proposal 3: RAN4 does not discuss SL-PRS comb configuration for test cases anymore.

References

[1] R4-2414033, WF on SL positioning and carrier phase positioning, CATT, RAN4#112.