3GPP RAN WG4 Meeting #112bis	R4-2415169

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on general aspects for AIML for NR air

Source: 	CATT

Agenda item:	6.17.1

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed the general aspects of Rel-19 AI/ML for NR air interface WI and agreements were captured in [1]. Some open issues listed in [2] need further discussions and we would like to present our views in this contribution.

Discussion

Testing goals

RAN4 has been discussion the test goal of AI/ML for NR air since R18 and finally a progress was made in last meeting which is shown below, i.e., RAN4 to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

One open issue is how many test configurations/parameters are used for tests. In our understanding, the answer to this issue depends on the granularity of “AI/ML functionality/feature” which will be decided by RAN1. For example, if the granularity of AI/ML functionality is use case and a UE indicates this functionality is supported, then any proper configuration should be supported by this UE, in which case, RAN4 can select a few preventative configurations for test. However, if the granularity is configuration, RAN4 probably have to provide sufficient test configurations because each UE may support some of the configurations. If limited number of test configurations is defined and none of them is supported by a DUT, then the test cannot be carried out, which is not very likely but possible. Besides, the discussions on details of test configurations/parameters should be based on each use case, and it is not efficient any more to discuss under general aspect agenda.

Observation 1: The number of test configurations/parameters depends on the granularity of functionality/ feature, pending on RAN1 discussions.

Proposal 1: RAN4 discusses the number of test configurations/parameters per use case after RAN1 agrees on the granularity of AI/ML functionality/feature.

AI/ML model performance

There are some proposals on whether AI.ML model performance should be better than legacy requirements, if defined. This issue relates to the motivation of each use case. For AI/ML enabled positioning, based on SI outcomes, apparently a higher positioning performance can be expected and it is reasonable for RAN4 to define a tighter requirements. So is it for CSI prediction. However, the main motivation of beam management is to reduce measurement resource and reporting overheads. Hence, it is enough to request UE to fulfil the legacy performance requirements. For CSI compression, it can be further discussed in WI part, if there will be one.

Proposal 2: Better performance can be expected for positioning and CSI prediction use case. For BM, UE is required to fulfil the legacy performance requirements. FFS for CSI compression in WI, if there will be one.

Generalization

Generalization highly affects the robustness of AI/ML functionality/model and some proposals were brought up:

- Proposals - Option 1: If non-static scenarios/configurations are supported for certain use cases, they can be included as part of generalization tests. - Option 2: take the requirements for inference as the minimum level performance for generalization. - Option 3: For generalization test, performance for other scenarios and/or configurations can be ensured by RAN4 legacy non-AI test - Option 4: Consider non-static scenarios separately from generalization over static conditions

Four options can be discussed in terms of two perspectives: test methodology (static vs. non-static) and performance.

Regarding the test methodology, in general both methods, i.e., static and non-static can serve the purpose of testing the generalization of functionality/feature. However, for some use cases, especially involving changes in time duration, non-static configuration is more suitable to be adopted, which needs further discussions on the details of such use cases after the basic test configurations for each use case are finished.

For the performance, we find that the degradations of generalization simulation performance are very divergent in R18 SI report. Therefore it seems quite difficult to reach a conclusion in this issue at current stage. Hence we propose to deprioritize the discussions on generalization performance until RAN4 completes the performance requirements for all AI/ML use cases and has a better understanding in the impacts of AI/ML mechanism.

In summary, deprioritizing the generalization discussion is more preferred, considering RAN4 has not figured out how to define the test configurations without generalization or what aspects to be generalized yet. Without consensus on both aspects, it is hardly to make any progress in generalization test discussions.

Proposal 3: RAN4 to deprioritize the discussions on generalization tests until RAN4 completes performance requirements and figures out how to define the tests without generalization.

LCM handling

- Proposals - Option 1: Postpone any LCM discussion until more RAN2/1 agreements are made. - agreements on possible requirements were made in the last meeting, all of them to be discussed later - Option 2: RAN4 to study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML). - FFS how this delay is defined and what it depends on (e.g. procedure delay, side conditions, etc) - Option 3: Consider LCM requirements per use-case. - Option 4: Other options

As Option 1 mentioned, possible requirements were listed in RAN4#111 meeting and they will be discussed later when RAN1/2 make sufficient progress, based on which RAN4 can start the discussions on LCM requirements. In addition, LCM requirements are a general issue for all AI/ML use cases, though some adjustments may be needed for each use case. In order to improve the efficiency, RAN4 can focus on one use case first and extends the LCM requirements to other use cases.

Proposal 4: Postpone LCM discussions until more RAN1/2 agreements are made. When LCM discussions start, RAN4 can focus on one use case and extends the LCM requirements to other use cases, if necessary.

Conclusions

This paper discussed the general issues related to AI/ML for NR air interface, and following observations and proposals are provided:

Observation 1: The number of test configurations/parameters depends on the granularity of functionality/ feature, pending on RAN1 discussions.

Proposal 1: RAN4 discusses the number of test configurations/parameters per use case after RAN1 agrees on the granularity of AI/ML functionality/feature.

Proposal 2: Better performance can be expected for positioning and CSI prediction use case. For BM, UE is required to fulfil the legacy performance requirements. FFS for CSI compression in WI, if there will be one.

Proposal 3: RAN4 to deprioritize the discussions on generalization tests until RAN4 completes performance requirements and figures out how to define the tests without generalization.

Proposal 4: Postpone LCM discussions until more RAN2/1 agreements are made. When LCM discussions start, RAN4 can focus on one use case and extends the LCM requirements to other use cases, if necessary.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] R4-2412831, Topic summary for [112][129] NR_AI/ML_air, Qualcomm Incorporated, RAN4#112.