3GPP RAN WG4 Meeting #112bis	R4-2415172

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on RRM core requirements for positioning

Source: 	CATT

Agenda item:	6.17.5.1

Document for:	Discussion

Introduction

In R19 RAN4 meetings, discussions for AI/ML enabled positioning accuracy enhancement were focused on testability and interoperability, while the RRM core requirements were barely discussed. In this contribution, we will present our views on the potential impacts on RRC core requirements.

Discussion

Impacts on RRM core requirements

First of all, considering RAN4 core requirements are mainly defined for user equipment, RAN4 only needs to define core requirements for use case 1/2a/2b which are all based on DL PRS measurements.

Proposal 1: Case 3a/3b has no impacts on RRM core requirements.

In case 2b, since the AI/ML models are deployed at LMF side and UE behaviours are exactly the same as the legacy, the existing positioning core requirements can be directly reused.

Proposal 2: For case 2b, the existing positioning core requirements can be directly reused.

In case 1 and 2a, the general procedures are quite similar and the difference is UE reports UE location in case 1 while it reports intermediate measurement quantities to LMF in case 2a. Therefore a single set of core requirements can be defined for both use cases.

Proposal 3: RAN4 to define a single set of core requirements for case 1 and 2a.

Regarding the specific core requirements, the following requirements are mainly defined in positioning WIs in previous releases:

-	Reporting requirements

-	Measurement period requirements

Since RAN2 agreed to reuse LPP signallings for at least case 1 in previous meeting and, in our opinion, the framework of the legacy reporting requirements and measurement period requirements can be the baseline. Discussions on the details of measurement period requirements need more inputs from RAN1/2.

Observation 1: RAN2 agreed to reuse LPP signallings for at least case 1.

Proposal 4: RAN4 to reuse the framework of legacy reporting and measurement period requirements. Details are FFS pending more RAN2 inputs.

Conclusions

This contribution discussed the potential impacts of AI/ML enabled positioning accuracy enhancement on RRM core requirements and following proposals are provided:

Proposal 1: Case 3a/3b has no impacts on RRM core requirements.

Proposal 2: For case 2b, the existing positioning core requirements can be directly reused.

Proposal 3: RAN4 to define a single set of core requirements for case 1 and 2a.

Observation 1: RAN2 agreed to use LPP signallings for at least case 1.

Proposal 4: RAN4 to reuse the framework of legacy reporting and measurement period requirements. Details are FFS pending more RAN2 inputs.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] R4-2412831, Topic summary for [112][129] NR_AIML_air, QC, RAN4#112.