3GPP RAN WG4 Meeting #112bis	R4-2415174

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on testability and interoperability issues for CSI compression

Source: 	CATT

Agenda item:	6.17.2.1

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But there are still some issues related to testability and interoperability that need to be further discussed. In this contribution, we will present our views on the following issues:

-	Simulation results for Option 3.

-	Decoder selection criteria.

-	Options for Option 4.

-	TE decoder verification/validation.

Discussion

Simulation results for Option 3

In last meeting, many companies submitted SGCS results of CSI compression based on the simulation assumption [2]. However, the alignment was not very well considering there was a 10% difference between the highest and the lowest result. More simulation parameters were aligned during the meeting and captured in [3]. We updated our simulation results based on the latest assumptions and the SGCS is shown in Figure 1 below.

Figure1. Performance of CNN based CSI compression

Observation 1: SGCS increases from 75% to 78% after simulation assumptions were refined.

Decoder selection criteria

There were several options for criteria of selecting a decoder which are copied below:

- Proposals - Option 1: Best performance (SGCS) and/or mean performance of corresponding eType II CSI feedback - Option 2: select multiple and work in parallel - few decoders can be selected and a subset of companies check feasibility on one decoder - Option 3: random selection, as long as performance is similar it does not matter which decoder is selected - Option 4: decoder trained with the highest number of sample - Option 5: To use an aggregated dataset of channel realizations/eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies. - Option 6: other criteria

Considering that RAN4 is carrying out the feasibility study of two-sided model tests and the selected model will not be captured in specifications, we think random selection is acceptable after the simulation performance is well aligned. In regards with other options, option 2 and 5 will result in extra simulation works and option 4 may not be a good criterion for model selection. In addition, eType II CSI performance can optionally be taken into consideration.

Proposal 1: RAN4 can randomly select a decoder after performance is well aligned. eType II performance can optionally be taken into account.

Options for Option 4

For feasibility of option 4, more than 10 proposals eventually converged to three sub-options as shown below:

Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Regarding the option 4a and 4b, we do not think they can work properly separately and any of them does not limit TE implementations of decoders. For 4a, a very possible case is UE vendor and TE vendor choose a different model backbone and they may not perform well with each other, even though the dataset is specified.

Observation 2: For 4a, different model backbone can be chosen by UE vendor and TE vendor and these models may perform badly with each other.

For 4b, TE can implement a decoder as they want without many limits. And since no datasets are specified in spec, it is unlikely for encoders developed by UE vendors and the decoders developed by TE vendors to cooperate well with each other.

Since the key purpose in two-sided model is to eliminate the impacts of TE implementations on test results, RAN4 should limit TE implementations of decoder and focus on testing the performance of encoder. From this point of view, one solution is that a reference decoder without weight parameters is documented somewhere and a dataset is also specified. UE vendors can develop their own encoder and train it with best effort to pass tests. In TE sides, they are better to implement the documented reference model and also train it with the specified dataset to provide a prepared decoder to cooperate with encoder to be tested. Furthermore, other aspects may also be specified, depending on future discussions, such as quantization method.

Proposal 2: RAN4 documents both reference decoder and dataset with potential key aspects specified, such as quantization method.

Option 4c is the final target for option 4. However, more studies and discussions are required. Actually, conclusions in Option 3 discussions can be reused for Option 4 study. A high-level procedure for possible Option 3/4 relationship is provided below for reference:

Figure2. A high-level procedure of the feasibility study on Option 3 and 4

According to the Step 1 in the right side, if companies achieve a similar evaluation result based on what RAN4 specifies now, i.e., model backbone/structure and simulations assumptions based on which the training/testing data are generated, it means these information are enough for UE and TE vendors to develop and train their own models respectively, and an expected test performance can be achieved. If so, RAN4 does not need to spend more efforts on defining reference models and datasets, etc. And RAN4 can further study whether more flexibility can be left to TE/UE vendors.

But if the results cannot be aligned, RAN4 can try to fix more necessary parameters to evaluate the feasibility of Option 4, including defining the channel generation method or datasets generated based on test scenarios and configurations or reference encoder/decoder.

Proposal 3: For Option 4c study, companies to run simulations with their own trained encoder and the chosen decoder and report the results for alignment.

TE decoder verification/validation

Some companies proposed that the AI/ML models implemented by TE vendors should be verified before UE vendors take tests in Option 4. All possible options were summarized in the last meeting, but no agreements were made.

Issue 4-6: TE decoder verification/validation - Proposals - Option 1: RAN4 will have to come up with a TE verification/validation procedure - details are FFS - Option 2: This should be left to RAN5 - Option 3: TE verification/validation is not needed - Option 4: other options

We totally understand the motivation why this issue is brought up and we also agree the quality of TE is important. But, from my understanding, evaluating the quality of TE implementations is not in RAN4 scope, and RAN4 should believe RAN5 have or will design mature procedures to ensure the AI/ML-based test results are fair.

Proposal 4: Verifying the TE verification/validation is out of RAN4 scope.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML enabled CSI compression and some observations and proposals are provided:

Observation 1: SGCS increases from 75% to 78% after simulation assumptions were refined.

Proposal 1: RAN4 can randomly select a decoder after performance is well aligned. eType II performance can optionally be taken into account.

Observation 2: For 4a, different model backbone can be chosen by UE vendor and TE vendor and these models may perform badly with each other.

Proposal 2: RAN4 documents both reference decoder and dataset with potential key aspects specified, such as quantization method.

Proposal 3: For Option 4c study, companies to run simulations with their own trained encoder and the chosen decoder and report the results for alignment.

Proposal 4: Verifying the TE verification/validation is out of RAN4 scope.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] R4-2410570, WF on AI/ML, Qualcomm Incorporated, RAN4#111.

[3] R4-2414447, Alignment of model training and data set parameters for AI/ML-based CSI compression, Nokia, RAN4#112.