3GPP RAN WG4 Meeting #112bis	R4-2415175

Hefei, China, October 14 – October 18, 2024

Title: 	Discussion on testability and interoperability issues for CSI prediction

Source: 	CATT

Agenda item:	6.17.3.1

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. The relative throughput was chosen to be the metric for CSI prediction, but the baseline was not finally determined. In this contribution, we will continue to discuss the metrics.

Discussion

CSI prediction accuracy metric

The agreement in last meeting is reproduced below:

Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

In legacy minimum performance requirements of PMI reporting, relative throughput is adopted and the baseline is the throughput obtained by using a random PMI [2]. In AI/ML enabled CSI prediction, there are two alternative for the baseline:

Alt1: Random PMI.

Alt2: PMI reported based on non-AI solution.

Both alternatives have merits and demerits. Alt1 has limited effects on TE and is also friendly to UE, because TE can reuse the existing solutions and UE does not need to implement AI and non-AI PMI reporting for a single CSI report. But the issue is even UE passes the tests, whether AI/ML based CSI prediction outperforms the non-AI solutions is not known. Alt2 can fix the issue caused by Alt1, while it may cause longer test duration and higher loads to UE. In our understanding, though Alt 2 has some demerits, they are acceptable for RAN4 conformance tests and the performance of AI/ML based CSI prediction over non-AI one can be tested. Therefore we prefer to use non-AI based PMI as the baseline for CSI prediction tests. Figure 1 below illustrates a high-level scheme of the CSI prediction tests.

Figure 1 High-level illustration of CSI prediction test

Proposal 1: RAN4 to use non-AI based PMI as the baseline of accuracy metric.

Conclusions

This contribution discussed the issues related to AI/ML enabled CSI prediction and some proposals are provided:

Proposal 1: RAN4 to use non-AI based PMI as the baseline of accuracy metric.

References

[1] R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112.

[2] TS 38.101-4, V18.4.0.