3GPP RAN WG4 Meeting #112bis	R4-2415176

Hefei, China, October 14 – October 18, 2024

Title: 	Views on impacts of AIML mobility on RAN4 requirements

Source: 	CATT

Agenda item:	6.18.2

Document for:	Discussion

Introduction

In RAN#105, RAN4 scope of AIML mobility was clarified and one aspect is to study the impacts of AI/ML mobility on RRM requirements [1]. The details are reproduced below:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this contribution, we will present our views on impacts of AI/ML mobility on RRM requirements.

Discussion

In this contribution, the impacts of AI/ML based mobility on RAN4 requirements will be discussed in terms of core and performance requirements. For core requirements, discussions will be further divided for AI/ML-specific requirements, i.e., LCM-related core requirements, and non-AI/ML-specific requirements, e.g., legacy delay requirements.

Impacts on core requirements

First of all, as discussions proceed in AI/ML for NR air WI, there will be some delay requirements for LCM procedures which have not been defined yet. For AI/ML based mobility, delay requirements are needed as well. The issue is if they will be the same as the ones defined in AI/ML for NR air WI, which is difficult to be answered at current stage. To avoid redundant discussions on the same topic, RAN4 should wait for the agreements on delay requirements for LCM procedures in AI/ML for NR air WI.

Proposal 1: LCM procedures have impacts on delay requirements and RAN4 waits for agreements in AI/ML for NR air WI.

In regards with the potential core requirements, except for the LCM-related requirements, due to introduction of AI/ML capabilities, such as measurement delay, handover delay and interruptions, it is premature to discuss detailed impacts on these requirements until a relatively stable mechanism of AI/ML assisted mobility is designed by RAN2.

Proposal 2: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility.

Impacts on performance requirements

As discussions in our contribution [2] on testability and interoperability issues, both input and output of AI/ML models are RSRP in basic cases and UE will report the predicted RSRP results to gNB for handover evaluations. Hence, an issue will be that whether to reuse or tighten the existing RSRP accuracy requirements, which needs to be discussed in RAN4.

Proposal 3: RAN4 discusses to reuse or tighten the existing accuracy requirements for RSRP measurements for RRM measurement prediction case.

Conclusions

This contribution discussed the impacts of AI/ML mobility on RRM requirements and some proposals are provided:

Proposal 1: LCM procedures have impacts on delay requirements and RAN4 waits for agreements in AI/ML for NR air WI.

Proposal 2: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility.

Proposal 3: RAN4 discusses to reuse or tighten the existing accuracy requirements for RSRP measurements for RRM measurement prediction case.

References

[1] RP-242393, Revised SID on AIML for mobility in NR, OPPO, RAN#105.

[2] R4-2415177, Discussion on testability and interoperability of AIML mobility, CATT, RAN4#112bis.