3GPP RAN WG4 Meeting #112bis	R4-2415178

Hefei, China, October 14 – October 18, 2024

Title: 	Discussions on SBFD RRM requirements

Source: 	CATT

Agenda item:	6.20.3

Document for:	Discussion

Introduction

In RAN4#112 meeting, RAN4 started the discussions on the RRM requirements of R19 SBFD WI. Some high-level agreements were achieved and captured in way forward [1]. In this contribution, we will continue to discuss the impacts on RRM requirements.

Discussion

RRM impacts of UE-to-UE CLI handling

Measurement period requirements for CLI

RAN4 agreed to define measurement periods for L1 based UE-to-UE CLI measurement requirements, but the details, such as number of measurement samples which affects the measurement accuracy, need further study.

Agreements (from online session) - RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements, - RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design. - Number of shots for simulation: 1, 2, 3. - Companies to bring proposals on other simulation parameters in the next meeting.

Except the number of measurement samples, RAN4 can reuse the framework of R16 CLI measurement period in clause 9.7.2.5 for CLI-SRS-RSRP measurement and the one in clause 9.7.3.5 for CLI-RSSI measurement [2].

Proposal 1: RAN4 to reuse the framework of R16 CLI measurement period in clause 9.7.2.5 for CLI-SRS-RSRP measurement and the one in clause 9.7.3.5 for CLI-RSSI measurement.

For the following simulation parameters, some of which may have impacts on measurement period requirements, the framework of R16 CLI simulation assumptions for CLI-SRS-RSRP and CLI-RSSI [3][4] can be reused while some parameters need to be revised based on RAN4 discussions, at least including

-				  SRS Es/Iot.

-				  Number of shots for SRS measurements.

-				  Timing offset.

-	  Bandwidth of measurement resources.

In our understanding, there are two parts in R19 CLI that are different from R16 CLI: (1) the source of interferences is intra-cell; (2) reporting is based on existing CSI framework for delay reduction.

The impacts of the first part include larger interferences due to shorter distances and a smaller timing offset. However, if RAN4 agrees to reuse the R16 SRS-based CLI Es/Iot value, the impact of interference change can be ignored, since it is reflected by the Es/Iot. In addition, it is reasonable to remove cell phase error from the legacy time offset assumption considering the interferences in R19 comes from intra-cell UEs.

With regards to the impact of the second part which is mainly for delay reduction, where L1/L2 signallings are used for reporting instead of L3 signallings. However, no matter which types of signallings are used for reporting, measurement accuracy should be stable enough to reflect a real interference level. Taking into account the timing offset assumption may be changed, RAN4 can simulate all three samples {1, 2, 3} for measurement accuracy comparison and choose a suitable one that provides a balance between measurement delay and measurement accuracy.

Regarding SRS Es/Iot, we prefer to reuse the legacy value, i.e., SRS Es/Iot ≥ 1 dB.

Proposal 2: RAN4 to reuse the R16 CLI simulation assumption frameworks and for CLI-SRS-RSRP,

- SRS Es/Iot ≥ 1 dB;

- Simulate {1, 2, 3} samples to choose one that provides a balance between measurement delay and accuracy.

- Remove the cell phase error from the legacy time offset assumption.

Other simulation assumptions follow R16 requirements and can be revisited if impacted by other WGs.

Impacts of measurement resources on CLI measurement accuracy

As discussed above, the assumptions of SRS measurement resources can reuse the R16 ones considering there is only one UL subband in SBFD symbols. However, one thing worth mentioning for CLI RSSI is there can one or two DL subbands. When two DL subbands are configured and UE is requested to measure CLI-RSSI on these discontinuous frequency resources, measurement accuracy degradation may occur and RAN4 should take it into account during simulations.

Proposal 3: Configuration with two discontinuous DL subbands should be taken into account when simulating and defining measurement accuracy requirements for CLI RSSI measurements.

Measurement reporting for CLI

In regards with measurement reporting, a FFS is whether L1- RSRP measurement reporting requirement can be reused for CLI aperiodic reporting.

Recommended WF: - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement - FFS whether L1-RSRP measurement reporting requirement can be re-used.

According to RAN1 agreement, L1-based CLI measurement and reporting are based on existing CSI framework and at current stage only aperiodic reporting is supported, indicating the aperiodic reporting will be triggered by DCI, which is not the same as R16 CLI reporting scheme. Hence, we prefer to reuse the L1-RSRP measurement reporting requirement, but the wording needs to be updated when the reporting scheme design is stable in the end.

Proposal 4: L1-RSRP measurement reporting requirement can be reused, but wording needs to be updated when reporting scheme design is stable.

Report mapping for CLI

Recommended WF - FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement. - FFS whether R16 report mapping can be re-used.

In our understanding, there is no huge performance difference between R16 CLI and R19 CLI. Therefore the range of R19 CLI measurement results might be the same as the R16 report mapping range and RAN4 can reuse the legacy mapping tables.

Proposal 5: RAN4 to reuse the R16 report mapping tables for R19 CLI measurements.

Rx beam assumption

In R16 CLI measurement requirements for FR2, the following Rx beam assumption was made:

For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

We think this assumption is still applicable and it can be revisited if RAN1 has a new assumption.

Proposal 6: The Rx beam assumption for R16 CLI measurement in FR2 can be reused and it can be revisited if RAN1 has a new assumption.

CLI measurement capability

Regarding the measurement capability for L1-based CLI measurement in terms of number of resources UE shall be able to monitor, it, in our opinion, should be decided by RAN1 instead of RAN4.

Proposal 7: Measurement capability for L1-based CLI measurement is out of RAN4 scope and should be decided by RAN1.

RRM impacts of SBFD operation

Requirements for SSB-based measurements

In last RAN4 meeting, requirements for SSB-based measurements were discussed and the following agreements were achieved. However, more discussions are needed for SSBs from neighbor cells since SSBs from neighbor cells may collide with DL receptions/UL transmissions in SBFD symbols in serving cell(s).

Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell.

Two options, which are quite straightforward, can be considered for the SSBs from neighbor cells:

- Option 1: RAN4 sends LS requesting RAN1 to clarify the SSB in RAN1#117 agreements [5] includes the SSBs from neighbor cells.

- Option 2: RAN4 assumes SSB-based neighbor cell measurements are prioritized, i.e., RAN4 to define scheduling restrictions on SSB symbols to be measured within SMTC windows without measurement gaps.

Considering the last meeting was the first meeting for SBFD WI and defining scheduling restrictions is not an urgent issue to be solved in RAN4#112bis, we think RAN4 can wait for more RAN1’s progress and RAN4’s discussions can based on the assumption that SSB-based neighbor cell measurements are prioritized first, if necessary.

Proposal 8: RAN4 to wait for more RAN1 progress on SSB-based neighbor cell measurements. RAN4’s discussions can base on the assumption that SSB-based neighbor cell measurements are prioritized first.

Requirements for CSI-RS-based measurements

During RAN1 discussions, several typical collision cases in serving cells were identified and corresponding solutions were provided. For periodic and semi-periodic CSI-RS from serving cell(s) in SBFD symbols, if UE is dynamically scheduled on the same SBFD symbols, CSI-RS receptions will be dropped according to the RAN1 solution. For the collision between aperiodic CSI-RS and dynamically scheduled UL transmissions, it is regarded as an error case which means no requirement impacts exist.

For CSI-RS-based neighbor cell measurements, it is clearer to be discussed in terms of intra-frequency measurements and inter-frequency measurements with two R18 prerequisites, i.e., MG is not needed for intra-frequency measurements and MG is needed for inter-frequency measurements.

For measurements without MG, the legacy scheduling restrictions defined for measurements in TDD bands can be reused, while for measurements with MG, there are no requirement impacts since communication transmissions are not allowed during the MG duration.

Observation 1: Measurement gap is not needed for intra-frequency CSI-RS based measurements and it is needed for inter-frequency CSI-RS-based measurements in R18.

Proposal 9: Requirements for CSI-RS-based measurements are defined based on the following principles:

- For CSI-RS-based serving cell measurements,

  - UL transmission is cancelled if cancellation timeline is met when semi-statically configured UL transmission collides with dynamically scheduled DL reception.

  - UE does not receive DL channel/signal when dynamically scheduled UL transmissions collide with semi-statically configured DL reception.

- For CSI-RS based neighbor cell measurements without MG, RAN4 reuses legacy scheduling restrictions for measurements in TDD bands.

- For CSI-RS based neighbor cell measurements with MG, there are no requirement impacts.

Requirements for scheduling restrictions

According to the discussions above, scheduling restrictions are needed for some cases. Besides, another case should also be considered. Assuming a SBFD-enabled UE is scheduled to receive DL transmission and then to transmit during SBFD symbols, a guard period at least equal to NTA should be reserved. In non-SBFD case, there is a guard period for this DL/UL transition to avoid interferences. However, no guard period is designed yet by RAN1 in SBFD scenarios. Hence, scheduling restriction probably is needed for DL/UL transition, and the specific value can be FFS based on RAN1 design, if any.

Proposal 10: Scheduling restrictions should take DL/UL transition time into account and the specific value can be FFS based on RAN1 design, if any.

Requirements for RACH

Considering the RACH requirements in current RAN4 spec are very general, we do not think there will be any impacts caused by SBFD operations. Therefore RAN4 does not need to update related RACH requirements.

Proposal 11: SBFD operations have no impacts on RAN4 RACH requirements.

Requirements for UL PC and/or spatial relation update

In regards with the UL PC, RAN1 agreed that separate UL power control for FR2 is supported based on the unified TCI state framework, which, in our understanding, has no new impacts on RRM requirements. For spatial relation update, we do not identify any impacts due to SBFD based on current RAN1 progress.

Proposal 12: Separate UL power control has no new impacts on RRM requirements. RAN4 can wait for RAN1 progress to check if there are any impacts on spatial relation switch.

Limits on the maximum number of DL/UL switching

A proposal to define a limit on the maximum number of DL/UL switching within SBFD periodicity was brought up to control the impacts on UE throughput. According to RAN1 agreements on SBFD configuration in RAN1#118 meeting, only one SBFD duration with consecutive SBFD OFDM symbols will be configured within a TDD-UL-DL pattern period, which means there will 2 DL/UL transition points ideally. But number of transition points may increase due to DL signals. The impacts of increased transition points might be minor because number of transition points may also increase in flexible symbols in legacy releases. Hence, RAN4 can further discuss the necessity of limiting the number of DL/UL switching within SBFD periodicity pending RAN1 progress.

Proposal 13: RAN4 to further discuss the necessity of limiting the number of DL/UL switching within SBFD periodicity pending RAN1 progress.

Conclusions

This contribution discussed the impacts of CLI and SBFD on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to reuse the framework of R16 CLI measurement period in clause 9.7.2.5 for CLI-SRS-RSRP measurement and the one in clause 9.7.3.5 for CLI-RSSI measurement.

Proposal 2: RAN4 to reuse the R16 CLI simulation assumption frameworks and for CLI-SRS-RSRP,

- SRS Es/Iot ≥ 1 dB;

- Simulate {1, 2, 3} samples to choose one that provides a balance between measurement delay and accuracy.

- Remove the cell phase error from the legacy time offset assumption.

Other simulation assumptions follow R16 requirements and can be revisited if impacted by other WGs.

Proposal 3: Configuration with two discontinuous DL subbands should be taken into account when simulating and defining measurement accuracy requirements for CLI RSSI measurements.

Proposal 4: L1-RSRP measurement reporting requirement can be reused, but wording needs to be updated when reporting scheme design is stable.

Proposal 5: RAN4 to reuse the R16 report mapping tables for R19 CLI measurements.

Proposal 6: The Rx beam assumption for R16 CLI measurement in FR2 can be reused and it can be revisited if RAN1 has a new assumption.

Proposal 7: Measurement capability for L1-based CLI measurement is out of RAN4 scope and should be decided by RAN1.

Proposal 8: RAN4 to wait for more RAN1 progress on SSB-based neighbor cell measurements. RAN4’s discussions can base on the assumption that SSB-based neighbor cell measurements are prioritized first.

Observation 1: Measurement gap is not needed for intra-frequency CSI-RS based measurements and it is needed for inter-frequency CSI-RS-based measurements in R18.

Proposal 9: Requirements for CSI-RS-based measurements are defined based on the following principles:

- For CSI-RS-based serving cell measurements,

  - UL transmission is cancelled if cancellation timeline is met when semi-statically configured UL transmission collides with dynamically scheduled DL reception.

  - UE does not receive DL channel/signal when dynamically scheduled UL transmissions collide with semi-statically configured DL reception.

- For CSI-RS based neighbor cell measurements without MG, RAN4 reuses legacy scheduling restrictions for measurements in TDD bands.

- For CSI-RS based neighbor cell measurements with MG, there are no requirement impacts.

Proposal 10: Scheduling restrictions should take DL/UL transition time into account and the specific value can be FFS based on RAN1 design, if any.

Proposal 11: SBFD operations have no impacts on RAN4 RACH requirements.

Proposal 12: Separate UL power control has no new impacts on RRM requirements. RAN4 can wait for RAN1 progress to check if there are any impacts on spatial relation switch.

Proposal 13: RAN4 to further discuss the necessity of limiting the number of DL/UL switching within SBFD periodicity pending RAN1 progress.

References

[1] R4-2413900, WF on SBFD RRM, Huawei, RAN4#112.

[2] 3GPP TS 38.133, V18.6.0.

[3] R4-1903438, Simulation assumption for CLI measurement requirements, LG Electronics, RAN4#90bis.

[4] R4-1907392, Simulation assumption for SRS-RSRP measurement requirements, LG Electronics, RAN4#91.

[5] Chair notes RAN1#117 eom1, RAN1#117.