3GPP TSG-RAN WG4 Meeting #112bis				                                         R4-2415184

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

Source:	CATT

Agenda Item:	6.22.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for adaptation of common signal/channel transmission and give our views.

2 Discussion

In last RAN4 meeting, it was agreed to deprioritize paging adaptation and PRACH adaptation until the RAN1 concrete conclusion is available, so in this part, we mainly discuss the requirements for SSB adaptation.

It was agreed in last RAN4 meeting as below to firstly discuss L1/L3 measurement requirements due to SSB adaptation impact.

Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

Since for now, RAN1 only agreed SSB periodicity adaptation, RAN4 can discuss the periodicity adaptation impact first. Based on the existing L1/L3 measurement requirements in TS 38.133, we can see that the requirements are based on SSB periodicity or SMTC periodicity. If network only indicate the periodicity adaption in time-domain, the measurement requirements definition would not be impacted.

Proposal 1: RAN4 to discuss SSB periodicity adaptation in time-domain as starting point.

Proposal 2: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply.

The other impact of the SSB periodicity change is that it may introduce new transition requirements for L1/L3 measurement. In the existing specification, the minimum requirement at transitions is defined for L1 and L3 measurement in clause 8.1.4 and 9.1.6 respectively. For L1 measurement, we understand the following transition requirements between different configurations of RLM resource can already accommodate the impact of SSB periodicity adaptation.

### 8.1.4 Minimum requirement at transitions When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each RLM-RS resource. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of RLM resources to a second configuration of RLM resources that is different from the first configuration, for each RLM resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for RLM present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. For UE supporting ncd-SSB-BWP-Wor-r18, when the UE transitions between RLM CD-SSB resource and RLM NCD-SSB resource due to BWP switching or due to SCG activation or deactivation during one evaluation period, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the first SSB type and the second SSB type after the BWP switching or the SCG activation or deactivation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each RLM-RS resource.

But for L3 measurement, since RAN1 only agreed to adapt SSB periodicity rather than SMTC periodicity and the requirements are based on SMTC periodicity, we think we can wait for RAN1 agreements on the SMTC configuration. If the SSB periodicity change does not make the SMTC reconfigured, we may not need to define the transition requirements.

Proposal 3: For SSB periodicity adaptation in time-domain, the existing minimum requirement at transitions for L1 measurement can still apply.

Proposal 4: Deprioritize the minimum requirement at transitions for L3 measurement until RAN1 has further progress on SMTC configuration.

In RAN1#118 meeting, RAN1 has the following agreements for SSB adaptation:

Agreement - For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Based on the above agreement, the SSB periodicity adaptation on SCell is supported while the adaptation on PCell is FFS. From RAN4 perspective, this would only impact the requirements applicability and RAN4 can wait for more progress to check if the defined requirements apply to PCell.

Proposal 5: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell.

3 Summary

In this paper, we have some further discussions on adaptation of common signal/channel transmission, and the following proposals are given:

Proposal 1: RAN4 to discuss SSB periodicity adaptation in time-domain as starting point.

Proposal 2: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply.

Proposal 3: For SSB periodicity adaptation in time-domain, the existing minimum requirement at transitions for L1 measurement can still apply.

Proposal 4: Deprioritize the minimum requirement at transitions for L3 measurement until RAN1 has further progress on SMTC configuration.

Proposal 5: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell.

4 References

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson, RAN4#112

- R4-2411817, Topic summary for [112][222] Netw_Energy_NR_enh, Moderator (Ericsson), RAN4#112