3GPP TSG RAN WG4 Meeting #112bis	R4-2415194

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.18.2

Source:	vivo

Title:	Discussion on impacts on RAN4 requirements for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we provide our initial views on impacts on RAN4 requirements for AI/ML mobility.

- Discussion

- 2.1 Potential requirements in AI/ML for mobility

RAN2 has concluded the following cases for RRM measurement prediction [2].

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) 3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For cell-level result measurements and prediction, the cell results are used as model input/output and model output, respectively. There is no potential RAN4 work for defining requirements.

Observation 1: There would be no RAN4 impact on cell-level result based measurements and predictions.

In terms of the beam-level result based measurements and predictions, L1 beam-level measurement and L3 beam-level measurement are all included in the current study phase.

In Fig. 1, beam-level results in A and A1 are all possible to be the input and output in the current stage. Based on the 6 sub-use cases and Fig. 1, we have:

- Sub-use case 1: Use A to predict A, or A1 to predict A1, and calculate C;

- Sub-use case 2: Use C to predict C;

- Sub-use case 3: Use A to predict C, or use A1 to predict C;

- Sub-use case 4: Use A to predict A, or use A1 to predict A1, and calculate C;

- Sub-use case 5: Use E to predict E;

- Sub-use case 6: Use A to predict E, or use A1 to predict E;

Where point A refers to the measurements (beam-specific samples) internal to the physical layer. And point A1 refers to the measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering. The internal layer 1 filters the inputs obtained at point A. The implementation (inputs A and Layer 1 filtering) of how the measurements are executed in the physical layer is not constrained in the standard.

Fig. 1 Measurement Model

Observation 2: It is still pending in RAN2 discussion on whether the measurement prediction is based on reported L3 results, or derived L3 results by combining measurement samples.

Given that it is still pending in RAN2 discussion on whether the measurement prediction is based on reported L3 results, or the L3 results derived by combining measurement samples, we provide our initial analysis of the potential impacts on the RRM requirements for these 2 cases.

Beam specific measurements based:

If measurement prediction is based on reported L3 results (A1 in Fig. 1), there would be no impact on existing RRM requirements except for considering defining corresponding requirements on RRM measurement prediction accuracy.

Beam specific samples based:

If measurement prediction is based on samples (A in Fig. 1), there could be some RRM impact. For example, for AI-based measurement, the original measurement samples are partially replaced by the predicted samples, which will facilitate the reduction of measurement samples as the two cases depicted in Fig. 2.

(a) Case A

(b) Case B

Fig. 2 Measurement samples reduced by AI prediction

Accordingly, the reduction of measurement samples will impact the measurement delay requirement. The details on impacts on the measurement delay depend on how to implement the reduction of the measurement sample by replacing it with prediction samples. For example, the measurement delay reduction depends on the measurement reduction rate in temporal domain (MRRT) and the UE measurement and report behaviour. Further study and discussion in RAN2 and RAN4 are needed. We give some analysis for the potential requirements based on case A and case B in Fig. 2.

Case A: Transmitted RS is configured with small period

Based on the measurement results of Mt RSs, AI will predict the future Pt instances in T2. From RAN4 perspective, if the measurement samples can be replaced by AI prediction values, the measurement delay requirement could be reduced, e.g., original 5 samples for SSB can be reduced to 3. The measurement delay could be considered as: reduced sample based measurement and AI processing.

Case B: Transmitted RS is configured with large period

According to the Fig. 2 (b), AI is used to predict the instance within the Yms in the future based the measurement results of large period RS of the first and last instance. The analysis is similar with case A that the measurement period could be reduced by replacing the measurement samples.

Note that if the last measurement sample is not required to predict, there is no need to reduce the measurement report delay requirement if it is required to report after the last measurement sample. Therefore, the UE measurement and predicted behaviour need to be clarified for RAN4 to discuss the requirements.

Prediction accuracy:

In RAN1 AI/ML air interface discussion, the beam prediction accuracy is the probability to reflect that the beam index is correctly predicted or not. However, in AI/ML mobility, there is no need to be concerned about such probability and only predicted RSRP is necessarily needed. Therefore, the prediction accuracy is based on the RSRP accuracy. We have mentioned that some beam level measurement could be skipped by AI prediction and the L3-RSRP value needs to be calculated by the measured samples and the predicted samples. Since the measurement values are more accurate than the predicted values, the derived L3-RSRP accuracy requirement could be more relaxed than the current requirement. The accuracy requirement for RSRP could also be related to the different MRRT and the SINR, RAN4 needs more evaluations to further study the prediction accuracy requirement.

Based on the above analysis, the potential RRM impacts mainly include two parts:

- The impact of measurement delay requirements due to the reduction of measurement samples;

- The RRM measurement prediction accuracy requirements.

Proposal 1: More clarifications for the model input of beam-level results are needed and RAN4 could discuss the potential RRM impacts at least including the following aspects at the initial study stage:

- The impact of measurement delay requirements due to the reduction of measurement samples;

- The RRM measurement prediction accuracy requirements.

- 2.2 Simulation aspects

RAN2 has agreed the simulation assumptions as Table 5.1-1 in TR 38.744. Both beam level and cell level results for training and inference all derived from the system level simulation in UMi and UMa channel. From RAN4 perspective, it may need to consider additional factors that may not be considered in RAN2 evaluation on prediction accuracy, e.g., measurement error modelling. At the initial study stage in RAN4 discussion, it can be considered as one of the study aspects. For beam level results, the measurement errors can be modelled as some types of probability distributions, or derived from link level simulations. System level simulation based beam level RSRPs are used as ideal dataset and add the beam level errors into the ideal dataset to derive the nonideal dataset to evaluate the measurement error impact is one feasible option which is also discussed in AI BM. Besides, if the model input is the cell-level result, there are two options for error modelling. First is to use the L3 filter to the nonideal beam-level results mentioned above to derive the nonideal cell-level results, second is to directly modelling the cell-level errors to add into the ideal cell-level dataset and the modelling method needs to be further studied and both options could be considered in RAN4.

Proposal 2: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

- Summary

In this contribution, we provide our initial views on impacts on RAN4 requirements for AI/ML mobility. Based on above analysis, following observations and proposals are present.

Observation 1: There would be no RAN4 impact on cell-level result based measurements and predictions.

Observation 2: It is still pending in RAN2 discussion on whether the measurement prediction is based on reported L3 results, or derived L3 results by combining measurement samples.

Proposal 1: More clarifications for the model input of beam-level results are needed and RAN4 could discuss the potential RRM impacts at least including the following aspects at the initial study stage:

- The impact of measurement delay requirements due to the reduction of measurement samples;

- The RRM measurement prediction accuracy requirements.

Proposal 2: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- TR 38.874 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Release 19)