3GPP TSG RAN WG4 Meeting #112bis	R4-2415195

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.18.3

Source:	vivo

Title:	Discussion on	testability and interoperability for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we provide our initial views on testability and interoperability issues for AI/ML mobility.

- Discussion

- 2.1	Testability issues in AI/ML based beam management

In AI/ML air interface discussion, many testability issues are raised and discussed in AI BM sub-case, where the key issue is whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics. A potential new FR2 OTA test system for AI BM will bring high cost and complexity for the related vendors and many technical issues are raised for the test setup.

Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion.

- 2.2 Potential testability issues in AI/ML for mobility

RAN2 has concluded the following cases for prediction.

Agreements - For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results

For cell-level result measurement prediction, the cell-level results are directly used as model input in case 2 and model output in case 3, respectively. There are no testability issues and the legacy conducted testing and OTA testing can be performed for FR1 and FR2.

Observation 2: There are no testability issues for cell-level result based measurements and predictions for AI mobility.

Regarding the beam-level result based measurements (case 3) and predictions (case 1), RAN2 has the following conclusion in simulation-related discussion.

Agreements on simulation assumptions For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation.

According to Fig.1 depicted in TS38.300, A and A1 points are all applicable in the current stage.

Fig. 1 Measurement Model

However, both processes are beam-level results based, and testability issues against cost and complexity should also be noticed. In AI/ML based beam management case 1, Top K beams are predicted in spatial domain and multi-probe anechoic chamber test system are needed to emulate the spatial channel environment. If one-probe setup or flat channel model is used, the real performance of AI model could not be reflected. In AI mobility, time and frequency domain prediction is performed and AI model performance can be verified in a test system without any spatial characteristics. For example, the AI model uses the measurement results of beam i, beam j and beam k during the observation window in t1 and predicts the corresponding results based on beam i, beam j and beam k during the prediction window in t2. The spatial characteristics in t1 and t2 are the same and no other beam-level results are needed to derive the eventual performance. Based on this understanding, 1 probe could emulate different Tx beams by changing the power weight of reference signal in FR2 for one cell. Note that 2 probes are needed if another cell is included in one test.

Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

Proposal 1: RAN4 to consider using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

Moreover, training methods and datasets are important factors that need to be taken it account. For training datasets, they are usually derived from simulations and field measurements. When we consider using fixed 8 beams pattern as the model input and 32 beams as the model output for deriving the spatial domain beam prediction dataset (set B = 8 and set A = 32) to train the model, one issue may be noticeable is that, once the model is already trained, only correct format of inference dataset can be applicable. If the data format is not matched, e.g., using 4 beams dataset as inference, it may cause the model to become unworkable since the model is only applicable for 8 beams format input. In AI mobility, the specific format issue also exists, which requires that AI model is capable of generalizing most types of input formats to guarantee a stable performance in the field since many Tx beams are usually drowned in the noise and may not be seen on UE side. From test perspective, bulid a simplified test system is the main goal and most of the Tx beams are skipped in the test lab, which will cause the input dimension mismatch issue. The issue is illustrated in Fig. 2, where ti is the time instance for measurement and tj is the time instance for prediction. RAN4 needs to consider this aspect in the following research and further perform evaluations before defining requirements and test methodologies.

Fig. 2 Illustration of model input dimension mismatch issue

Observation 4: A simplified RAN4 test system would skip Tx beams and the input dimension mismatch issue may happen in the lab.

Proposal 2: RAN4 to consider evaluating the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance.

- Summary

In this contribution, we provide our initial views on testability and interoperability issues for AI/ML mobility. Based on above analysis, following observations and proposals are present.

Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion.

Observation 2: There are no testability issues for cell-level result based measurements and predictions for AI mobility.

Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

Proposal 1: RAN4 to consider use one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

Observation 4: A simplified RAN4 test system would skip Tx beams and the imput dimension mismatch issue may happened in the lab.

Proposal 2: RAN4 to consider evaluate the impact of model input dimensions (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance.

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO