3GPP TSG-RAN WG4 Meeting #112bis												R4-2415267

Hefei, China, 14 – 18 October, 2024

Agenda item:			6.12.2.3

Source:	Huawei, HiSilicon

Title:	on MIMO dynamic channel tests

Document for:

# Introduction

The WF [1] in the last meeting has a few remaining issues such as the maximum number of HARQ transmissions on MIMO OTA tests. This contribution discusses MIMO dynamic channel related issues.

Issue 4-2-3: Test parameters for FR1 dynamic MIMO OTA

Agreements:

  - Set the maximum number of HARQ transmissions to [x] for FR1 dynamic MIMO OTA with x>1.

# Discussion

It is agreed in the last meeting that the maximum number of HARQ transmissions should be larger than one due to the dynamic nature of the channels. Typically, a maximum number of HARQ transmissions of 8 is used in network operations. However, the actual re-transmission operation does not have to use all 8 transmissions because once an ACK is received by gNB before reaching the maximum number of re-transmissions, the re-transmission would stop. Namely 8 is an upper limit. Obviously, we can set a value higher than 8, which may or may not represent realistic network settings.

Proposal 1: set the maximum number of HARQ transmissions to [8].

UMA, as its name suggests, represents urban macro scenarios, while UMi reflects channel conditions in micro environments. Dynamic MIMO OTA should include a broad selection of channel conditions in order to bear out network scenarios. In other words, the use of only UMa or only UMi for MIMO OTA tests would be unnecessarily restrictive. It is therefore more representative to test MIMO OTA performances under both UMa and UMi channel conditions.

The text proposal in [2] includes both UMa and UMi in various way points. Practical test issues and difficulties in arbitrarily combining UMa and UMi among way points have been highlighted in the last meeting. To avoid such difficulties, dynamic channel MIMO OTA tests for UMa and UMi can be carried out separately and their measurement results combined into a single data pool for processing, e.g. to derive performance metrics.

The only drawback for separately testing UMa and UMi is the lack of continuity between UMa and UMi channels. However, this should have little or no impact on the overall performance metrics because linear interpolation is used between way points.

Proposal 2: Dynamic channel MIMO OTA tests should include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for processing.

RI Restriction in Table 9.3-1: CSI configuration in [2] provides the following two options.

| RI Restriction |  | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| --- | --- | --- |

The option of “00000010 for fixed Rank 2” only allows for fixed rank two and is not realistic under fading channels.

The option of “00001111 for follow RI” allows UE to follow the rank indicator (RI) as determined by the channel conditions. In this case, UE can report any rank depending on the channel conditions and its capabilities. This option is appropriate for fading channel conditions. In addition, it is agreed in the last meeting that test emulators should use RI (Rank Indicator) reported by UEs.

Proposal 3: remove option “00000010 for fixed Rank 2” from CSI configuration in Table 9.3-1 of R4-2411252 and only keep option “00001111 for follow RI”.

# Conclusion

This contribution makes the following proposals.

Proposal 1: set the maximum number of HARQ transmissions to [8].

Proposal 2: Dynamic channel MIMO OTA tests should include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for processing.

Proposal 3: remove option “00000010 for fixed Rank 2” from CSI configuration in Table 9.3-1 of R4-2411252 and only keep option “00001111 for follow RI”.

# Reference

[1] R4-2413603 WF for TRP_TRS_MIMO_OTA

[2] R4-2411252 TP on Dyn MIMO