3GPP TSG-RAN WG4 Meeting #112bis												R4-2415268

Hefei, China, 14 – 18 October, 2024

Agenda item:			6.12.2.2

Source:	Huawei, HiSilicon

Title:	on NTN OTA test

Document for:

# Introduction

The WF [1] in RAN4 #112 highlights the issue with polarisation. It is also noted that the measurement parameters for band n254 has not proposed so far. In addition, RAN4 #111 did not conclude on the performance metrics for NTN devices.

Issue 3-3-4: How to resolve UE antenna circular polarization issue

Agreements:

  - FFS RAN4 how to accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

This contribution further discusses the above issues.

# Discussion

TRP is defined as

TRP= 14πθ=0πϕ=02πEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ

TRS is defined as

TRS=4πθ=0πϕ=02π1EISθθ,ϕ+1EISϕθ,ϕsinθdϕdθ

The formulae for TRP and TRS capture energy on a spherical surface by using θ and ϕ components. Those two components are orthogonal to each other and are able to collect power on a spherical surface regardless of wave polarisation or direction of polarisation. In release 17 and 18 measurement campaigns, all devices use linear polarisation.

For example, TRP and TRS measurement in previous releases for terrestrial networks are likely for devices with linear polarisation. The orthogonality of θ and ϕ components ensures full energy capture because any linear polarisation can be decomposed into θ and ϕ components.

By the same token, circular polarisation is two linearly polarised wave with a phase shift of 90 degrees as shown below under the plane wave assumption, which is valid because spherical waves from satellites can be considered as plane wave when reaching devices on earth surface.

E=E0(xcos⁡(ωt)+ysin⁡(ωt))

To be pedantic, the above equation can be re-written in terms of θ and ϕ components to reflect the spherical nature of the wave from satellites.

E=E0(θcos⁡(ωt)+ϕsin⁡(ωt))

Regardless of which expression is used, a linearly polarised receiver or probe in θ direction would capture the energy from one of the components in a circular polarisation. This is our common understanding of 3dB loss for a linearly polarised receiver to collect energy from a circularly polarised wave.

In case of two linearly polarised receivers placed in orthogonal directions as in TRP and TRS measurement, the other linearly polarised receiver would measure the energy in the orthogonal direction, namely ϕ direction.

In summary, the current TRP and TRS measurement arrangement works for both linear and circular polarisations and no modification is needed to measure NTN devices with circular polarisation.

Proposal 1: the current TRP and TRS measurement arrangement works for both linear and circular polarisations and no modification is needed to measure NTN devices with circular polarisation.

There are currently three NTN bands listed in TS 38.101-5 as shown below.

Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

The proposed parameters in R4-2412055 [2] do not include band 254. Using the same principles for band n256 and n255, parameters for band n254 are proposed below.

Table 1: NR FR1 NR-NTN TRP measurement parameters

| NR Band | CBW [MHz] | SCS (kHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | 5 | 15 | DFT-s-OFDM QPSK | Low | 322500 | 1612.5 | 497200 | 2486 | 12@6 | N/A |
|  |  |  |  | Mid | 323700 | 1618.5 | 498400 | 2492 |  |  |
|  |  |  |  | High | 324800 | 1624 | 499500 | 2497.5 |  |  |

Table 2: NR FR1 NR-NTN TRS measurement parameters

| NR Band | CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 322500 | 1612.5 | 497200 | 2486 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 323700 | 1618.5 | 498400 | 2492 |  |  |
|  |  |  |  |  | High | 324800 | 1624 | 499500 | 2497.5 |  |  |

Attention is drawn to the fact that the Mid frequency values are modified from 1618.25 MHz to 1618.5 MHz in UL and from 2491.75 MHz to 2492 MHz in DL, respectively. The reason for such modifications is that NTN allows two raster values, one at 100 kHz and an enhanced one at 10 kHz, and the larger raster value is chosen because a device supporting the enhanced raster value of 10 kHz would also support 100 kHz raster value.

The two raster values of 100 kHz and 10 kHz are shown in the tables below from TS 38.101-5.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 | Yes |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 | Yes |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 | Yes |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |  |

Proposal 2: Adopt Table 1 and Table 2 in R4-2415268 as test parameters for NR-NTN band n254 OTA testing

WF [3] in RAN4 #111 has the following directions for NTN performance metrics and there does not seem to be further progress on this in RAN4 # 112.

Issue 3-2-1: Categorized performance metric for NR-NTN handheld

Agreements

- RAN4 should not adopt direction 1, other directions are FFS

  - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS

  - Direction 2: Single point + measured full sphere (integrated or CDF), e.g.,

    - D2a, Peak EIRP/EIS+TRP/TRS,

    - D2b, Peak EIRP/EIS+ full Spherical coverage CDF

  - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g.,

    - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles),

    - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles)

  - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting,

    - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile

    - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere)

  - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

Out of the remaining 4 directions, full sphere measurement in direction 2 and 4 is the only feasible option because the pre-requisite of partial sphere measurement is to know the peak direction a priori and such information can either be obtained from device manufacturers or coarse grid measurement. However, peak direction is normally not available from manufacturers and coarse grid measurement is a full sphere measurement anyway. In other words, all the remaining 4 directions lead to full sphere measurement.

Granted that full sphere measurement would take more time to do, it is the only sure way to remove uncertainties on peak direction.

Proposal 3: given the uncertainty of peak direction of devices, full sphere measurement in direction 2 and 4 should be adopted for NTN performance metrics.

# Conclusion

This contribution makes the following proposals.

Proposal 1: the current TRP and TRS measurement arrangement works for both linear and circular polarisations and no modification is needed to measure NTN devices with circular polarisation.

Proposal 2: Adopt Table 1 and Table 2 in R4-2415268 as test parameters for NR-NTN band n254 OTA testing.

Proposal 3: given the uncertainty of peak direction of devices, full sphere measurement in direction 2 and 4 should be adopted for NTN performance metrics.

# Reference

[1] R4-2413603 WF for TRP_TRS_MIMO_OTA

[2] R4-2412055 Discussions on NTN OTA test method

[3] R4-2409931 WF for for [111][338] TRP_TRS_MIMO_OTA